3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505171

Bengaluru, India, 25th – 29th Aug 2025

Source:	CATT

Title:	Consideration on Scheduling Enhancement

Agenda Item:	8.7.4

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the remaining issues on LCP enhancement and BSR enhancement, our view is also given in this contribution.

Discussion

On LCP enhancement

The dependency of LCP adjustment and DSR enhancement

#### RAN2#126 agreement: #### RAN2 to confirm that the LCP adjustment could be independent with the DSR. - For LCP enhancements, Granularity: - LCP prioritization within a logical channel will not be considered in RAN2 discussions; - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). #### RAN2#127bis agreement: #### Agreements on LCP enhancements: - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation. - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

Based on the RAN2 agreement on LCP enhancement, it can be seen that LCP enhancement will have independent parameters from the DSR enhancement, there is no any restriction between these two features. For a logical channel, although it is natural that the LCP enhancement and DSR enhancement are used together to improve the capacity for delay critical data, it is also allowed that the network could configure only one feature to a logical channel. Thus, it is totally network implementation to configure the LCP adjustment and DSR enhancement together or not for a logical channel.

Proposal 1: RAN2 to confirm that it is network implementation to configure the LCP adjustment and DSR enhancement together or not for a logical channel, there is no further specification impact foreseen.

On BSR enhancement

[UE  capability-01] UE capability

RAN2#129bis agreement: 4. An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.

In RAN2#129bis meeting, RAN2 already achieved some agreements on UE capability. One FFS issue is whether UE supporting this feature shall also indicate support of delayStatusReport-r18. Based on the procedure of R18 DSR and R19 enhanced DSR, it seems that the UE supporting R19 enhanced DSR will naturally support R18 DSR. However, in fact, the UE supporting R19 DSR will compute and report the data volume based on the configured reporting threshold, while the UE supporting R18 DSR needs to compute and report the data volume based on the triggering threshold, the UE implementation to threshold is different. If these two features are coupled together for UE supporting R19 DSR, the UE needs to support two procedures considering the reporting threshold and triggering threshold for data volume computing simultaneously, so, it is not reasonable to think a UE supporting R19 DSR could also support R18 DSR. It is suggested not coupling the R19 DSR with R18 DSR together.

Proposal 2: (UE capability-01) UE supporting R19 enhanced DSR will separately report the capability for R18 DSR.

[MAC-1] On pending DSR cancel:

TS 38.321 A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. . A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain any DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR. NOTE 2: It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate this DSR MAC CE plus its subheader.

Based on above legacy procedure, the PDCP SDU associated with a DSR is a delay-critical PDCP SDU, and the pending DSR will be cancelled, such as, if all the PDCP SDUs associated with the DSR have been discarded or transmitted. For R19 DSR which could include the non-delay critical PDCP SDU if the reporting threshold is larger than the triggering threshold, if the legacy definition on PDCP SDU is applied, a pending R19 DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported. Considering that R19 DSR has already been agreed to include the non-delay critical data size to give network more information for scheduling, it is suggest not to cancel the pending DSR if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported, the definition of DSR could be changed to support this option. The PDCP associated with a DSR could be defined as follows:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU(as defined in TS 38.323 [4]) associated with the logical channel which triggered the Single Entry DSR, or if it has not been transmitted in any MAC PDU and is a delay-reporting PDCP SDU or a non-delay-reporting PDCP SDU(as defined in TS 38.323[4]) associated with the logical channel which triggered the Multiple Entry DSR.

Proposal 3: (MAC-1) Not to cancel the pending DSR if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported, the following TP on the definition of PDCP SDU is as below:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU(as defined in TS 38.323 [4]) associated with the logical channel which triggered the Single Entry DSR, or if it has not been transmitted in any MAC PDU and is a delay-reporting PDCP SDU or  a non-delay-reporting PDCP SDU(as defined in TS 38.323[4]) associated with the logical channel which triggered the Multiple Entry DSR.

[MAC-2] BSR cancellation by DSR

BSR provides the buffer status which includes UL data volume per LCG, while DSR provides the delay status which includes remaining time and delay-critical UL data volume per LCG. Since the trigger conditions of BSR and DSR are different, it is possible that both DSR and BSR are both triggered and pending, e.g, periodical BSR and DSR. In this case, if UL grant is received and the UL grant may only accommodate the DSR based on the LCP procedure, only DSR MAC CE can be transmitted. Considering that R19 DSR could be configured with a  reporting threshold higher than the triggering threshold and the non-delay critical data ahead of the critical data needs to be computed, it is very possible that the data volume in R19 DSR could cover the data volume in BSR. Hence, it is unnecessary to report the BSR again. In this case, it had better cancel the triggered BSR.

Proposal 4: (MAC-2) BSR shall be cancelled when the data volume it reports is already reported in a DSR MAC CE.

[RLC-13] whether delay-reporting/non-delay-reporting information is visible or not to RLC

The issue RLC-13 that the delay-reporting or non-delay-reporting information can be invisible to RLC is proposed, and all the description related to non-delay reporting RLC SDU can be removed.

| Index | Issue description |
| --- | --- |
| RLC-13 | Whether the delay-reporting or non-delay-reporting information is visible or not to RLC? [Rapp]: This is related to how to capture DSR in RLC. Whether the delay-reporting or non-delay-reporting information can be invisible to RLC? As we commented in the RLC running CR, it is benifical to not distinguish delay-reporting RLC SDU and non-delay-reporting RLC SDU in RLC spec. RLC can rely on current delay reporting indication from PDCP to evaluate RLC data volume. For example, if dsr-ReportNonDelayCriticalData is configured, PDCP indicates both delay-reporting PDCP SDUs and non-delay reporting PDCP PDUs as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. Otherwise, PDCP only indicates the delay-reporting PDCP SDUs as the delay-reporting RLC SDUs associated with the i-th dsr-ReportingThreshold. We see that the delay-reporting or non-delay-reporting information can be invisible to RLC, and all the description related to non-delay reporting RLC SDU can be removed. [Rapp] A new open issue is added, i.e. RLC-13, to check companies’ views. |

For issue RLC-13, based on below running TS 38.323 running CR, the non-delay-reporting PDCP SDU or PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs will be indicated by PDCP as part of delay-reporting PDCP data volume, both of them are not submitted to lower layers. But the PDCP will provide a delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers. Hence, it can be seen that non-delay-reporting data included in delay-reporting PDCP data volume can be invisible to RLC.

TS 38.323 Running CR in [POST130][507][XR] If dsr-ReportNonDelayCriticalData is configured, the transmitting PDCP entity shall further consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold: - the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i. The transmitting PDCP entity provides a delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

However, RAN2 needs to discuss whether there is non-delay-reporting RLC SDU prior to delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold.  Based on above analysis, as PDCP entity only computes the PDCU SDUs/Data PDUs not submitted to lower layers in the delay-reporting PDCP data volume, it is possible that there are non-delay-reporting PDCP Data PDUs submitted to RLC layer and not included in PDCP data volume. Once the delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU is provided to RLC entity by PDCP, it is a possible case that a non-delay-reporting RLC SDU or segment will be prior to delay-reporting RLC SDU or segment in the buffer, which could not be known by PDCP entity. Hence, the definition on non-delay-reporting RLC SDU is needed and not removed.

Delay-reporting RLC SDU: a delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]). Editor’s Note: The terminology is to be aligned with other specifications. Non-delay-reporting RLC SDU: a non-delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU that will be transmitted prior to any of the delay-reporting RLC SDU(s) associated with the i:th dsr-ReportingThreshold. Editor’s Note: The terminology is to be aligned with other specifications.

Proposal 5: (RLC-13) Delay-reporting/non-delay-reporting information needs to be visible to RLC, the definition on non-delay-reporting RLC SDU cannot be removed.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: RAN2 to confirm that it is network implementation to configure the LCP adjustment and DSR enhancement together or not for a logical channel, there is no further specification impact foreseen.

Proposal 2: (UE capability-01) UE supporting R19 enhanced DSR will separately report the capability for R18 DSR.

Proposal 3: (MAC-1) Not to cancel the pending DSR if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported, the following TP on the definition of PDCP SDU is as below:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU(as defined in TS 38.323 [4]) associated with the logical channel which triggered the Single Entry DSR, or if it has not been transmitted in any MAC PDU and is a delay-reporting PDCP SDU or  a non-delay-reporting PDCP SDU(as defined in TS 38.323[4]) associated with the logical channel which triggered the Multiple Entry DSR.

Proposal 4: (MAC-2) BSR shall be cancelled when the data volume it reports is already reported in a DSR MAC CE.

Proposal 5: (RLC-13) Delay-reporting/non-delay-reporting information needs to be visible to RLC, the definition on non-delay-reporting RLC SDU cannot be removed.

Reference

- RAN2#130 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei