3GPP TSG-RAN WG2 Meeting #131                                                        R2-2505172

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT

Title:	Remaining issues on XR-specific RLC Enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues on unnecessary retransmission avoidance for XR, our views on these issues are given in the following part.

Discussion

[RLC-11] No SDU to retransmit the poll with

In RAN2#130 meeting, the agreement on RLC-11 is as follows [1].

- No SDU to retransmit the poll with (RLC-11) Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

For unnecessary retransmission, RAN2 adopt a “combined” approach for avoiding unnecessary RLC retransmissions and has the following agreements in RAN2#127bis meeting [2]. Therefore, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, SR would always be triggered from RX side and window stalling won’t happen.

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

Observation 1: SR would always be triggered from RX side and window stalling won’t happen.

Proposal 1: (RLC-11) Confirm the working assumption that no need to address window stalling issue with polling retransmission, this should not happen due to SR triggering from Rx side.

[RLC-10] Configuration clarifications

In the RLC running CR discussion, the below open issue RLC-10 is raised.

| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| --- | --- | --- |

During discussion in RAN2#130 meeting, there’re two main proposals as follows for open issue RLC-10 [1]..

R2-2503830 Remaining open issues on RLC enhancements for XR LG Electronics Inc. discussion Rel-19 NR_XR_Ph3-Core Proposal 2. Add RRC configuration guidance for the stopReTxObsoleteSDU and t-RxDiscard, i.e., if stopReTxObsoleteSDU is configured, the t-RxDiscard is configured at the peer receiving entity. R2-2503439 RLC AM retransmission enhancements Xiaomi discussion Rel-19 NR_XR_Ph3-Core Proposal 2: (RLC-10) It is up to network implementation on whether to configure stopReTxDiscardedSDU / t-RxDiscard and related network behavior.

RAN2 have agreement that this is a “combined” approach for avoiding unnecessary RLC retransmissions, hence it should be specified in specification. Otherwise, this function may not work well if network only configure stopReTxDiscardedSDU.

Proposal 2: (RLC-10) Add RRC configuration guidance for the stopReTxDiscardedSDU and t-RxDiscard, i.e., stopReTxDiscardedSDU and t-RxDiscard are configured together.

Proposal 3: (RLC-10) Besides RRC configuration guidance, capture in stage-2 that this is a “combined” approach for unnecessary retransmission avoidance.

[RLC-12] Impact of discard on PDCP SN gap report

In the RLC running CR discussion, the below open issue RLC-12 is identified.

| RLC-12 | FFS how to handle PDCP SN gap report handling during UE mobility - As raised by Qualcomm below. The corresponding Rel-18 agreement is: - RAN2 can discuss in the context of Rel-19 whether this problem is a critical problem to solve due to RLC enhancements. If it is a problem further discussions are needed on the alternatives. | Issue Type: Not essential not important? How to address it: based on companies’ contribution |
| --- | --- | --- |

And in RAN3#128 meeting, RAN3 have the following solution to handle this issue [3]. This means RAN3 cover the case that source gNB receives the SN gap report. RAN2 would face the case that UE transmits the SN gap report but gNB does not receive it.

As discussed in the correlated discussion paper R3-253334, to solve the issue of PDCP SN gap report handling during UE handover procedure, one alternative is that the source gNB forwards the SN gap report to the target gNB. To implement this, new IEs should be included in the SN STATUS TRANSFER message to indicate the packets discarded beafore handover. The new IEs can be added into the DRBs Subject To Status Transfer List IE and use a bit map to indicate the COUNT of the discarded packets.

During discussion in RAN2#130 meeting, there’re three main proposals as follows [1]..

Proposal 3: RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 3): - For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. - A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration. Proposal 8. No enhancement is needed for PDCP SN gap report handling during UE mobility. Proposal 9. If the proposal 8 is not agreeable, the UE retransmits a PDCP SN gap report when an SDU indicated in a previous SN gap report is negatively acknowledged in a PDCP status report.

As we can see, the proposals to solve the problem impact the UE behavior. And with RAN3 solution, it would be a corner case and would not have serious impact to XR service. Considering the timeline, we propose not to handle it in RAN2 in Rel-19, no enhancement is needed.

Proposal 4: (RLC-12) No enhancement is needed for PDCP SN gap reporting during UE mobility.

[PDCP-2] Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers

The scenario about this open issue had been discussed in RAN2#128 meeting [5], and the agreement is as follows.

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced

We think some additional delay in discarding is not a critical issue. Besides that, if PDCP triggers a PDCP SN gap in this scenario, but t-RxDiscard in receiver side does not expire, the status would be different between RLC entity and PDCP entity in receiver side. Therefore we propose not to trigger PDCP SN gap report in this scenario and keep the original specification.

Proposal 5: (PDCP-2) No enhancement is needed in this scenario and keep the original specification.

[UE capability-04] Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19

In RAN2#130 meeting, the discussion and agreement about UE capability-04 is as follows [1].

- OPPO thinks we do not have mandate UE supporting Tx side to also support Rx side. Vivo agrees. - LGE thinks Tx side requires Rx side, otherwise we can have window stalling issue we just discussed. LGE supports the proposals. - OPPO indicates that this is for UL so SR is triggered by the network. - (UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report. - (UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

If UE supports txRLC-StopReTxDiscardedSDU-r19 and does not indicate support rxRLC-Discard-r19, with UE capability information, there are two options for gNB to configure. Option1, gNB can configure stopReTxDiscardedSDU-r19 and not configure t-RxDiscard-r19. Option2, gNB can configure both stopReTxDiscardedSDU-r19 and t-RxDiscard-r19. With option1, window stalling may happen in TX side both in gNB and UE. With option 2, window stalling may happen in TX side in gNB. Therefore a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

Proposal 6: (UE capability-04) a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

The two RRC parameters, stopReTxDiscardedSDU-r19 and t-RxDiscard-r19 are defined as follows [6].

DL-AM-RLC-v19xy ::= SEQUENCE { t-RxDiscard-r19 T-RxDiscard-r19 OPTIONAL -- Need R } UL-AM-RLC-v19xy ::= SEQUENCE { stopReTxDiscardedSDU-r19 ENUMERATED {enabled} OPTIONAL -- Need R }

Conclusion

According to the analysis in section 2, the following observations and proposals were made:

Observation 1: SR would always be triggered from RX side and window stalling won’t happen.

Proposal 1: (RLC-11) Confirm the working assumption that no need to address window stalling issue with polling retransmission, this should not happen due to SR triggering from Rx side.

Proposal 2: (RLC-10) Add RRC configuration guidance for the stopReTxDiscardedSDU and t-RxDiscard, i.e., stopReTxDiscardedSDU and t-RxDiscard are configured together.

Proposal 3: (RLC-10) Besides RRC configuration guidance, capture in stage-2 that this is a “combined” approach for unnecessary retransmission avoidance.

Proposal 4: (RLC-12) No enhancement is needed for PDCP SN gap reporting during UE mobility.

Proposal 5: (PDCP-2) No enhancement is needed in this scenario and keep the original specification.

Proposal 6: (UE capability-04) a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

Reference

- Draft_RAN2_130_Meeting_Report_v2.

- R2-2409502, Report of 3GPP TSG RAN WG2 meeting #127bis.

- R3-253905, Correction for PDCP SN gap report during UE’s handover – Alt 1, CR, Rel-19

- RP-251866, New WID on XR (eXtended Reality) for NR Phase 4, WI,  Rel-20

- Draft_RAN2_128_Meeting_Report_v1.

- R2-250xxxx Introduction of R19 XR enhancements for RRC spec_v14_Rapp.