3GPP TSG-RAN WG2 Meeting #131                                                        R2-2505174

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT

Title:	Discussion on Multi-hop Discovery and (Re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN2#130 meeting, the issue on Rel-19 multi-hop SL U2N relay discovery and (re)selection had been discussed[1]. In the current contribution, we further discuss the key leftover issues:

- Rules for relay (re)selection.

- [RRC-6] FFS on PC5 AS conditions for discovery model B.

- FFS on Uu upper bound for intermediate relay UE.

Discussion

Rules for relay (re)selection

For the Rel-17 U2N relay and Rel-18 U2U relay, the below agreements were reached for rules of relay (re)selection.

For Rel-17:

Besides serving cell ID, PLMN ID, L2/L3 relay support (if agreed in discovery session) and relay load, other additional AS criteria are not considered in this release.

Relay load is not considered as a (re)selection criterion in Rel-17.

For relay selection and reselection, when remote UE has multiple suitable relay UE candidates which meet all AS-layer & higher layer criteria and remote UE need to select one relay UE by itself, it is up to UE implementation to choose one relay UE.  This does not exclude gNB involvement in service continuity for U2N.

For Rel-18:

SL-RSRP and SD-RSRP can be used for relay selection/reselection criteria. FFS when each of the two quantities are used and whether to re-use the criteria in Rel17.

Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

For Rel-19 multi-hop sidelink U2N relay case, the question in RAN2 is whether need to further consider the hop count information as an extra AS-criteria. In the current SA2’s specification, it clearly records that the hop count information had already been considered as a high layer criterion, hence, there is no need to further define rule to select among multiple relays which further consider the number of hops to the gNB in AS layer.

10.	If the Remote UE receives multiple Discovery Response messages within a pre-configured time window, it may perform relay path selection based on e.g. path information, the PC5 signal strength between Remote UE and its neighbour Intermediate UE-to-Network Relay, Accumuled QoS for PC5 link and the number of hops to the 5G ProSe UE-to-Network Relay.

Proposal 1: Hop count information is not considered as a relay (re)selection AS-criterion in Rel-19.

[RRC-6] FFS on PC5 AS conditions for discovery model B

In RAN2#129bis meeting, the below agreements were reached for discovery model B:

If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration.  If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages.

For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold.

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE.

For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold.

There is one FFS that whether the intermediate relay UE always establishes itself as a remote UE firstly for discovery model B. For this issue, we think it is not needed for discovery procedure, only for RRC connection procedure. Before the intermediate relay UE acting as an intermediate relay UE, whether it needs to always act as a remote UE to access the gNB firstly. Our view is this case can happen but should not be mandatory. Take one step back, in case that the intermediate relay UE establishes itself as a remote UE firstly, that is to say, the PC5-S between the intermediate relay UE and its parent UE had been established, this case can be handled by the previous RAN2#129 meeting’s agreement (For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.). All in all, we think the case that the last relay UE needs to check AS conditions before responding to intermediate relay UE’s discovery model B request is valid and the corresponding FFS can be deleted without further handling.

Proposal 2: (RRC-6) RAN2 confirms that the case for the last relay UE to check AS conditions before responding to intermediate relay UE’s discovery model B request is valid and the corresponding FFS can be deleted without further handling.

Another issue is whether the same SD-RSRP threshold (in SL-RelayUE-ConfigMH-r19) - that is used by the Intermediate Relay UE to determine whether to forward the solicitation message - can also be applied by the Last Relay UE when determining whether to send a discovery response message to the Intermediate Relay UE. The proponent raised that it would be straightforward for the Last Relay UE to apply the same threshold when deciding whether to send the discovery respond message. But there is a little bit unclear about “the same threshold”: one understanding is a new threshold is needed with limitation the value of the new introduced threshold. Another understanding is no new threshold introduced while we directly reuse the threshold. From flexibility point of view, different function should correspond to different parameter/threshold, hence:

Proposal 3: In case the Last Relay UE needs to decide whether to send a discovery response message to the Intermediate Relay UE, RAN2 introduces a separate SD-RSRP threshold similar as the threshold used by the Intermediate Relay UE to determine whether to forward the solicitation message be applied.

FFS on Uu upper bound for intermediate relay UE

In RAN2#128 meeting, whether an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE is needed or not was discussed and reached the below conclusion:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

The first FFS was solved with the below agreement in RAN2#129 meeting:

The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.  FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive.

For the second FFS, in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, some additional restriction should be introduced to avoid the scenario related to the above agreement. RAN2 can leave this issue to SA2, or the UE role can be used as one AS condition to judge that whether a UE can be operated as an intermediate relay UE or not.

Proposal 4: The role of UE can be used as AS condition to judge that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 5: If RAN2 decides to leave the issue that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Hop count information is not considered as a relay (re)selection AS-criterion in Rel-19.

Proposal 2: (RRC-6) RAN2 confirms that the case for the last relay UE to check AS conditions before responding to intermediate relay UE’s discovery model B request is valid and the corresponding FFS can be deleted without further handling.

Proposal 3: In case the Last Relay UE needs to decide whether to send a discovery response message to the Intermediate Relay UE, RAN2 introduces a separate SD-RSRP threshold similar as the threshold used by the Intermediate Relay UE to determine whether to forward the solicitation message be applied.

Proposal 4: The role of UE can be used as AS condition to judge that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 5: If RAN2 decides to leave the issue that whether a UE can be operated as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

Reference

- R2-2504674	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report

- 3GPP TS 23.304 Proximity based Services (ProSe) in the 5G System (5GS)