3GPP TSG-RAN WG2 Meeting #131                              R2-2505178

Bangalore, India, Aug 25th–29th, 2025

Agenda Item:	8.9.2

Source: 	Transsion Holdings

Title:  	Discussion on support of Store&Forward

Document for:	Discussion and Decision

Introduction

In the last RAN2#130 meeting, the support of Store&Forward for IoT-NTN was discussed, and the following agreements were made[1]:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In this contribution, we provide our views on these issues to proceed the work item.

Discussion

Transition time information between satellite S&F mode and normal mode

- It has been agreed in last meeting that transition time information between satellite S&F mode and normal mode will be introduced, and when the S&F operation indication is broadcast, it means the satellite is working in S&F mode, the transition time information is interpreted as the time when the current satellite operation will transit from S&F mode to normal mode, RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the satellite operation transits to normal mode. When the S&F operation indication is not broadcast, it means the satellite is working in normal mode, the transition time information is interpreted as the time when the current satellite operation will transit from normal mode to S&F operation. However how the transition time information indicating when the current satellite operation will transit from normal mode to S&F operation can be used by UE is not clear. As for the R19 UE not supporting S&F mode can’t work well in the satellite working in S&F mode, we think this information at least can be used by the R19 UE not supporting S&F mode to decide whether to trigger to access the network when the transition time is near.

- Proposal 1:With the the transition time information indicating when the current satellite operation will transit from normal mode to S&F mode, if the remaining time from normal mode to S&F mode is not enough to finish the service, the UEs not supporting S&F mode may not triggered to access the network.

Paging enhancement in satellite Store&Forward mode

- As the satellite has already stored the downlink data for UE in advance, it means the network knows which UE needed to be paged and the paging demand is not random anymore and as the large coverage of the satellite, the number of UEs needed to be paged and the amount of downlink data needed to be send by the satellite maybe huge, which may cause paging congestion considering the limited coverage time.

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 2: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- The satellite in S&F mode can distribute the paging message in time to avoid congestion from network’s perspective, however the UE has to monitor the PF/POs in every DRX cycle no matter whether there is a paging for the UE or not and there are no more paging message from the core network as there is no feeder link between the satellite and the ground, so the paging send by satellite and paging monitoring by UE can be optimized for power saving. For example there can be two DRX parameters which is different for S&F mode and normal mode, the DRX cycle or nB for S&F mode can be set larger than those in normal mode. Moreover if the UE keeps monitoring paging for some cycle or some times or some duration and if no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

- Proposal 3: Two different Paging parameters can be configured for S&F mode and normal mode.

- Proposal 4: UE keeps monitoring paging for some cycle or some times or some duration and if no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

UP CIoT EPS optimisation support in satellite Store&Forward mode

For UP mode, when UE resume the RRC connection with a eNB, if the eNB is not the old eNB which suspend the connection, the eNB has to fetch the context from the old eNB. However for satellite in S&F mode, as the fast moving satellite and the no ISL between the satellite, the eNB the UE trys to resume may not the old eNB and can’t get the context from the old eNB, the UE has to fallback to the RRC connection setup procedure. To avoid this, it has to make sure that the eNB the UE resume has the UE context. To achieve this, the context can be transfer to specific satellites and UE only resume the connection to this satellites. As to how to decide these satellites, there can be two methods: one is the UE itself determines the satellites based on the satellite assistance information and the UE traffic pattern and send the satellite list to the network, then the network confirms some or all of them and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered. The other method is the UE provide some assistant information such as UE location and traffic pattern to the network and the network determines the satellite list and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered.

- Proposal 5: When UP mode is used, the UE context can be transfered/distributed to specific satellites via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

When UE resume the RRC connection to a new eNB, the security key used by the new eNB is generated by the old eNB using the old key or NH、the target PCI and target EARFCN-DL. For S&F mode, if the satellite list the UE will resume are determined, the security key used by the new eNB in the satellite list in the future can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list. The generated security key are associated with each of the satellite in the satllite list and will be transfered to these satellites in advance via the ground network when the feeder link recovered.

- Proposal 6: The security key used by the new eNB can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list and transfered to these satellties via the ground network.

Conclusion

In this contribution we discussed issues related to S&F mode in IoT-NTN and made the following proposals:

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 1:With the the transition time information when the current satellite operation will transit from normal mode to S&F mode, If the remaining time from normal mode to S&F mode is not enough to finish the service, the UEs not supporting S&F mode may not triggered to access the network.

- Proposal 2: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- Proposal 3: Two different Paging parameters can be configured for S&F mode and normal mode.

- Proposal 4: UE keeps monitoring paging for some cycle or some times or some duration and if no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

- Proposal 5: When UP mode is used, the UE context can be transfered/distributed to specific satellies via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

- Proposal 6: The security key used by the new eNB can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list and transfered to these satellties via the ground network.

Reference

- “Draft_RAN2_130_Meeting_Report_v3”