3GPP TSG-RAN WG2 Meeting #131	R2-2505187

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.2.2

Source:	OPPO

Title:	Discussion on functionality management for measurement event prediction

Document for:	Discussion, Decision

Introduction

This paper will discuss applicability procedure including inference parameters for Measurement event prediction. In addition, inference configuration reporting and performance monitoring will be also covered in this paper.

Applicability report

The conclusions on applicability procedure for AI mobility is discussed in such way that no agreement is scenario specific and use case specific. Because of this, agreements for RRM measurement prediction can be also applied for Measurement event prediction.

Proposal 1: Agreements on applicability for RRM measurement prediction are also applied for measurement event prediction unless specific issue is identified

The difference between RRM measurement prediction and measurement event is the event itself. For indirect measurement event prediction, measurement event related parameters shall be part of the inference configuration. Assuming one inference configuration is applicable for RRM measurement, then it shall be also applicable for corresponding measurement event prediction because assessing a measurement event based on actual/predicted measurement results is not part of the model at all. And due to same reason indirect prediction can be generalized among measurement event type. So, to indicate that an inference configuration for indirect measurement event prediction, maybe one bit is sufficient.

Proposal 2: For indirect measurement event prediction, inference parameter of partial configuration of corresponding RRM measurement prediction can be reused.

Proposal 2a: For indirect measurement event prediction, it is sufficient to have one bit to indicate the inference configuration for measurement event prediction.

For direct measurement event prediction at least measurement event type should be configured as inference configuration. Because assessing measurement event itself is also part of the model it is possible that model can’t be generalized among measurement events. Hence measurement event type could be inference parameter of partial configuration for measurement event prediction. This conclusion is necessary only if direct measurement prediction is supported.

Proposal 3: Measurement event type could be inference parameter of partial configuration for direct measurement event prediction (if supported).

Inference report

For measurement event report, how to report measurement event could be related to the captured HO model in TR even it is for evaluation purpose in 1st place. Among all the 3 options UE should report predicted measurement event at t0. For spatial domain and frequency domain prediction, the time instance when a measurement event is predicted should be the same time instance the event occurring i.e. when corresponding TTT timer expires. For temporal domain case B, technically it is possible that TTT timer expires upon a predicted measurement result. In this case the event occurs in future instead of current time instance. But we think such optimization is not necessary because the intention of temporal case B is to save measurement. And simulation also show that minor or even no system level performance degrades by following such principle. Since network supposes to know the measurement event is based on which domain prediction, nothing else other than measurement event itself should be reported.

Proposal 4: For measurement event prediction based on temporal domain case B, spatial domain and frequency domain prediction, UE need report predicted measurement event by following legacy procedure i.e. no spec impact is introduced.

For temporal domain case A, however more information is needed. The most important information is when the event supposes to happen from current time instance i.e. the time point when TTT timer supposes to expire. As for the measurement result, both t0 and t1 are interested in gNB to assess current situation and predicted situation.

For prediction based on temporal domain case A, measurementReport message should be transmitted early. For handover model option 1 in the TR, UE could report predicted measurement event @t0. For handover model option 2 in the TR, network supposes to transmit handover command message upon time instance where entering condition is met as indicated in Table 3.2.

Figure 3-1 HO model option 2

Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

Table 3-2

The issue is whether 2nd measurement report message around t2 in Figure 3-1 is necessary?

When UE report a future measurement event @t1 as illustrated in Figure 3-1, UE should also indicate the timing information of t1 because otherwise network has no idea when the event will occur in future. Because of such timing information network can know when is t2 considering t1-t2 = TTT.

Observation 1: After receiving a future measurement event with timing information of measurement event, network can know when entering condition could be met.

The t1 in Figure 3-1 shall be within PW i.e. t1-t0<=PW. And the PW should be set as such that predict accuracy, L3 RSRP difference and/or F1 score, should be good enough because otherwise e.g. false alarm could hurt handover performance on the contrary. Meeting entering condition can also be realized by configuring PW<=TTT by network’s implementation, if network really want to make sure it is met based on actual measurement result.

Observation 2: Whether handover command is transmitted upon entering condition could be left for network’s implementation

Based on analysis for both handover models, we propose:

Proposal 4: For measurement event prediction based on temporal domain case A, @t0 UE can report timing information when corresponding TTT timer expires in future i.e. t1, measurement event in t1 and relevant measurement result in t0 and t1. How to treat such measurement report is up to network’s implementation.

Performance monitoring

For measurement event prediction, the first issue is what metric need be monitored? For indirect prediction methodology, prediction accuracy can already reflect the performance of the model. For direct prediction methodology, intermediate KPI i.e. F1 score and/or its components can reflect performance of the model. But it is bit tricky for UE to calculate F1 score. During simulation the ground truth is gathered via legacy HO procedure in advance. In field deployment UE may not be able to know anything about ground truth. For example, for temporal domain case A, handover may be triggered based on predicted measurement event before a real measurement event is triggered. For measurement event prediction based on temporal domain case B, spatial domain prediction and frequency domain prediction, a good model is expected to predict a measurement event closely around a ground truth measurement event. On the other hand, the preparation phase between reporting measurementReport message and handover execution is such short (40~50ms) it is likely that handover command is received before a ground truth measurement event is observed.

Observation 3: For measurement event prediction, it is possible that ground truth measurement event will never happen during monitoring.

One alternative for monitoring F1 score is to monitor system level performance e.g. HO failure rate. It means UE need collect handover history across serving cells. The problem for such scheme is that it may take long time. In case the real performance is not good, long time also means degradation of system level performance lasts long time.

Observation 4: Monitoring system level performance may last long time, which may defeat the very purpose of AIML scheme

Based on these Observation3 and Observation4, one way is to rely on performance monitoring scheme of RRM measurement prediction. It also means only indirect measurement event prediction is supported.

Proposal 5: Only indirect measurement event prediction is supported

For indirect measurement event prediction, the performance of the model is the same as corresponding RRM measurement prediction. So logically same monitoring scheme as for RRM measurement prediction can be reused.

Proposal 6: For indirect measurement event prediction, same monitoring scheme and metric as for corresponding scenario of RRM measurement prediction is reused.

Conclusion

Here are our observations and proposals:

Proposal 1: Agreements on applicability for RRM measurement prediction are also applied for measurement event prediction unless specific issue is identified

Proposal 2: For indirect measurement event prediction, inference parameter of partial configuration of corresponding RRM measurement prediction can be reused.

Proposal 2a: For indirect measurement event prediction, it is sufficient to have one bit to indicate the inference configuration for measurement event prediction.

Proposal 3: Measurement event type could be inference parameter of partial configuration for direct measurement event prediction (if supported).

Proposal 4: For measurement event prediction based on temporal domain case B, spatial domain and frequency domain prediction, UE need report predicted measurement event by following legacy procedure i.e. no spec impact is introduced.

Observation 1: After receiving a future measurement event with timing information of measurement event, network can know when entering condition could be met.

Observation 2: Whether handover command is transmitted upon entering condition could be left for network’s implementation

Proposal 4: For measurement event prediction based on temporal domain case A, @t0 UE can report timing information when corresponding TTT timer expires in future i.e. t1, measurement event in t1 and relevant measurement result in t0 and t1. How to treat such measurement report is up to network’s implementation.

Observation 3: For measurement event prediction, it is possible that ground truth measurement event will never happen during monitoring.

Observation 4: Monitoring system level performance may last long time, which may defeat the very purpose of AIML scheme

Proposal 5: Only indirect measurement event prediction is supported

Proposal 6: For indirect measurement event prediction, same monitoring scheme and metric as for corresponding scenario of RRM measurement prediction is reused.