3GPP TSG-RAN WG2 Meeting#131	R2-2505196

Bengaluru, India, 25th – 29th Aug, 2025

Source:	vivo

Title:	Remaining issues of A-IoT Paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

RAN2#130 meeting has made the following progress in terms of the format of A-IoT paging [1].

- For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. - For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e., as long as it is addressed to the corresponding device). - To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. - Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message. - Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

In this contribution, with the guidance of issues [3] left with FFS, we investigate the remaining issues of A-IoT paging message.

- Discussion

## On Issue 1-4: Configuration for access occasions

| Issue 1-4: AO number field | How to indicate the number of access occasions, e.g. the maximum number, the length of field, format design. - Relevant agreements: - Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS - Status in running CR: the field name and format is captured in 6.2.1.1 with proposed value range to be reviewed by companies. |
| --- | --- |

As agreed in the last meeting, the number of access occasions is included in a form of exponential encoding with four bits. It means that the number of access occasions that the reader can allocate should only be the power of 2, from 2^0 to 2^(24-1). According to the possible values of time and frequency resources [2] ({1,2} and {1,2,3,4,5,6,7,8}, respectively), the designed exponential encoding of the number of access occasions prevents some possible values of Y (Y is equal to the number of possible values of the small frequency shift factor, i.e., the number of bits with value 1 in the frequency domain indication bitmap). We consider it constrains the reader implementation to some extent, and this does not well implement RAN1 agreement. In this sense, the exponential encoding should be avoided for the number of access occasions in order to allow any possible value for the number of access occasions designed by RAN1.

- (Issue1-4) With the current design of 4-bit exponential encoding of the number of access occasions, half values of Y (i.e., 3,5,6,7) cannot be indicated according to RAN1 agreement, which loses the system flexibility.

Actually, the number of access occasions can be any possible integer obtained from combinations of X, Y and the number of resource sets, assuming that the resource set means the set of resources followed by the paging message or the access trigger message. If the paging message still includes the number of access occasions, since only linear coding can achieve the exact indication on any possible value combinations, we would like to have the number of resource sets rather than the number of access occasions in the paging message to reduce some signaling overhead. With the time/frequency resource indicated in the D2R scheduling, the device can thereafter obtain the total number of access occasions by a simple multiplication operation. Regarding the detailed value of this field, we are open to discussing the coding methods. Considering the extreme flexibility of the total number of access occasions, linear encoding is slightly preferred with 10 bits to indicate the number of resource sets from 1 to 1024, leading the number of access occasions to reach 16000 approximately for one paging round.

- (Issue1-4) RAN2 to revisit the agreement and to introduce the number of resource sets (a set of resources following Paging message/Access trigger messages) instead of the number of access occasions in the paging message, via 10-bit linear encoding way.

## On Issue 2-3: R2D trigger message byte alignment

| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. - Relevant agreements: - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message. - FFS R2D byte alignment dependent on TBS size discussion - Status in running CR: not captured yet. |
| --- | --- |

Regarding the format of the Access Trigger message, we primarily believe it is worth sticking on byte-aligned design principle to let the system to be implementation-wise. The major intention is to let the device be aware of whether the received Access Trigger message is associated with the exact paging message. In the case that the transaction ID field is not introduced, if the next paging message (Paging round 2) starts a new service and it is miss detected together with some former access trigger messages belonging to Paging round 1 by the device, the device may consider the following Access Trigger messages belonging to Paging round 1. It may cause malfunction to the subsequent access of other devices paged in Paging round 2, which has a fairness issue. The logical illustration is in Figure 2.2-1.

Figure 2.2-1: fairness issue caused by no TI in Access Trigger Message

Figure 2.2-2: how TI in Access Trigger Message solves the fairness issue

On the contrary, if Transaction ID is carried in Access Trigger message, the device can be aware of the current received message with the stored one, and decide whether to attempt to access as in Figure 2.2-2. To keep the whole system working fluently, the signaling overhead can be trivial for extra carrying 4-bit Transaction ID.

- (Issue2-3) Device cannot recognize the Access Trigger message associated with which Paging message, if transaction ID is not included in the Access Trigger message, which may cause access collision and unfairness issue when miss-detection happens.

Figure 2.2-3: A-IoT MAC format for Access Trigger message

- (Issue2-3) The MAC format of Access Trigger message is byte-aligned, containing fields of Message type to indicate Access Trigger and Transaction ID.

## On Issue 1-6: Reader visibility of paging identifier

| Issue 1-6: Paging ID visibility | Whether Paging ID is invisible or visible to MAC. - Relevant agreements: - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - From the previous discussion, there are some motivations to make paging ID visible to MAC: - 1. Reader can operate on the paging ID for further sub-grouping. The Rapp understands this can be considered as an enhancement from reader side for better system efficiency. From device side, since there is an explicit indication for CBRA and CFRA, the device (even in multi-device CFRA) can determine how to perform random access instead of paging ID/group ID. In this case, such visibility is not an essential function. And according to guidance from chair lady, such enhancement can be considered with lower priority. - 2. Reader can associate the paging ID/device ID and AS ID for a given device within a service request. The Rapp understands according to RAN3 LS R3-252481, reader will allocate NGAP device ID for each device and maintain the per-session per-device context, via which the reader can associate the command receiving from the NG interface with the AS ID assigned for a device, i.e. such device management/association does not rely on the paging ID/device ID. - 3. The Temp ID may have impact on this visibility discussion. The Rapp understands SA3 has not concluded on the solution of Temp ID. But majority seems think this Temp ID is maintained/managed between CN and device, since they already concluded there is no AS security in A-IoT. Therefore, no RAN2 discussion is needed before SA3 further inputs. |
| --- | --- |

As Rapp suggested, RAN2 has assumed that paging identifier is transparent to A-IoT MAC layer [1].

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer.

In our understanding, the reason why such issue exists lies in how the inventory procedure is triggered and performed for a command service. In SA TS 23.369, the command procedure from the AIoT server can trigger firstly an inventory procedure and then multiple per-device command procedures between devices and AIOTF [3] as in Annex. However, due to the inventory procedure may not target for specific devices, i.e., paging identifier in the air interface (or information about the target AIoT Device(s) among CN nodes) can be in the form of “filtering criteria” to filter a number of devices in one paging message with a number of AS ID assigned for each paged device, it cannot help this problem even if the reader is aware of the paging identifier. If the reader is exposed with device ID in MSG3, this problem can somehow be solved within RAN interaction. However, there is no way for such a NAS layer information of device ID to be exposed to the reader for security consideration.

- (Issue1-6) The paging identifier can be in the form of filtering criteria to filter one or multiple devices, which cannot be associated to a specific AS ID assigned for one device for the further command service.

With regard to this issue, we find that RAN3 reaches one related agreement in RAN3#127bis meeting.

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

This RAN NGAP device ID between RAN reader and AIOTF is similar to RAN NGAP UE ID maintained between gNB and AMF, to dedicatedly identify a device in NGAP interface.

- (Issue1-6) RAN3 introduces RAN NGAP device ID maintained per device by the reader, reported together with inventory report to AIOTF, where inventory report is based on MSG3 in the air associated to a specific AS ID.

In this sense, the reader can allocate this RAN NGAP Device ID for a device together with the inventory report, which is to transfer the permanent device ID via MSG3 over the air to the AIOTF. And since the AS ID (if assigned) is assigned in MSG2, indicating a dedicated MSG3 occasion for such device. Therefore, when receiving MSG3, the reader can know it is associated with which specific device’s AS ID. As illustrated in Figure 2.2-1, with such a mapping relationship, the reader can associate the AS ID assigned for the device and the command targeted for this device, and then use the corresponding AS ID for later command scheduling.

Figure 2.3-1: how reader attach AS ID and NGAP RAN device ID for a specific device

- (Issue1-6) RAN2 to confirm that the paging identifier is invisible at reader’s MAC layer, i.e., the reader maintains the mapping between RAN NGAP device ID used in the interface and AS ID assigned for the device in the air, to ensure that the command is toward which specific target device.

- Conclusion

On a basis of above analysis, we make the following proposals regarding A-IoT paging:

- (Issue1-4) With the current design of 4-bit exponential encoding of the number of access occasions, half values of Y (i.e., 3,5,6,7) cannot be indicated according to RAN1 agreement, which loses the system flexibility.

- (Issue1-4) RAN2 to revisit the agreement and to introduce the number of resource sets (a set of resources following Paging message/Access trigger messages) instead of the number of access occasions in the paging message, via 10-bit linear encoding way.

- (Issue2-3) Device cannot recognize the Access Trigger message associated with which Paging message, if transaction ID is not included in the Access Trigger message, which may cause access collision and unfairness issue when miss-detection happens.

- (Issue2-3) The MAC format of Access Trigger message is byte-aligned, containing fields of Message type to indicate Access Trigger and Transaction ID.

- (Issue1-6) The paging identifier can be in the form of filtering criteria to filter one or multiple devices, which cannot be associated to a specific AS ID assigned for one device for the further command service.

- (Issue1-6) RAN3 introduces RAN NGAP device ID maintained per device by the reader, reported together with inventory report to AIOTF, where inventory report is based on MSG3 in the air associated to a specific AS ID.

- (Issue1-6) RAN2 to confirm that the paging identifier is invisible at reader’s MAC layer, i.e., the reader maintains the mapping between RAN NGAP device ID used in the interface and AS ID assigned for the device in the air, to ensure that the command is toward which specific target device.

- Reference

- Report of 3GPP TSG RAN WG2 meeting#130

- R2-2505520	A-IoT MAC running CR

- R2-2505521	Summary of A-IoT MAC open issues

- TS 23.369		Architecture support for Ambient power-enabled Internet of Things

- Annex

### 6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters]) message to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The External Target Area information is specified in clause 5.3.

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this command service operation, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters]) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

NOTE:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data), AF ID, [Last Report Indication]).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data), AF ID, [Last Report Indication]).