3GPP TSG RAN WG2 #131                                                                                  R2-2505213

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Discussion on LCM for UE-sided model for BM use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#130 meeting, RAN2 discussed the open issues about the LCM for UE-sided model for beam management use case, including applicability reporting and UE side data collection, and achieved the agreements as follows [1].

Applicability reporting

Agreements - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)

Agreements

1.	When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported).

2.	The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable.

3.	The UE shall report when CSI-ReportConfig becomes not applicable

Agreements

- (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design.

- (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations).  FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free).

- (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

UE side data collection

Agreements - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

In this contribution, we will further discuss the leftover open issues for the applicability reporting and UE side data collection respectively, and also discuss the LCM for CSI prediction use case.

- Discussion

After RAN2#130 meeting, the RRC open issues were provided by rapporteur to discuss in “[POST130][026][AI PHY] 38.331 CR (Ericsson)” email discussion [2]. In this section, we will discuss the RRC open issues included in [2], and also other additional issues which not be included in [2].

2.1 Applicability reporting

- Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

From the current discussion in RAN1 and RAN2, there is a consensus that associated ID is optional configured by network in applicability reporting procedure. In previous meeting, RAN2 discussed and agreed that UE determines the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. However, it is unclear what the UE’s behaviour is when the associated ID is not provided by the network.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

Some companies propose the UE may optionally provide associated ID to network in applicability reporting if associated ID is not provided by the network, which can help NW to better understand under which condition the configuration is applicable. From our perspective, supporting such function (i.e., UE reports associated ID to network) can introduce extra complexity, e.g., we can need to consider three cases that UE cannot determine the applicability by associated ID: case1) network provides associated ID in training, but not in inference, case2) network provides associated ID in inference, but not in training, case3) network does not provide associated ID in both training and inference, reporting associated ID to network by UE seems only to address the case 1), how to solve the issue for case 2) and case 3) also need to be considered. Therefore, for a simple and unified solution, we prefer that it is up to UE implementation for all case 1), case 2) and case3), and the model monitoring can ensure the inference performance.

Proposal 1: (RRC-16) It is up to UE implementation to report the applicability (applicable/inapplicable) when the associated ID is not provided by network no matter in training or inference phase.

- Option B for applicability reporting

For option B for applicability reporting, for performing inference, it is nature that the network needs to send the full inference configuration (CSI-ReportConfig) to UE after receiving the applicability reporting from UE.

Proposal 2: For option B, NW needs to configure full inference configuration (i.e., CSI-ReportConfig) after receiving applicability reporting from UE.

After the UE receives the full inference configuration, the UE does not need to report the applicability again as it has been reported before, if the applicability change occurs, the UE can trigger UAI reporting to inform network, e.g., to update the full inference configuration.

Proposal 3: For option B, UE does not need to report the applicability after receiving the full inference configuration.

- Applicable functionalities activation for Option B

In RAN2#129 meeting, for option A, RAN2 agreed the following agreements about the applicable functionalities activation.

Agreements - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: Semi-persistent reporting, activated by MAC CE/DCI Aperiodic CSI reporting, activated by DCI

For option B, considering that the network needs to send the full inference configuration (CSI-ReportConfig) to UE after receiving the applicability reporting, therefore, for periodic CSI reporting, the UE activates the applicable functionalities when receiving the full inference configuration of the applicable functionality, for semi-persistent and aperiodic CSI report, legacy CSI framework is used, i.e., semi-persistent reporting is activated by MAC CE/DCI, aperiodic CSI reporting is activated by DCI.

Proposal 4: For option B, the UE activates the applicable functionalities as follow:

- Periodic CSI report is activated when receiving the full inference configuration of the applicable functionality;

- Semi-persistent and aperiodic CSI report are activated follow legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

  - UE side data collection

After RAN2#130 meeting, an post email discussion “[POST130][037][AI PHY] UE candidate data collection (Xiaomi_Ericsson)” [3] was hosted to discuss the UE side data collection related issues. However, there is still an FFS issue need to be discussed and clarified which not included in the post email discussion (as follow).

RAN2#128 meeting 10. FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

From our perspective, it seems to be unnecessary that the UE reports the indicator to network to indicate UE can’t perform data collection based on received configuration. On the one hand, the UE can provide the sufficient information in the data collection request procedure, e.g., preferred configuration, to make sure the network can get the sufficient information to configure the effective data collection configuration. Even if, finally, the configuration from network is invalid, the UE can trigger request again. On the other hand, if the UE can’t perform data collection based on received configuration, e.g., low power or low memory, the UE can stop the data collection, and continue to collect data when the UE has high power or sufficient memory again, or extra mechanism similar as NW side data collection will be supported.

Proposal 5: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

2.3 CSI prediction use case

- Open issue RRC-13: CSI prediction LCM framework

In previous meeting, RAN2 focused on LCM framework for beam management use case and positioning accuracy enhancement use case, the LCM framework for CSI prediction use case has not been discussed until now; it is still FFS and need to be considered. Considering that the similarity between beam management use case and CSI prediction use case, we understand that RAN2 can assume the LCM framework for beam management use case can be reused as baseline for CSI prediction use case, including e.g., the procedure and signaling for applicability reporting, UE side data collection and inference configuration.

Proposal 6: (RRC-13) RAN2 assumes the LCM framework (including e.g., applicability reporting, UE side data collection and inference configuration) for beam management use case can be reused as baseline for CSI prediction use case.

Furthermore, we need to consider the enhancement and difference for CSI prediction use case from beam management use case. For applicability reporting, the procedure and configuration signaling for Option A and Option B can be reused, however for the inference related parameters for Option B, i.e., based on which the UE can determine the applicable functionality, it can be different for beam management use case and CSI prediction use case. For beam management use case, in RAN1#121 meeting, RAN1 has achieved the following agreement, but, there is no related agreement towards to Option B for CSI prediction use case in RAN1, if Option B is supported for applicability reporting for CSI prediction use case, it can be up to RAN1 to determine the inference related parameters.

Agreement For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused: - For both BM-Case 1 and BM-Case 2: - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19 - For BM-Case 2: - TimeGap-r19, nroftimeinstance-r19, - Note: this doesn’t imply the associated ID is always present

Proposal 7: (RRC-13) For CSI prediction use case, the inference related parameters for Option B for applicability reporting (if supported) is up to RAN1.

For UE side data collection, the procedure and configuration signaling for beam management use case can be reused for CSI prediction use case, and for beam management use case, it was agreed that the network can provide the candidate data collection configuration list to UE, based which the UE provides the preference candidate data collection configuration to network, the candidate data collection configuration includes the following contents agreed in RAN2#130 meeting, based on RAN1’s agreement on UE side data collection configuration as follow.

RAN2’s agreement on candidate data collection configuration 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

RAN1’s agreement on UE side data collection configuration Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

If the procedure and configuration signaling for UE side data collection for beam management use case is supported also for CSI prediction use case, from our perspective, considering that RAN1 has no agreement specific on UE side data collection configuration parameter expect for none-CSI-r19 for CSI prediction use case, we understand the legacy CSI resource configuration is reuse for AI/ML CSI prediction, and only none-CSI-r19 is introduced to inform UE it is for UE side data collection, no CSI reporting is needed. In addition, associated ID seems not to be supported in CSI prediction use case. Thus, for candidate data collection configuration for CSI prediction use case, it can be only a CSI resource list is enough, it can be checked with RAN1.

Proposal 8: (RRC-13) For CSI prediction use case, the candidate data collection configuration (if supported) includes only a CSI resource list, and can be checked with RAN1.

Considering that RAN1 has completed the Rel-19 AI/ML PHY WI in RAN1#121 meeting, thus, RAN1 will not trigger the discussion about Proposal 7 and Proposal 8 autonomously, RAN2 needs to send an LS to RAN1 to trigger the discussion about Proposal 7 and Proposal 8 in RAN1, and also can inform RAN2’s agreement for CSI prediction use case, if any.

Proposal 9: (RRC-13) Sends an LS to RAN1 to trigger RAN1’s discussion about Proposal 7 and Proposal 8, and inform RAN2’s agreement for CSI prediction use case, if any.

- Conclusion

According to the discussion in section 2, we propose:

Applicability reporting

Proposal 1: (RRC-16) It is up to UE implementation to report the applicability (applicable/inapplicable) when the associated ID is not provided by network no matter in training or inference phase.

Proposal 2: For option B, NW needs to configure full inference configuration (i.e., CSI-ReportConfig) after receiving applicability reporting from UE.

Proposal 3: For option B, UE does not need to report the applicability after receiving the full inference configuration.

Proposal 4: For option B, the UE activates the applicable functionalities as follow:

- Periodic CSI report is activated when receiving the full inference configuration of the applicable functionality;

- Semi-persistent and aperiodic CSI report are activated follow legacy CSI framework:

- Semi-persistent reporting, activated by MAC CE/DCI

- Aperiodic CSI reporting, activated by DCI

UE side data collection

Proposal 5: No indication information from UE to network is needed when UE can’t perform data collection based on received configuration.

CSI prediction use case

Proposal 6: (RRC-13) RAN2 assumes the LCM framework (including e.g., applicability reporting, UE side data collection and inference configuration) for beam management use case can be reused as baseline for CSI prediction use case.

Proposal 7: (RRC-13) For CSI prediction use case, the inference related parameters for Option B for applicability reporting (if supported) is up to RAN1.

Proposal 8: (RRC-13) For CSI prediction use case, the candidate data collection configuration (if supported) includes only a CSI resource list, and can be checked with RAN1.

Proposal 9: (RRC-13) Sends an LS to RAN1 to trigger RAN1’s discussion about Proposal 7 and Proposal 8, and inform RAN2’s agreement for CSI prediction use case, if any.

- References

- R2-2505002    RAN2#130 Meeting Report    MCC

- R2-2505778 RRC open issues for AIML for NR air interface

- R2-2505296 Report of [POST130][037][AI PHY] UE candidate data collection (XiaomiEricsson)