3GPP TSG RAN WG2 Meeting #131                                                             R2-2505215

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Consideration on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], many agreements have been achieved for RRC open issues of NW side data collection for BM use case:

RRC-19/20/29 Reporting assistance info on logged measurements

Agreements

- (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced.  In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data).   We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it.   Notify RAN3

- Buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB instead of percentage

- UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

- No additional signaling from the UE is required when the low power issue is resolved

- No additional signaling from the UE is required when the buffer full issue is resolved

RRC-21 Time related content of logged data

Agreements

1	Data is collected on per data logging configuration basis and UE indicates data logging configuration ID.    An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.    FFS if timestamp and relative time stamp for each group is needed per “group”.

RRC-23 Cell ID in logged data

Agreement

The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

In this contribution, we further discuss the issues which are suggested companies to provide contributions to resolve, based on the open issue list in email discussion [POST130][026][AI PHY] 38.331 CR (Ericsson).

- Discussion

  - Time related content of logged data [RRC-21]

Based on the RAN2#130 agreement, it has still not been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity). This may because there have no valid measurement result to be recorded by UE in this period, e.g. due to coverage hole.

The timestamp between the specific sample/entry and the absolute timestamp could be used, similar as logged MDT. The absolute timestamp could be the time the UE receives the data collection configuration, or the first time UE stores a sample/entry at the beginning of logging.

Proposal 1: (RRC-21) The timestamp between the specific sample/entry and the absolute timestamp could be used to indicated the time gap, similar as logged MDT.

  - Dynamic activation/deactivation of data collection configurations for logging [RRC-25]

For periodic CSI reporting of UE-side data collection, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 if option A is configured in Step 3, but for option B it is FFS; For semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework, i.e. activated by lower layer signaling.

RAN2#127 made the following agreement regarding NW-side data collection:

1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

The second FFS on dynamic activation/deactivation has not yet been addressed.

For NW-side data collection, it is all by the NW intent to activate/de-activate the functionalities. Therefore RRC framework based activation/de-activation could be performed, and it can be controlled by the add/modify/release operation of the related data collection configurations, dynamic activation/deactivation is not needed.

Proposal 2: (RRC-25) Dynamic activation/deactivation of data collection configurations for logging is not needed, as it can be implicitly achieved by the add/modify/release operation of the related data collection configurations.

  - The release of logged AIML data in UE’s buffer [RRC-31]

RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, it is suggested the following can be also considered:

- Power off or deregistration

- 48 hrs after the release of logged measurement configuration

- Explicit indication from the serving gNB.

We think some of the release conditions can be the same as logged MDT, i.e. UE Power-off/ deregistration or explicit indicator from NW. But 48h timer should not be used if we only indicate data available when buffer/power satisfies the condition (not pure data available indicator). Since the measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases, receiving another NW-side data collection configuration should also not be considered as a release condition.

Proposal 3: (RRC-31) UE Power-off/ deregistration and explicit indication from the serving gNB could be considered as the release conditions of logged AIML data in UE’s buffer.

  - Available indications [RRC-29]

It is an open issue that whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched.

Since the buffer related indicator implies the UE has stored a lot of available data, there is no confusion for this indicator. For sending other available indicators, two different network actions are expected by UE:

- For indicating data available: Network fetches the data from UE;

- For indicating low power state: Network indicates UE to stop logging;

To prevent frequent reporting, there is no agreement on indicating the network the UE has data available without fulfilling buffer or power condition. Therefore the low power state indicator can be sent with or without data available indicator, but the data available indicator cannot be sent individually.

Proposal 4: (RRC-29) UE could send the low power state indicator with or without data available indicator, but the data available indicator cannot be sent individually.

  - Per use case enhancement or common for all AI use cases [RRC-34]

It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication.

For AS layer memory, it is suggested to share common memory size among all AIML use-cases. Different use cases could be configured to different UEs by network control at the same time, define separate memory is complex and unnecessary.

And since it is the NW triggered data collection, it could be the network’s intention to require all the data collected by the UE fulfill the report condition as soon as possible, so all use case could use common data available indicator or low-power indicator, and then the network could retrieve the data irrespective which use case the data belongs.

Proposal 5: (RRC-34): All use case could share common memory size, and use common data available indicator or low-power indicator.

  - Memory for NW side training data [RRC-34, RRC-38]

It is agreed that:

UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases

For logged MDT, the minimum memory size of MDT logged measurements is 64KB. Therefore 64KB could also be assumed as the minimum memory size of AI/ML training data across all use cases.

Proposal 6: (RRC-34/38) Set the minimum memory size of AI/ML training data logging to be 64KB across all use cases.

  - Data forwarding to OAM or source gNB after HO [RRC-36]

RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed.

The SON/MDT-like mechanism could be used that the NW node which receive the collected data forward it to the node based on cell index (e.g. CGI) of the serving cell or based on data collection entity ID (similar as TCE ID in MDT) provided by the OAM.

Proposal 7: (RRC-36): The SON/MDT-like mechanism could be used that the NW node which receives the collected data could forward it to the node based on cell index (e.g. CGI) of the serving cell or based on data collection entity ID provided by the OAM.

- Conclusion

In this contribution, we provide our views on the training data collection for NW side model, and the proposals are summarized as follows:

Proposal 1: (RRC-21) The timestamp between the specific sample/entry and the absolute timestamp could be used to indicated the time gap, similar as logged MDT.

Proposal 2: (RRC-25) Dynamic activation/deactivation of data collection configurations for logging is not needed, as it can be implicitly achieved by the add/modify/release operation of the related data collection configurations.

Proposal 3: (RRC-31) UE Power-off/ deregistration and explicit indication from the serving gNB could be considered as the release conditions of logged AIML data in UE’s buffer.

Proposal 4: (RRC-29) UE could send the low power state indicator with or without data available indicator, but the data available indicator cannot be sent individually.

Proposal 5: (RRC-34): All use case could share common memory size, and use common data available indicator or low-power indicator.

Proposal 6: (RRC-34/38) Set the minimum memory size of AI/ML training data logging to be 64KB across all use cases.

Proposal 7: (RRC-36): The SON/MDT-like mechanism could be used that the NW node which receives the collected data could forward it to the node based on cell index (e.g. CGI) of the serving cell or based on data collection entity ID provided by the OAM.

- References

- R2-2505002 RAN2#130 Meeting Report    MCC