3GPP TSG-RAN WG2 Meeting #131                                              	R2-2505220

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, CMCC, Ericsson, Nokia

Title:	Discussion on TEI19 UAV mobility enhancements

Agenda Item:	8.19.1

Document for:	Discussion and Decision

Introduction

In RAN#107, the initial conclusions had been endorsed for 5GA UAV enhancements as follows [1]:

Then in RAN#108 [2], further progress had been made to confirm that UAV enhancements can be submitted as TEI19 or TEI20 item as below.

Agreements on UAV - No new UAV WI - The enhancements can be submitted as TEI19 or TEI20 items in RAN2 and follow TEI process.

Considering there are already some field trials for UAV service and especially the interests from a number of operators witnessed in previous RAN plenary discussions, it may be better to support further UAV enhancements in R19. In this contribution, the corresponding RAN2 spec impact is analysed and the draft CRs are also provided in companion papers [3-6].

Discussion

IDLE/INACTIVE mobility enhancements

- Frequency Prioritization

Currently, the coverage of the terrestrial cell is mainly applicable for the UEs on the ground, which will lead to frequent cell reselection/handover for UAV UE flying at a certain altitude, as studied in TR 36.777 [7]. In order to provide better service for UAV UE, UAV dedicated aerial coverage (e.g. with up-tilted antenna and UAV specific frequency, or by UAV specific beams) could be provided, as illustrated in Figure 1.

Figure 1 UAV dedicated aerial coverage

Observation 1: UAV dedicated aerial coverage (e.g. with up-tilted antenna and UAV specific frequency, or by UAV specific beams) could provide better service for UAV UE to avoid frequent cell reselection and handover.

For connected UE, it is up to network to handover the UE to one cell with suitable aerial coverage. But for the idle/inactive UE, it is better to reselect to a cell with UAV dedicated coverage as much as possible. Hence the cell reselection priority of the UAV frequency with aerial coverage should be higher than other frequencies.

The UAV dedicated frequencies should be informed to UE. One option is to introduce an indication in SIB2 and SIB4 (per frequency) to inform UE whether the serving frequency or inter-frequency is UAV dedicated frequency. Based on the indication, the UAV UE could distinguish the UAV frequency and may prioritize these frequencies for cell reselection. Optionally, an altitude range can be configured by network to better align with the realistic aerial deployment. Accordingly, only when a UAV UE is within this altitude range, it may prioritize UAV frequency for cell reselection.

In order to avoid non-UAV UE to reselect to the UAV dedicated frequencies, the network should not set the explicit cell reselection priority for the UAV dedicated frequency as specified in TS 38.304 as below. And for UAV UEs the priority can be provided by the UAV frequency indication implicitly.

5.2.4.1 Reselection priorities handling … The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

Proposal 1: One indication is introduced in SIB2 to inform whether the serving frequency is UAV dedicated frequency, and another per frequency indication is introduced in SIB4 to inform it is a UAV dedicated frequency.

Proposal 2: UAV UE may consider the UAV dedicated frequency to be the highest priority for cell reselection. Optionally, an altitude range can be configured by network and only when a UAV UE is within this altitude range, it may prioritize UAV frequency for cell reselection.

- Beam prioritization

Altitude-based SSB measurements for RRC CONNECTED mode is supported in R18 UAV WI. If the UE is within a certain altitude range, the UE will perform measurement based on the corresponding SSB resources configured by IE ssb-ToMeasureAltitudeBasedList. Otherwise the UE will perform measurement based on the common configured SSB resources specified as follows.

ssb-ToMeasureAltitudeBasedList List of altitude-dependent ssb-ToMeasure. When the UE is within an altitude range indicated by altitudeRange, it ignores the ssb-ToMeasure (without suffix), and applies the corresponding ssb-ToMeasure-r18 if present, otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies. For each altitude range, altitudeMin indicates the minimum altitude in meters relative to sea level, altitudeMax indicates the maximum altitude in meters relative to sea level, and if included, altitudeHyst indicates hysteresis in meters for determination of the altitude range. I.e., when altitudeHyst is configured for an altitude range, the UE considers itself to have entered the range if altitudeMin ≤ UE altitude ≤ altitudeMax and after entering the range considers itself to be in the range while (altitudeMin – altitudeHyst) ≤ UE altitude ≤ (altitudeMax + altitudeHyst). For each altitudeRange, if altitudeMin is absent, value minAltitude-r18 is used and if altitudeMax is absent, value maxAltitude-r18 is used.

As for the idle/inactive UE, the similar mechanism could bring the similar benefit too, i.e., to reduce UAV UE power consumption for measurement. Specifically, the altitude-based SSB can be configured in SIB2 and SIB4 (similar to what is already supported in measurement configuration in RRC CONNECTED mode).

Proposal 3: for the idle/inactive UE, ssb-ToMeasureAltitudeBasedList is configured in SIB2 and SIB4 (for each frequency).

Different from connected UE within an altitude range, which ignores the SSB outside of the ssb-ToMeasure-r18, it may be unnecessary to prevent idle/inactive UE to perform more measurement. Therefore, for idle/inactive UE, if ssb-ToMeasureAltitudeBasedList is configured and when the UE is within an altitude range indicated by altitudeRange, it may prioritize measurements on the SSBs configured in ssb-ToMeasure-r18 (if present). Otherwise (i.e., the UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies.

Proposal 4: If ssb-ToMeasureAltitudeBasedList is configured in SIB2 or SIB4, when the idle/inactive UE is within an altitude range indicated by altitudeRange, it may prioritize the measurements on the SSB(s) configured in ssb-ToMeasure-r18 (if present). Otherwise (i.e., the idle/inactive UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the idle/inactive UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies.

CHO enhancements

Currently, the following measurement events are defined for triggering CHO:

- condEventA3/ condEventA5

- condEventA4 (For NTN and (S)CPAC)

- condEventD1/ condEventT1/ condEventD2 (for NTN)

In R18 the altitude based event Hx and AxHy are supported for UAV. The similar altitude based event could also be adopted for CHO. This can enable UAV mobility to be sufficiently considered during handover procedure, envisioning performance gain for the support of UAV mobility.

Considering up to 2 measID could be configured for one execution condition, there are two options to implement this new enhancement:

- Option 1: define conditional event Hx to trigger CHO

As up to 2 measID could be configured for one execution condition, the network could configure one altitude event e.g. H1 and one RRM event e.g. A3. Combining the two events, the UE could evaluate when to perform CHO execution. Since altitude only is not accurate enough to decide when to perform CHO, the condEventHx couldn’t be configured alone without condEventAx.

- Option 2: define conditional event AxHy to trigger CHO

The network can directly configure one AxHy event or two AxHy events as execution condition to combine the altitude factor and RRM results.

Obviously, the option 2 could cover the case supported by option 1. And Option 2 is more flexible (e.g. option 1 can only configure one altitude based event, but option 2 could configure up to 2 altitude based events e.g. A3H1 and A3H2). Hence it is sufficient to support only condEventAxHy for UAV CHO triggering.

As in legacy only condEvent3 and condEvent5 are supported for CHO, which take the signal quality of both the neighbor cell and serving cell into account, hence for condEventAxHy, only condEvenA3H1, condEventA3H2, condEvenA5H1, condEventA5H2 are needed.

Proposal 5: Define the following altitude-based CHO trigger events for UAV

- condEvenA3H1, condEventA3H2, condEvenA5H1, condEventA5H2

Conclusion

This contribution discusses the TEI19 UAV enhancement, and the corresponding observations and proposals are listed as follows:

Observation 1: UAV dedicated aerial coverage (e.g. with up-tilted antenna and UAV specific frequency, or by UAV specific beams) could provide better service for UAV UE to avoid frequent cell reselection and handover.

Proposal 1: One indication is introduced in SIB2 to inform whether the serving frequency is UAV dedicated frequency, and another per frequency indication is introduced in SIB4 to inform it is a UAV dedicated frequency.

Proposal 2: UAV UE may consider the UAV dedicated frequency to be the highest priority for cell reselection. Optionally, an altitude range can be configured by network and only when a UAV UE is within this altitude range, it may prioritize UAV frequency for cell reselection.

Proposal 3: for the idle/inactive UE, ssb-ToMeasureAltitudeBasedList is configured in SIB2 and SIB4 (for each frequency).

Proposal 4: If ssb-ToMeasureAltitudeBasedList is configured in SIB2 or SIB4, when the idle/inactive UE is within an altitude range indicated by altitudeRange, it may prioritize the measurements on the SSB(s) configured in ssb-ToMeasure-r18 (if present). Otherwise (i.e., the idle/inactive UE is within an altitude range indicated by altitudeRange and ssb-ToMeasure-r18 is absent) it measures on all SS-blocks. When the idle/inactive UE is outside all the altitude ranges indicated by altitudeRange (if any), ssb-ToMeasure (without suffix) applies.

Proposal 5: Define the following altitude-based CHO trigger events for UAV

- condEvenA3H1, condEventA3H2, condEvenA5H1, condEventA5H2

Note: The proposed UAV enhancements can be easily extended to support Urban Air Mobility (UAM) scenarios.

Reference

- RP-250812, Summary for RAN Release 20 5G-Adv, RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs

- RP-251828, Moderator's summary for Other RAN2-led topics, RAN2 Chair (Interdigital)

- R2-2505221, 38.300 CR for Introduction of UAV mobility enhancements [UAV_Mobility], CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia

- R2-2505222, 38.304 CR for Introduction of UAV mobility enhancements [UAV_Mobility], CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia

- R2-2505223, 38.306 CR for Introduction of UAV mobility enhancements [UAV_Mobility], CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia

- R2-2505224, 38.331 CR for Introduction of UAV mobility enhancements [UAV_Mobility], CATT, NTT DOCOMO, LG Electronics Inc., Kyocera, LGU+, China Telecom, NEC, SK Telecom, Qualcomm Incorporated, Ericsson, Nokia

- TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles