3GPP TSG-RAN WG2 Meeting #131	R2-2505230

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.9.2

Source: 	CATT

Title: 	Discussion on leftover issues of S&F operation

Document for: 	Discussion and Decision

Introduction

Based on the LS from CT1 [1] and last-meeting discussion in RAN2 [2], this contribution further discusses some leftover issues on supporting S&F operation.

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Discussion

2.1	On the reply LS from CT1

In RAN#129bis, RAN2 sent an LS to CT1 indicates the support of mode transition times [3] for S&F operation.

# 1 Overall description RAN2 agreed to support the time information for the transition between "store-and-forward (S&F) operation mode" and "normal mode" which can be provided by a Rel-19 IoT NTN satellite supporting S&F operation. Specifically: - RAN2 agreed that time information for the transition from current "S&F operation mode" to "normal mode" is provided in system information (i.e. SIB31). For UEs supporting S&F operation, the UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (if received) to the upper layers, e.g. to delay some NAS procedures till the feeder link is resumed. - RAN2 also agreed to introduce an indication in system information for the "normal mode" to "S&F operation mode" transition, at least for NAS use. The information on transition time for the "normal mode" to "S&F operation mode" transition (if received) is sent from AS to NAS. RAN2 assumed that whether/how the above transition time information is used by the upper layers is up to CT1. # 2 Actions ACTION: RAN2 respectfully asks CT1 to take above information into account, consider possible specification changes in NAS and provide feedback (if needed).

We receives reply LS from CT1 [1], in which CT1 indicates that they concludes the mode transition time has no impact to NAS layer. This is against with our assumption that whether/how the transition time info is used by the upper layer is up to CT1. Therefore, RAN2 should further discuss whether the mode transition time information is actually required, e.g., for AS layer operations; if yes, specify the exact scenarios in which the times are useful and how they are used; otherwise, the times will not be supported in Rel-19.

1. Overall Description: CT1 would like to thank RAN2 for their LS C1-253033 / R2-2503070 on providing S&F mode indications in system information for the transition between "normal mode" and "S&F operation mode" to NAS. CT1 discussed the topic and concludes that there is no impact to the NAS layer for the moment according to the current requirements for S&F mode provided by SA2. 2. Actions: To RAN2 group: ACTION: CT1 kindly requests RAN2 to take the above information into account.

Observation 1: CT1 indicates that the mode transition time has no impact to NAS layer.

Proposal 1: RAN2 discuss whether the mode transition time information is actually required for AS layer operations; if yes, specify the exact scenarios in which the times are useful and how they are used; otherwise, the times will not be supported in Rel-19.

2.2	On providing operation mode of neighbour cells

In last meeting, we discussed whether to provide operation mode of neighbour cells for the UE and reached a working assumption as below.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

- For operation mode:

The main obstacle to achieve consensus on this mechanism is the lack of clarity regarding the scenarios in which the neighbour cell’s operation mode actually benefits for UE. We think at least the operation mode is benefit for Rel-19 S&F non-capable UEs during cell reselection, i.e., a Rel-19 S&F non-capable UE can skip measuring on cells in S&F operation mode, because it is unable to camp on/access such a cell. For an S&F capable UE, using the neighbour cell’s operation mode information, it could perform cell reselection with prioritizing either S&F mode cells or normal mode. For instance, the UE could prioritize cells same as its current operating mode, thereby avoiding mode ping-pong and reducing associated AS/NAS signalling/procedure.

- For operation mode transition time:

We see no necessary scenario for providing neighbour cell’s operation transition time towards UE. The neighbour cell’s operation mode is expected to change rarely, so the potential benefit is unclear. Moreover, if the transition time is introduced, we need to discuss and specify the UE behaviour on using this information, which cannot be completed with only one RAN2 meeting remaining in Rel-19. Therefore, we proposed not supporting the operation mode transition time for neighbour cell.

Proposal 2: Confirm the working assumption “In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not”. The transition time is not introduced for neighbour cells.

Proposal 2a: Based on the operation mode introduced for the neighbour cells:

- Rel-19 non-S&F capable UE may deprioritize the S&F cells during cell re-selection.

- Rel-19 S&F capable UE may prioritize the cells according to its current operating mode (e.g., an UE operating in S&F mode may prioritize the S&F cells; an UE operating in normal mode may prioritize the normal cells).

Conclusion

In this contribution, some leftover issues on support of S&F operation are discussed with corresponding proposals listed as follows:

Observation 1: CT1 indicates that the mode transition time has no impact to NAS layer.

Proposal 1: RAN2 discuss whether the mode transition time information is actually required for AS layer operations; if yes, specify the exact scenarios in which the times are useful and how they are used; otherwise, the times will not be supported in Rel-19.

Proposal 2: Confirm the working assumption “In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not”. The transition time is not introduced for neighbour cells.

Proposal 2a: Based on the operation mode introduced for the neighbour cells:

- Rel-19 non-S&F capable UE may deprioritize the S&F cells during cell re-selection.

- Rel-19 S&F capable UE may prioritize the cells according to its current operating mode (e.g., an UE operating in S&F mode may prioritize the S&F cells; an UE operating in normal mode may prioritize the normal cells).

Reference

- C1-254119	Reply LS on S&F mode indications to NAS.

- RAN2#130 ChairNotes.

- R2-2503070	LS on S&F mode indications to NAS.

- 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".