3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505236

Bangalore, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Open issues on LP-WUS in IDLE and INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our considerations on some open issues of LP-WUS in IDLE and INACTIVE.

Discussion

(RRC-12) whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

According to the discussion in [1], there are four options to enable/disable LP-WUS.

- Option 1: Enabling/disabling LP-WUS per UE via NAS.

- Option 2: Enabling/disabling LP-WUS per UE via RRC Release message.

- Option 3: Disabling UE specific LP-WUS monitoring control for a particular UE by disabling CN based subgrouping method in a cell.

- Option 4: UE/gNB determines whether to enable/disable LP-WUS monitoring by comparing UE i-DRX (i,e., DRX cycle of the UE as specified in TS 38.304) with the frame-level offset between the LO and a reference PO/PF.

The main motivation of enabling/disable LP-WUS is to avoid additional latency introduced by LP-WUS brings significant impacts on service performance. Considering the value range of the frame-level offset between an LO and a reference PO/PF is from 80ms to 2000ms but the UE i-DRX cycle maybe shorter than the frame-level offset, e.g., 320ms, the additional latency introduced by LP-WUS maybe much larger than the UE i-DRX cycle.  In this case, the LP-WUS can be disabled to save the possible large latency.

Some companies argue that enabling/disabling LP-WUS is beneficial on reducing false paging probability. For LP-WUS, it was agreed that LP-WUS has maximum 31 sub-groups, Hence, false paging probability is not a big issue. And if frequent transition between MR and LR impacts the power saving, smart UEs can decide not to monitor LP-WUS with UE implementation.

Hence, from our perspective, the motivation of enabling/disabling LP-WUS is to avoid the additional latency introduce by LP-WUS.

Observation 1: The motivation of enabling/disabling LP-WUS is to avoid the additional latency introduce by LP-WUS, but not to reduce false alarm probability.

With Option 1 and Option 2, it depends on the network to decide whether to enable/disable LP-WUS per UE via dedicated signaling in advance. However, the network cannot determine UE i-DRX cycle and the frame-level offset between an LO and a reference PO/PF for LP-WUS in advance, as the camped cell may change after the UE enters RRC_IDLE or RRC_INACTIVE from RRC_CONNECTED. Hence, the network cannot have the accurate information of the latency impact introduced by LP-WUS in one cell for a UE in advance for both option 1 and option 2.

In addition, both option 1 and option 2 have impact on other WGs. With Option 1, NAS message needs to be enhanced which brings impact on SA2. In addition, CN paging message or RAN paging message between NW nodes need to indicate whether to enable/disable LP-WUS monitoring for one UE. Option 2 brings impact on RAN3, e.g. RAN node needs to notify CN whether to enable/disable LP-WUS monitoring for the UE when it releases the UE, and CN paging message or RAN paging message between NW nodes need to indicate whether to enable/disable LP-WUS monitoring for the UE. Therefore, neither Option 1 nor Option 2 is preferred.

With option 3, it is unclear how to disable LP-WUS for a UE if the UE doesn’t support CN assigned subgrouping for LP-WUS.

Considering there is no impact on other WGs in Option 4 and it can be used to address the issue mentioned in Observation 0. Therefore, option 4 is preferred.

Proposal 1: (RRC-12) RAN2 consider UE/gNB determines whether to enable/disable LP-WUS monitoring by comparing UE i-DRX (i,e., DRX cycle of the UE as specified in TS 38.304) with the frame-level offset between the LO and a reference PO/PF.

(38304-8) Whether LP-WUS is only used in the last used cell or in any cell

As LP-WUS has maximum 31 sub-groups, false paging probability is not a big issue. So the motivation to restrict that LP-WUS is only used in the last used cell is unclear. In addition, in order to support LP-WUS is only used in the last used cell, how to align the last used cell between the network and the UE needs to be considered. Additional complexity is introduced. And RAN3 need to be involved as well. We propose RAN2 to discuss whether to support this function.

Proposal 2: (38304-8) Suggest RAN2 discussing whether LP-WUS is only used in the last used cell or in any cell.

(38304-9) Subgroup ID for LP-WUS outside CN PTW

In RAN2#130, the issue was discussed without conclusion.

R2-2504677 Discussion on LP-WUS procedure and configuration OPPO discussion Rel-19 NR_LPWUS-Core Late - Noted Proposal 6 In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Same as PEI, N is used for the calculation of UE_ID based subgrouping for LP-WUS as follows in current TS 38.304 running CR:

the subgroup ID of the UE for LP-WUS is determined by the formula below: SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF

According to current description, T is the DRX cycle of RRC_IDLE state as specified in clause 7.1 in TS 38.304, no matter the UE is in RRC_IDLE state or RRC_INACTIVE state.

UEs in RRC_INACTIVE need to monitor paging outside the CN configured PTW if TeDRX, CN is configured and longer than 1024 radio frames. The following is the determination of T in clause 7.1 in TS 38.304. According the following highlighted yellow part, if TeDRX, CN is configured, how to determine the T during CN configured PTW for RRC_IDLE state is specified, but there is no specification on how to determine the outside CN configured PTW for RRC_IDLE state. Then, although UEs in RRC_INACTIVE use the DRX cycle of RRC_IDLE state as specified in clause 7.1 in TS 38.304 for the calculation of UE_ID based subgrouping for LP-WUS, it is still unclear how to determine the T outside CN configured PTW for RRC_IDLE state.

T: DRX cycle of the UE. If the UE does not operate in eDRX as defined in clause 7.4: - T is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. For L2 U2N Relay UE, T for a L2 U2N Remote UE is determined by the shortest of the UE specific DRX value provided in PC5-RRC signalling and a default DRX value broadcast in system information. In RRC_IDLE state, if the UE operates in eDRX and eDRX is configured by upper layers, i.e., TeDRX, CN, according to clause 7.4: - If TeDRX, CN is no longer than 1024 radio frames: - T = TeDRX, CN; - else: - During CN configured PTW, T is determined by the shortest of UE specific DRX value, if configured by upper layers, and the default DRX value broadcast in system information. In RRC_INACTIVE state, if the UE operates in eDRX and eDRX is configured by RRC, i.e., TeDRX, RAN (if any), and upper layers, i.e., TeDRX, CN, as defined in clause 7.4: - If both TeDRX, CN and used TeDRX, RAN are no longer than 1024 radio frames: - T = min{TeDRX, RAN, TeDRX, CN}. - If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured or used: - T is determined by the shortest of UE specific DRX value configured by RRC and TeDRX, CN. - If TeDRX, CN is longer than 1024 radio frames: - If TeDRX, RAN is not configured or used: - During CN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by the UE specific DRX value configured by RRC; - else if used TeDRX, RAN is no longer than 1024 radio frames: - During CN configured PTW, T is determined by the shortest of the UE specific DRX value, if configured by upper layers and TeDRX, RAN, and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by TeDRX, RAN; - else if used TeDRX, RAN is longer than 1024 radio frames: - During the overlapped part of CN configured PTW and RAN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information; - During CN configured PTW and outside RAN configured PTW, T is determined by the shortest of the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information; - Outside CN configured PTW and during RAN configured PTW, T is determined by the UE specific DRX value configured by RRC.

The similar issue exits in the subgroup ID for PEI. In PEI, it is clarified that in RRC_INACTIVE state with CN configured PTW the SubgroupID used outside CN PTW is the same as the SubgroupID used inside CN PTW, as shown below.

In RRC_INACTIVE state with CN configured PTW the SubgroupID used outside CN PTW is the same as the SubgroupID used inside CN PTW.

The similar clarification can be reused for LP-WUS.

Proposal 3: (38304-9) In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

(38304-10) FFS whether/how LP-WUS with SDT is supported

In RAN2#130, the issue was discussed without conclusion.

R2-2504264 LP-WUS in IDLE and INACTIVE Nokia discussion Rel-19 NR_LPWUS-Core - Noted Proposal 3: LP-WUS UE may support MO-SDT and/or MT-SDT Proposal 4: The UE can initiate MO-SDT while UE is monitoring LP-WUS Proposal 5: NW can initiate MT-SDT while UE is monitoring LP-WUS Discussion - Ericsson think intention is OK and want to think more. - vivo think it is naturally support and there is no impact to the spec. Nokia think SDT has related rsrp threshold and there may be impact if UE is monitoring LPWUS. vivo do not see issue. - QC think in this case UE should wake up the MR and then initiate SDT based on current procedure. ZTE, LG E agree.

As mentioned in [2], SDT is initiated only if the DL RSRP based on MR is above a configured threshold. But for UE normal state transition to RRC_CONNECTED, DL RSRP based on MR needs to be considered during Random Access procedure as well. In addition, according to RAN1 agreements, the wake-up delay for a UE is at least 70ms. After the UE wakes up from LR to MR, DL RSRP based on MR has already been obtained. Hence, there is no problem for a UE monitoring LP-WUS to determine whether to initiate SDT procedure.

Observation 2: DL RSRP based on MR needs to be considered not only in conditions for initiating SDT but also during Random Access procedure. There is no problem for a UE monitoring LP-WUS to determine whether to initiate SDT procedure based on DL RSRP.

According to the following highlighted part in TS 38.331, during SDT procedure the UE is not required to monitor neither a CN paging nor a RAN paging except the case that SDT procedure is ongoing and T319a is not running while CG-SDT is selected and if extended CG-SDT periodicity is configured.

4.2.1 UE states and state transitions including inter RAT //skip - The UE: - Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5); - While T319a is running, monitors control channels associated with the shared data channel to determine if data is scheduled for it; - While SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI except if the UE is acting as a L2 U2N Remote UE; - While SDT procedure is not ongoing, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI, except if the UE is acting as a L2 U2N Remote UE; //skip

According to the RAN1#121 agreement in RAN1 [3], UE is not able to operate with LR and MR simultaneously in Rel-19. Hence, when SDT procedure is ongoing,

- if T319a is not running and if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), the UE needs to monitor a Paging channel for CN paging and RAN paging. In this case, the UE may monitor LP-WUS according to exiting procedures in TS 38.304. No additional specification impact is required.

- In other cases, the UE needs to operate with MR for SDT procedure. In this case, the UE is not able to operate with LP-WUS monitoring in RRC_IDLE/INACTIVE. No additional specification impact is foreseen.

Observation 3: When SDT procedure is ongoing,

- if T319a is not running and if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), the UE needs to monitor a Paging channel for CN paging and RAN paging. In this case, the UE may monitor LP-WUS according to exiting procedures in TS 38.304. No additional specification impact is required.

- In other cases, the UE needs to operate with MR for SDT procedure. In this case, the UE is not able to operate with LP-WUS monitoring in RRC_IDLE/INACTIVE. No additional specification impact is foreseen.

Based on observation 2 and observation 3, there is no problem and no specification impact for a UE supporting both LP-WUS and SDT. SDT can reduce signaling overhead while LP-WUS can reduce UE power consumption. Hence, it is beneficial UE supporting LP-WUS in IDLE/INACTIVE can support MO-SDT and/or MT-SDT. And there is no additional specification impact to support both features.

Proposal 4: (38304-10) UE supporting LP-WUS in IDLE/INACTIVE can support MO-SDT and/or MT-SDT. There is no additional specification impact to support both features.

Conclusion

According to the analysis in Section 2, we observe and propose:

Observation 1: The motivation of enabling/disabling LP-WUS is to avoid the additional latency introduce by LP-WUS, but not to reduce false alarm probability.

Observation 2: DL RSRP based on MR needs to be considered not only in conditions for initiating SDT but also during Random Access procedure. There is no problem for a UE monitoring LP-WUS to determine whether to initiate SDT procedure based on DL RSRP.

Observation 3: When SDT procedure is ongoing,

- if T319a is not running and if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), the UE needs to monitor a Paging channel for CN paging and RAN paging. In this case, the UE may monitor LP-WUS according to exiting procedures in TS 38.304. No additional specification impact is required.

In other cases, the UE needs to operate with MR for SDT procedure. In this case, the UE is not able to operate with LP-WUS monitoring in RRC_IDLE/INACTIVE. No additional specification impact is foreseen.

Proposal 1: (RRC-12) RAN2 consider UE/gNB determines whether to enable/disable LP-WUS monitoring by comparing UE i-DRX (i,e., DRX cycle of the UE as specified in TS 38.304) with the frame-level offset between the LO and a reference PO/PF.

Proposal 2: (38304-8) Suggest RAN2 discussing whether LP-WUS is only used in the last used cell or in any cell.

Proposal 3: (38304-9) In RRC_INACTIVE state with CN configured PTW, the SubgroupID for LP-WUS used outside CN PTW is the same as the SubgroupID used inside CN PTW.

Proposal 4: (38304-10) UE supporting LP-WUS in IDLE/INACTIVE can support MO-SDT and/or MT-SDT. There is no additional specification impact to support both features.

Reference

- R2-2505381, Summary of [Post130][222][LPWUS] Potential solution to support enabling/disabling LP-WUS monitoring in IDLEI/NACTVE per UE (Huawei)

- R2-2504264, LP-WUS in IDLE and INACTIVE, Nokia, RAN2#130

- R1-2504888, Reply LS on LP-WUS in RRC_CONNECTED