3GPP TSG-RAN WG2 Meeting #131	R2-2505239

Bangalore, India  Aug 25th – 29th , 2025

Agenda item:	8.1.2.2

Source:	LG Electronics Inc.

Title:	Remaining issues on Functionality based LCM for BM

Document for:	Discussion and Decision

1. Introduction

During the RAN2 #130 meeting, RAN2 reached the following agreements.

- Agreements on Applicability Report 1. Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) 2. Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified) 3. When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). 4. The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable. 5. The UE shall report when CSI-ReportConfig becomes not applicable 6. (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design. 7. (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations). FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free). 8. (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. - Agreements on UE side data collection 1. The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 2. For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

This contribution aims to address issued regarding applicable functionality reporting and data collection for UE sided training.

2. Discussion

Remaining Open issues

Open issue RRC-1: Cause of inapplicability

Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec. This issue refers to the RAN2#129bis agreement: Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec. The issue is captured as an editor’s note in the endorsed RRC running CR [R2-2504349], clause 5.3.5.3. Proposed resolution: This open issue was resolved by the RAN2#130 agreement below. The remaining FFS can be addressed directly in the RRC running CR, where companies can provide comments to the text suggestions made by the rapporteur. If it is not possible to conclude from the running CR discussion, this FFS can be discussed in the next RAN2 meeting based on a proposal from the rapporteur. 1 Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified).

The purpose of the UE indicating inapplicability with a release flag is to inform the network that configuration can no longer be used. Currently, the differentiated network behaviour based on causes is unclear. Also, defining and managing cause information increases specification complexity and implementation burden on both UE and network. Since the network can treats all causes the same by releasing the configuration, this added complexity is unnecessary.

Proposal 1. [RRC1] UE includes a release flag only when requesting configuration release without specifying an explicit cause, e.g., due to model unavailability

Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

Issue description: Based on the RAN2 agreements below, it is not clear whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations stored at the UE, or whether it can contain only updates for the inference configurations for which the UE already reported the applicability status in the past. This open issue refers to the RAN2#129 agreements: 3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause. In the rapporteur’s understanding, there are two interpretations of the agreements above: - The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates. - The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

During the email discussion on running CR [2], the contents of the applicability report had two interpretations (i.e., full applicability or partial/updated applicability in RRCReconfigurationComplete).

- O1. The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates.

- O2. The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates.

At least for option A, delta configuration is possible because applicable configuration can be set via the ToAddModeList or the ToReleaseList types. If there are no changes in applicability since the previous reporting, the UE can report only the applicability related to the added and/or modified configurations. From this perspective, O1 may cause unnecessary signaling overhead by repeatedly reporting unchanged applicability information. On the other hand, O2 can avoid this redundancy by reporting only changed applicability, which improves efficiency.

Proposal 2. [RRC44] Applicability report via RRCReconfigurationComplete only includes applicability changes from the previous report (including applicability based on newly added or modified configurations)

Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

Issue description: RAN2 agreed that the UE should activate a periodic inference configuration in CSI-ReportConfig only if it reports that it is applicable. There are several possibilities to capture this in the specifications, as commented by several companies, and it should be discussed how to best do this. Furthermore, it should be clarified if the solution can be captured only in RRC specifications or whether RAN1 should also capture (part of) the solution. This open issue refers to the RAN2#129 agreement: If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).. Some solutions suggested by companies for several aspects are: - Where exactly in RRC to capture the solution: - In Section 5.3.5.3 when setting the content of RRCReconfigurationComplete, or - In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers - What to write in RRC specifications: - Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or - Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation. Furthermore, there is an incoming LS [R4-2508085], which is discussing this issue, so it is suggested that according to this LS the companies bring contributions about whether: 1) the applicable functionalities are already activated before reporting applicable functionalities via RRCReconfigurationComplete; or 2) UE starts to activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete. In particular, related to the periodic reporting, according to Rapporteur point of view, it should be taken into account that the gNB cannot know whether the UE is sending L1 prediction reports or not, until the NW receives successfully the applicability report from the UE. Until then, if the UE is allowed to multiplex UCI (with CSI inference predictions) in PUCCH or PUSCH, the NW will try to decode the multiplexed information, which will turn into decoding issues if the inference predictions are not sent by the UE and this will impact also other decoded information from the UE. So a standardized behaviour is needed in order to avoid the gNB incurring in such ambiguity, e.g: a) the UE always sends a L1 report in UCI for periodic CSI-ReportConfig, even if the configuration is not active/applicable; b) the UE does not multiplex UCI containing inference predictions and specify that the configuration is activated only after finishing sending RRCReconfigurationComplete. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

Regarding periodic CSI reporting, it may be unclear when the UE should start sending CSI reports. If the UE begins measurement and reporting immediately after sending the RRCReconfigurationComplete message, the initial reports may be inaccurate if the network has not yet started transmitting the necessary reference signals (e.g., CSI-RS). In such cases, the UE could measure before the reference signal is available and report CSI values based on invalid data.

Although the network can transmit reference signals regardless of applicability, it does not know when the UE will start periodic CSI reporting. Therefore, the network would need to perform blind decoding until it receives the RRCReconfigurationComplete message, which results in unnecessary processing overhead and potential false detections.

Observation 1. [RRC45] If the UE performs measurement and reporting immediately after sending RRCReconfigurationComplete, the initial reported values may not be accurate if the network has not yet started transmitting the necessary reference signals (e.g., CSI-RS)

Observation 2. [RRC45] Although the network can transmit the reference signal, since the network does not know when the UE starts reporting periodic CSI reporting, the network should perform blindly decoding until it receives RRCReconfigurationComplete message

To resolve the issues, the UE can start periodic CSI reporting upon successfully reporting applicability report, i.e., upon receiving acknowledgement from the network for the RRCReconfigurationComplete message.

Proposal 3. [RRC45] UE starts periodic CSI reporting upon successfully reporting applicability report, i.e., upon receiving acknowledgement from the network for the RRCReconfigurationComplete message. Previous agreement can be revised by adding “successfully”

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon successfully reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

Issue description: Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other. The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability. One solution is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

From a signaling perspective, there is no inherent issue with supporting both options concurrently. However, the motivation for simultaneous use of both options within a single use case remains unclear. Option A and Option B represent different trade-offs between signalling simplicity and configuration efficiency.

For the same use cases, when both option A and option B reports coexist, several issues may arise:

- Duplication/Conflicting Report: If option A and option B contain the same items, the UE may report different values for them, creating ambiguity for the network on which value to trust

- Network State Inconsistency: If the UE’s state changs after reporting B but before the network applies the corresponding option A configuration, the network may apply an outdated configuration

- Increased Reporting Overhead: Reporting changs for both option A and Option B, it can lead to significant signaling overhead

Therefore, for at least one-use case, it is suggested that the UE provides either the applicability of inference related parameter set or inference configuration. If more than one use case is present, the network can configure each use case with its preferred option independently, provided that both options are not applied to the same use case simultaneously.

Proposal 4. [RRC48] For at least one-use case, the network configures either (i) the inference configuration (Option A) or (ii) the applicability of the inference-related parameter set (Option B), i.e., not both options for the same use case

3. Conclusion

Open issue RRC-1: Cause of inapplicability

Proposal 1. [RRC1] UE includes a release flag only when requesting configuration release without specifying an explicit cause, e.g., due to model unavailability

Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

Proposal 2. [RRC44] Applicability report via RRCReconfigurationComplete only includes applicability changes from the previous report (including applicability based on newly added or modified configurations)

Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

Observation 1. [RRC45] If the UE performs measurement and reporting immediately after sending RRCReconfigurationComplete, the initial reported values may not be accurate if the network has not yet started transmitting the necessary reference signals (e.g., CSI-RS)

Observation 2. [RRC45] Although the network can transmit the reference signal, since the network does not know when the UE starts reporting periodic CSI reporting, the network should perform blindly decoding until it receives RRCReconfigurationComplete message

Proposal 3. [RRC45] UE starts periodic CSI reporting upon successfully reporting applicability report, i.e., upon receiving acknowledgement from the network for the RRCReconfigurationComplete message. Previous agreement can be revised by adding “successfully”

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon successfully reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

Proposal 4. [RRC48] For at least one-use case, the network configures either (i) the inference configuration (Option A) or (ii) the applicability of the inference-related parameter set (Option B), i.e., not both options for the same use case

4. Reference

[1] R2-250xxxx - RRC open issues AIML PHY

[2] R2-25xxxxx - Draft RRC running CR for AIML PHY_v16_Rapp