# 3GPP TSG-RAN WG2 Meeting #131										                      R2-2505266

# Bengaluru, India, August 25th - 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2097 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SP positioning SRS transmission with frequency hopping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SP positioning SRS frequency hopping support was agreed in RAN1#114 and the exisiting MAC CE mechanism was agreed to be used. However, during RAN1#121 it was concluded that the existing mechanism has an issue due to SRS-PosTx-Hopping not being associated with a BWP ID: Conclusion Companies are expected to bring contributions to RAN2 to fix the issue described in R1-2504787 in RAN2 specifications. The current SP Positioning SRS Activation/Deactivation MAC CE contains a BWP ID field which means the MAC CE is not able to point to the positioning SRS resource set configured by SRS-PosTx-Hopping. Therefore, a change is needed in order to complete SP positioning SRS frequency hopping support. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a frequency hopping indication (FHI) field with its corresponding description in the SP Positioning SRS Activation/Deactivation MAC CE to enable the support of SP positioning SRS frequency hopping. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: RedCap positioning, SP positioning SRS transmission with frequency hopping Inter-operability RRC Resume condition: - If UE implements as per the CR and network does not, the network cannot enable the support of SP positioning SRS frequency hopping. - If network implements as per the CR and UE does not, UE cannot decode the frequency hopping indication (FHI) field in the SP Positioning SRS Activation/Deactivation MAC CE to enable support of SP positioning SRS frequency hopping. - If one UE implements the CR and the other UE does not, no inter-operability is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of SP positioning SRS frequency hopping. Not implementing WI agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==================================================CHANGE BEGINS============================================================

#### 6.1.3.36	SP Positioning SRS Activation/Deactivation MAC CE

The SP Positioning SRS Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP Positioning SRS resource set. The field is set to 1 to indicate activation, otherwise it indicates deactivation;

-	Positioning SRS Resource Set's Cell ID: This field indicates the identity of the Serving Cell, which contains activated/deactivated SP Positioning SRS Resource Set. If the MAC CE is used for activation/deactivation of the SP positioning SRS transmission in RRC_INACTIVE, this field indicates the identity of the cell from where positioning SRS configuration was received. If the C field is set to 0, this field also indicates the identity of the Serving Cell which contains all resources indicated by the Spatial Relation for Resource IDi fields, if present. The length of the field is 5 bits;

-	Positioning SRS Resource Set's BWP ID: This field indicates a UL BWP as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9], which contains activated/deactivated SP Positioning SRS Resource Set. If the C field is set to 0, this field also indicates the identity of the BWP which contains all resources indicated by the Spatial Relation for Resource IDi fields, if present. If the MAC CE is used for activation/deactivation of the SP positioning SRS transmission in RRC_INACTIVE and the SP positioning SRS is configured outside the initial BWP, this field should be ignored by UE. The length of the field is 2 bits;

-	C: This field indicates whether the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) within the field Spatial Relation for Resource ID i are present, except for Spatial Relation Resource IDi with DL-PRS or SSB. When A/D is set to 1, if this field is set to 1, the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) in the field Spatial Relation for Resource IDi are present, otherwise if this field is set to 0, they are not present. When A/D is set to 0, this field is always set to 0 that they are not present;

-	SUL: This field indicates whether the MAC CE applies to the NUL carrier or SUL carrier configuration. This field is set to 1 to indicate that it applies to the SUL carrier configuration, and it is set to 0 to indicate that it applies to the NUL carrier configuration;

-	Positioning SRS Resource Set ID: This field indicates the SP Positioning SRS Resource Set identified by SRS-PosResourceSetId as specified in TS 38.331 [5], which is to be activated or deactivated. The length of the field is 4 bits;

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. the A/D field is set to 1. M is the total number of Positioning SRS resource(s) configured under the SP Positioning SRS resource set indicated by the field Positioning SRS Resource Set ID. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within. The fields within Spatial Relation for Resource IDi are shown in Figures 6.1.3.36-2 to 6.1.3.36-5 for the 4 types of Spatial Relations for Resource IDi;

-	S: This field indicates whether the fields Spatial Relation for Resource IDi for the positioning SRS resource i within the positioning SRS resource set are present. If the field is set to 1, the fields Spatial Relation for Resource IDi are present; otherwise, they are absent;

- 	FHI: If the UE is configured with SRS-PosTx-Hopping, this field indicates whether this MAC CE is used for activation/deactivation of the SP positioning SRS resource set configured by SRS-PosTx-Hopping. If this field is set to ‘1’, this MAC CE is used for activation/deactivation of the SP positioning SRS resource set configured by SRS-PosTx-Hopping, and the UE should ignore the BWP ID field in this MAC CE. If this field is set to ‘0’, this MAC CE is not used for activation/deactivation of the SP positioning SRS resource set configured by SRS-PosTx-Hopping. If the UE is not configured with SRS-PosTx-Hopping, the R bit is present instead;

-	R: Reserved bit, set to 0.

Figure 6.1.3.36-1: SP Positioning SRS Activation/Deactivation MAC CE

====================================================== CHANGE ENDS=========================================================