3GPP TSG RAN WG2 Meeting #131                                                       R2-2505267

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.12.2

Source:	Ofinno

Title:	Enhancements for Asymmetric DL sTRP and UL mTRP

Document for:	Discussion and Decision

# Introduction

The WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#102 meeting. The objectives related to this agenda item are listed below:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present our view on the enhancement of asymmetric DL sTRP and UL mTRP scenario.

# Open issues

## Asymmetric UL mTRP in LTM cell switch

Regarding sDCI mTRP 2TA, the following agreement have been reached in RAN2#130.

RAN2#130 Agreements 2TA operation is supported for Rel-19 single-DCI mTRP without the restriction that coresetPoolIndex needs to be configured with more than one value, and single-DCI mTRP 2TA is applied to both the scenarios that PL offset is configured and PL offset is not configured.

According to RAN2 #130 agreement, the use of two TAs for asymmetric DL sTRP/UL mTRP are supported in two cases i) with PL offset, and ii) without PL offset. When the PL offset is not configured, the UL TRP(s) may transmit SSBs.

In the absence of PL offset configuration, all Rel-18 2TA procedures remain applicable, and the Rel-18 LTM cell switch procedure which does not support PL offset for LTM candidate TCI states can also be reused. In another words, supporting Rel-19 sDCI mTRP 2TA without PL offset configuration for Rel-18 LTM cell switch has not MAC impact.

- sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

Asymmetric sDCI mTRP is scoped as intra-DU in the WID [1]. In our view, the PL offset can be obtained and preconfigured in advance for intra-DU scenarios. For RACH-less LTM, the UE accesses the target cell using a configured grant. The UE selects the configured grant occasion associated with the TCI state indicated in the LTM cell switch command. If the indicated candidate TCI state is configured with a PL offset, the UE applies it for the first PUSCH transmission in the RACH-less LTM cell switch.

- Support PL offset being configured for a candidate TCI state indicated in the LTM cell switch command (e.g., the PL offset can be applied for the first PUSCH transmission in RACH-less LTM cell switch.)

# Remaining issues

## Expiry of Time alignment timer

In the RAN2#123 and RAN2#123bis meetings, the following agreements have been reached regarding the clearing of configured uplink grants and PUSCH resources for semi-persistent CSI reporting upon expiry of TAT associated with one of the two TAGs of a serving cell (e.g., SCell, SpCell).

RAN2#123 Agreements Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired. 1. not perform any uplink transmission except the Random Access Preamble and MSGA transmission; 2. flush all HARQ buffers; 3. notify RRC to release PUCCH, if configured; 4. notify RRC to release SRS, if configured; 5. clear any configured downlink assignments and configured uplink grants; 6. clear any PUSCH resource for semi-persistent CSI reporting; 7. maintain NTA (defined in TS 38.211 [8]) of this TAG; 8. consider all running timeAlignmentTimers as expired. RAN2#123bis Agreements The following are taken as baseline - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired, i.e., 2 is not applied. - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired, i.e., 2 is not applied.

The following highlighted texts in TS 38.321 [2] capture the above agreements:

TS 38.321 [2] 5.2 Maintenance of Uplink Time Alignment 1> when a timeAlignmentTimer expires: … 3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 4> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

In our view, RAN1 does not restrict the application of multi-TRP uplink schemes, such as Multi-TRP PUSCH/PUCCH repetitions and simultaneous multiple-panel PUSCH/PUCCH transmissions, in Rel-19 asymmetric DL sTRP/UL mTRP scenarios. That is, the UE may transmit repetitions of a PUSCH or PUCCH transmission to both the anchor TRP and the UL-only TRP, or it may perform simultaneous PUSCH/PUCCH transmission using two panels (e.g., via multipanelSchemeSDM or multipanelSchemeSFN) targeting both TRPs (as shown in Figure 1 below).

- : STxMP schemes

According to the RAN2#123 and RAN2#123bis agreement, under the Rel-18 multi-DCI multi-TRP scheme, if the TAT of one TAG expires while another TAG's TAT is running, the UE clears any configured uplink grant and any PUSCH resource for semi-persistent CSI reporting associated with the expired TAG because the configured uplink grants and the PUSCH resources for semi-persistent CSI reporting should be associated with the same TAG of a TRP. In the Rel-18 multi-DCI multi-TRP scheme, a configured uplink grant and a PUSCH resource for semi-persistent CSI reporting cannot be associated with both TAGs because the TDM repetitions and SDM/SFN simultaneous transmissions necessitate close coordination between the two TRPs, which is a characteristic inherent in the single-DCI multi-TRP scheme. Conversely, the multi-DCI multi-TRP scheme is based on a non-ideal backhaul, which imposes limitations on the coordination achievable between the TRPs, thus precluding the support of the TDM repetitions and the SDM/SFN simultaneous transmissions.

However, in the Rel-19 single-DCI multi-TRP scheme, the difference is that a configured uplink grant and a PUSCH resource for semi-persistent CSI reporting can be associated with both TAGs of both TRPs (e.g., when applyIndicatedTCI-State is set to ‘both’, or when the SRS resource set indicator in the DCI activating the semi-persistent CSI reporting is set to ‘10’ or ‘11’). In such cases, it may be inefficient or unnecessary for the UE to clear the configured uplink grant or PUSCH reporting resources associated with the TAG with expired TAT, particularly if the other TAG with a TAT running. The UE should only clear the configured uplink grant or PUSCH resource for semi-persistent CSI reporting if both the TATs of both TAGs are expired. In our view, this does not have MAC specification impact.

However, there is one particular case that warrants careful consideration, i.e., when the UE is configured with a multi-panel simultaneous uplink transmission SDM scheme (e.g., multipanelSchemeSDM) for PUSCH. For example, some layers of the PUSCH transmissions are mapped to the TCI state associated with a first TAG with expired TAT, while the other layers of the PUSCH transmissions are mapped to the TCI state associated with a second TAG with running TAT. Based on the current MAC behavior, the UE only clears the configured uplink grant or PUSCH resource for semi-persistent CSI reporting when the two TATs of the two TAGs are expired. Since one of the two TAGs is not valid, the network may not be able to successfully decode the partial PUSCH transmissions that composed only of the layers corresponding to the still-valid TAG. It’s better to clear the configured uplink grant or PUSCH resource for semi-persistent CSI reporting even if only one of the two TATs of the two TAGs is expired (i.e., if at least one activated TCI state for the PUSCH resource is associated with the TAG of the expired TAT).

A text proposal is shown below:

Text Proposal for TS 38.321 5.2 Maintenance of Uplink Time Alignment 1> when a timeAlignmentTimer expires: … 3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> if this Serving Cell is configured with multipanelSchemeSDM: 5> clear any configured uplink grant, if at least one activated TCI state for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 5> clear any PUSCH resource for semi-persistent CSI reporting, if at least one activated TCI state for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> else: 5> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 5> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

- When a serving cell is configured with multi-panel simultaneous uplink transmission SDM scheme for PUSCH (e.g., multipanelSchemeSDM), the UE should clear any configured uplink grant if at least one activated TCI state for the configured uplink grant is associated with the TAG of the expired TAT.

- When a serving cell is configured with multi-panel simultaneous uplink transmission SDM scheme for PUSCH (e.g., multipanelSchemeSDM), the UE should clear any PUSCH resource for semi-persistent CSI reporting if at least one activated TCI state for the PUSCH resource is associated with the TAG of the expired TAT.

# Conclusion

In this contribution, we make the following proposals.

- sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

- Support PL offset being configured for a candidate TCI state indicated in the LTM cell switch command (e.g., the PL offset can be applied for the first PUSCH transmission in RACH-less LTM cell switch.)

- When a serving cell is configured with multi-panel simultaneous uplink transmission SDM scheme for PUSCH (e.g., multipanelSchemeSDM), the UE should clear any configured uplink grant if at least one activated TCI state for the configured uplink grant is associated with the TAG of the expired TAT.

- When a serving cell is configured with multi-panel simultaneous uplink transmission SDM scheme for PUSCH (e.g., multipanelSchemeSDM), the UE should clear any PUSCH resource for semi-persistent CSI reporting if at least one activated TCI state for the PUSCH resource is associated with the TAG of the expired TAT.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024

- TS 38.321 V18.6.0; Medium Access Control (MAC) protocol specification.