3GPP TSG-RAN WG2 Meeting #131	R2-2505276
Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.2

Source:	Quectel

Title:	Some details on OD-SSB for NES cell

Document for:	Discussion, Decision

Introduction

The activation of OD-SSB is discussed in RAN2#130 meeting, with follow agreements [1]:

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

In this contribution, some details on on-demand SSB reception are discussed.

Discussion

The frequency of OD-SSB

Per R2 agreement, RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported. However, regarding this agreement, some details have not been clarified, which may lead to ambiguity. Specifically, there are two interpretations exist:

- Option 1:​​ Multiple OD-SSBs for an SCell are located entirely on a single, fixed frequency point for different activation MAC CEs.

- Option 2:​​ Multiple OD-SSBs for an SCell can be located on different frequency points for different activation MAC CEs.

As illustrated in figure 1, the first activation MAC CE indicates that OD-SSB is transmitted on frequency point f1, while the second activation MAC CE may indicate that OD-SSB is transmitted on a different frequency point f2.

Figure 1: ​​variable frequency points of multiple OD-SSBs for different activation MAC CEs

Observation 1:	It needs to be clarified whether the frequency point of multiple OD-SSBs for an SCell can be variable.

For option 1, only one frequency value shall be included in the RRC message, and it shall not be indicated via MAC CE. This design facilitates network configuration and simplifies implementation.

For option 2, configuring variable frequency points will increase signalling overhead, which is not expected.

Therefore, we prefer option 1.

Proposal 1:	Multiple OD-SSBs for an SCell are located entirely on a single, fixed frequency point for different activation MAC CEs.

RAN1 has endorsed a list of new higher layers parameters and/or updates on the existing higher-layer parameters for the following Rel-19 work items [2] in which the configuration index is associated with a set of parameters, namely ​​od-ssb-Periodicity​​, ​​od-ssb-PositionsInBurst​​, and ​​od-ssb-nrofBurst​​, which collectively define the ​​ time domain configurations of OD-SSB​​.

How to design the message for these parameters with frequency configuration?  Two distinct approaches as illuminated in figure 2:

Figure 2: designing for RRC message

- Alternative 1:​​ The configuration index is outside the SCell configuration.

For this alternative, the RRC signalling content is clean and simple. All the configuration indexes could be mapped to every SCell via MAC CE, which leads to inefficient utilization of MAC CE bits.

- Alternative 2:​​ The configuration index is within the SCell configuration.

As for alternative 2, the configuration indexes shall be repeated for every SCell, so the RRC signalling content is redundant. The benefit of this design is the less bit requirement for MAC CE. However, if an SCell supports all configuration indices, the issue of inefficient MAC CE bits utilization persists.

In RAN2#130 meeting, RAN2 has confirmed that TP in [3] will be baseline for the running CR. According to [3], the MAC CE contains 7 bits to represent the configuration index, which is sufficient. Therefore, we prefer to choose alternative 1.

Proposal 2:	The configuration index is outside the SCell configuration, and the frequency of each SCell can be associated with all the configuration indices.

Change of OD-SSB configuration index

In RAN2#130 meeting, RAN2 agreed that the OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. For the example in figure 3, UE receives the first MAC CE which indicates the OD-SSB shall be transmitted as pattern A in SCell-C4 for a duration. Then, UE receives the second MAC CE to indicates the OD-SSB in another SCell-C7.  How to deal with the configuration index for SCell-C4 in the second MAC CE shall be clarified.

If the configuration index for SCell-C4 is included in the second MAC CE, whether the value shall be same as that in the first MAC CE? Considering the flexibility processing for gNB, it is not needed to restrict it in specification.

Observation 2:	For an activated OD-SSB SCell, whether the configuration index can be changed within the duration of one activation has not been discussed.

Figure 3: Change of ConfigIndex of OD-SSB

For the second MAC CE in figure 3, if the UE receives the same configuration index for SCell-C4, it considers that SSB will continue to be transmitted according to the existing configuration index. If a different configuration index is received, as in figure 3, the configuration index for SCell-C4 is modified from A to B, the UE considers that upon the new configuration takes effect, SSB will no longer be transmitted in the SSB occasions corresponding to the original configuration index A.

Upon receiving a different configuration index, the UE considers that the new configuration index to be effective from time t1​​ and shall monitor SSB in the SSB occasions corresponding to the new configuration index.

Proposal 3:	Upon receiving a different configuration index, the UE considers that the new configuration index to be effective from time t1.

Proposal 4:	Once the new configuration index takes effect, the UE shall monitor SSB in the SSB occasions corresponding to the new configuration index.

Conclusion

The following proposals are provided for on-demand SSB reception on SCells:

Observation 1:	It needs to be clarified whether the frequency point of multiple OD-SSBs for an SCell can be variable.

Observation 2:	For an activated OD-SSB SCell, whether the configuration index can be changed within the duration of one activation has not been discussed.

Proposal 1:	Multiple OD-SSBs for an SCell are located entirely on a single, fixed frequency point for different activation MAC CEs.

Proposal 2:	The configuration index is outside the SCell configuration, and the frequency of each SCell can be associated with all the configuration indices.

Proposal 3:	Upon receiving a different configuration index, the UE considers that the new configuration index to be effective from time t1.

Proposal 4:	Once the new configuration index takes effect, the UE shall monitor SSB in the SSB occasions corresponding to the new configuration index.

References

Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta

R1-2503155, “Consolidated Rel-19 higher layers parameters list Post RAN1#120bis”, RAN1#120bis

R2-2504280, “On demand SSB transmission for SCell”, InterDigital