3GPP TSG-RAN WG2 #131	R2-2505294

Bengaluru, India, August 25th - 29th, 2025

Source:                    	DENSO CORPORATION

Title:  	Remaining consideration on Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2		Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#130[2], the following agreements are made for S&F operation:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

This paper attempts to investigate about remaining consideration on S&F operation based on the above objectives and agreements.

- Discussion

2.1 Neighbour cells’ information

In the last meeting (RAN2#130), indication for neighbour cells’ operation mode was marked as working assumption. We think the working assumption should be confirmed, because the indication is useful for UE to have suitable choices for cell selection, e.g.) prioritise connecting to normal mode cell, select S&F mode cell if the communication characteristic is delay tolerant.

Proposal 1: RAN2 to confirm the working assumption of indication for neighbour cells’ operation mode.

Besides the operation mode, it was treated as FFS whether the transition time information of neighbour cells is also included. We think the transition time information is also useful to select a cell with a longer remaining time in normal mode, so that frequent cell change could be avoided. Therefore, we would like to propose that neighbour cells’ mode transition time information could also be provided to UE.

Proposal 2: Neighbour cells’ mode transition time information could also be provided to UE

However, there might be cases where it is difficult for a satellite to update the S&F related information for neighbour cells frequently. Therefore, we think that it can be up to network implementation whether S&F related information for neighbour cells is included in system information.

Proposal 3: It can be up to network implementation whether S&F related information for neighbour cells is included in system information

2.2 Coexistence of transition time information and t-Service

In the last meeting (RAN2#130), it was agreed that the time information for switching to S&F mode is indicated by using the agreed time information for switching to normal mode, and the switching direction is determined by operation mode indication. For Rel-19 UE, we have no concern with the agreement, however, we have a concern for legacy UE. From legacy UE point of view, starting S&F mode equals to NTN service stop, since non-S&F capable UE is barred when the satellite is in S&F mode. Therefore, to inform that legacy UE will not be able to use the NTN service, the satellite may need to set S&F mode starting time in t-Service also. However, in that case, Rel-19 (S&F capable) UE may not be able to distinguish whether NTN service will actually stop or not, since a value is set in t-Service. To avoid this issue, we would like to propose to clarify that S&F capable UE can ignore t-Service if the value of t-Service is same as the time information for switching to S&F mode.

Observation 1: Satellite may need to set same value of S&F mode starting time in t-Service also, since starting S&F mode equals to NTN service stop from legacy UE point of view.

Proposal 4: RAN2 to clarify that S&F capable UE can ignore t-Service, if the value of t-Service is same as the time information for switching to S&F mode

- Summary and proposal

This paper discussed about remaining consideration on S&F operation and proposed the following:

Proposal 1: RAN2 to confirm the working assumption of indication for neighbour cells’ operation mode.

Proposal 2: Neighbour cells’ mode transition time information could also be provided to UE

Proposal 3: It can be up to network implementation whether S&F related information for neighbour cells is included in system information

Observation 1: Satellite may need to set same value of S&F mode starting time in t-Service also, since starting S&F mode equals to NTN service stop from legacy UE point of view.

Proposal 4: RAN2 to clarify that S&F capable UE can ignore t-Service, if the value of t-Service is same as the time information for switching to S&F mode

- References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2504673, “Report from Break-out session on NR-NTN and IoT-NTN”