3GPP TSG RAN WG2 Meeting #131            	                          R2-2505297

Bengaluru, India, Aug 25 – 29, 2025

Agenda Item:	8.1.2.2

Source:	Xiaomi, Ericsson

Title:	Text proposal of UE candidate data collection configuration

Document for:	Discussion and Decision

# 1	Introduction

This document provides a text proposal for the following post-meeting email discussion based on the summarized proposals:

- [POST130][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson)

Intended outcome: Discuss how to capture ‘The UE doesn’t need to measure the candidate data collection configuration(s)’ in RAN2 spec, and signalling details (including contents). Provide TP(s) for candidate UE side data collection configuration.

Deadline:  June 20th

The baseline CR of this TP is based on the outcome running CR of [POST130][026][AI PHY] 38.331 CR (Ericssson).

# 2	Text Proposal

FIRST CHANGE

## 5.7	Other

<Text Omitted>

### 5.7.4	UE Assistance Information

<Text Omitted>

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

<Text Omitted>

1>	if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE data collection according to 5.7.4.2:

2>	include dataCollectionPreference in this UEAssistanceInformation message;

2>	if the UE prefers to be configured with radio resources to perform data collection:

3>	set dataCollectionStartStop to start;

3>	for each serving cell configured with candidate UE data collection configuration(s) in dataCollectionCandidateConfigList, and for which the UE has one or more preferred radio resource configuration(s):

4>	include an entry in dataCollectionPreferredConfigurationList and set the content as follows:

5>	set the dataCollectionServCellIndex to the serving cell index of the cell;

5>	include in dataCollectionPreferredConfigurationIdList the value(s) corresponding to preferred configuration(s) from dataCollectionCandidateConfigParameterList;

2>	else (if the UE no longer prefers to be configured with radio resources to perform data collection):

3>	set dataCollectionStartStop to stop;

NEXT CHANGE

## 6.2	RRC messages

<Text Omitted>

### 6.2.2	Message definitions

<Text Omitted>

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

<Text Omitted>

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionStartStop-r19                      ENUMERATED {start, stop}                   OPTIONAL,

dataCollectionPreferredConfigurationList-r19     SEQUENCE (SIZE (1..FFS)) OF DataCollectionPreferredConfiguration-r19         OPTIONAL,

...

}

LoggedDataCollectionAssistance-r19 ::=    SEQUENCE {

lowPowerState-r19                         ENUMERATED {true}                                                   OPTIONAL,

bufferStatus-r19                          ENUMERATED {full, aboveThreshold}                                   OPTIONAL,

...

}

DataCollectionPreferredConfiguration-r19 :: = SEQUENCE {

dataCollectionPreferredConfigurationIdList-r19      SEQUENCE ((SIZE (1..maxPreferredConfig-r19)) OF DataCollectionCandidateConfigId-r19

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

activeDuration Indicates the UE's preferred active duration to resolve the IDC problem. Value in multiples of 1/32 ms (subMilliSeconds) or in ms (milliSecond). For the latter, value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, and so on.

affectedBandwidth Indicates the bandwidth around the center frequency of the carrier frequency range which is affected by the IDC problem. Value mhz5 corresponds to 5 MHz, value mhz10 corresponds to 10 MHz and so on. If candidateBandwidth is not configured, the UE is allowed to report the frequency range for any bandwidth as indicated by affectedBandwidth, within the frequency band limitation as defined in TS 38.101-1 [15], TS 38.101-2 [39], TS 38.101-3 [34] and TS 38.101-5 [75].

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqRangeList Indicates a list of NR carrier frequency ranges that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequency combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC.

affectedCarrierFreqRangeCombList Indicates a list of NR carrier frequency range combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA or NR-DC

bfd-MeasRelaxationState Indicates the relaxation state of BFD measurements. Each bit corresponds to a serving cell of the cell group. A serving cell is mapped to the (servCellIndex+1)-th bit, starting from MSB. A bit that is set to 1 indicates that the UE is performing BFD measurements relaxation on the serving cell mapped on the bit. A bit that is set to 0 indicates that the UE is not performing BFD measurements relaxation on the serving cell mapped on the bit. If a serving cell is not configured to the UE, the corresponding bit is set to 0.

centerFreq Indicates the center frequency of the carrier frequency range which is affected by the IDC problem.

cycleLength Indicates the UE's preferred cycle length to resolve the IDC problem. Value in ms. Value ms2 corresponds to 2 ms, value ms3 corresponds to 3 ms, and so on.

dataCollectionStartStop If it is set to 'start', it indicates the UE's preference to be configured with radio resources for UE data collection. If it is set to 'stop', it indicates the UE's preference not to be configured any longer with radio resources for UE data collection.

dataCollectionPreferredConfigurationList Indicates a list of the UE's preferred radio resource configuration(s) for UE data collection for a cell indicated by dataCollectinServCellIndex.

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

<Text Omitted>

## 6.3	RRC information elements

<Text Omitted>

### 6.3.4	Other information elements

<Text Omitted>

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

<Text Omitted>

DataCollectionPreferenceConfig-r19 :: = SEQUENCE {

dataCollectionCandidateConfigList-r19   SEQUENCE (SIZE (1..FFS)) OF DataCollectionCandidateConfig-r19            -- Need N

}

LoggedDataCollectionAssistanceConfig-r19 ::= SEQUENCE {

loggedDataCollectionBufferThreshold-r19                      ENUMERATED {FFS}                                       OPTIONAL, -- Need R

loggedDataCollectionFullBuffer-r19                           ENUMERATED {true}                                      OPTIONAL, -- Need R

loggedDataCollectionPowerLow-r19                             ENUMERATED {true}                                      OPTIONAL  -- Need R

}

DataCollectionCandidateConfig-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19                  ServCellIndex,

dataCollectionCandidateConfigParameterList-r19   SEQUENCE (SIZE (1..maxCandidateConfig-r19))

OF DataCollectionCandidateConfigParameters-r19

}

DataCollectionCandidateConfigParameters-r19 ::= SEQUENCE {

dataCollectionCandidateConfigId-r19         DataCollectionCandidateConfigId-r19,

CSI-ResourceConfigIdSetA-r19                CSI-ResourceConfigId                                     OPTIONAL, -- Need R

CSI-ResourceConfigIdSetB-r19                CSI-ResourceConfigId                                     OPTIONAL, -- Need R

associatedIdA-r19                           AssociatedId-r19                                         OPTIONAL, -- Need R

associatedIdB-r19                           AssociatedId-r19                                         OPTIONAL  -- Need R

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

aerial-FlightPathAvailabilityConfig Configuration for the UE to indicate the availability of flight path information for Aerial UE operation.

applicabilityReportConfig Configuration for the UE to indicate the applicability of configurations subject to the applicability determination procedure.

bfd-RelaxationReportingConfig Configuration for the UE to report the relaxation state of BFD measurements.

btNameList Configuration for the UE to report measurements from specific Bluetooth beacons. NG-RAN configures the field if includeBT-Meas is configured for one or more measurements.

candidateBandwidth Indicates the bandwidth of the candidate frequency range around the center frequency.

candidateCenterFreq Indicates the center frequency of the candidate frequency range.

candidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues.

candidateServingFreqRangeListNR Indicates the candidate frequency range with the combination of the center frequency and the candidate bandwidth, around which the UE is requested to report IDC issues.

connectedReporting Indicates that the UE can report a preference to remain in RRC_CONNECTED state following a report to leave RRC_CONNECTED state. If absent, the UE cannot report a preference to stay in RRC_CONNECTED state.

dataCollectionPreferenceConfig Configuration for the UE to report its preference to be configured with radio resources for UE data collection.

dataCollectionCandidateConfig Indicates for each serving cells, the list of candidate radio resources configured for UE data collection. The UE is not expected to perform measurements solely based on the configurations provided by this IE.

NEXT CHANGE

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

<Text Omitted>

maxLogCSI-MeasReport-r19                      INTEGER ::= FFS      -- Maximum number of entries for logged measurements for network data collection

maxNrofApplicabilityReports-r19               INTEGER ::= FFS      -- Maximum number of applicability reports

maxNrofAssociatedIDs-r19                      INTEGER ::= FFS      -- Maximum number of associated IDs

maxNrofAssociatedIDs-1-r19                    INTEGER ::= FFS-1    -- Maximum number of associated IDs minus one

maxNrofLoggedMeasurementConfigurations-r19    INTEGER ::= FFS      -- Maximum number of logged measurement configurations

maxNrofLoggedMeasurementConfigurations-1-r19  INTEGER ::= FFS-1    -- Maximum number of logged measurement configurations minus one

maxCandidateConfig-r19                        INTEGER ::= FFS      -- Maximum number of candidate UE data collection configurations maxPreferredConfig-r19                        INTEGER ::= FFS      -- Maximum number of preferred UE data collection configurations

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

NEXT CHANGE