3GPP TSG-RAN WG2 Meeting #131	R2-2505312

Bangalore, India Aug 25th – 29th, 2025

# Agenda item:	8.6.4

# Source:	ETRI

# Title:	Remaining issues on conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#130 meeting, agreements on conditional LTM design have been made as the followings [1]:

Agreements on conditional LTM - (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed. - For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant. - Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. - During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. - In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS. - RAN2 will not do anything for the separate TCI mode optimization.

In this paper, we discuss remaining open issues regarding conditional LTM.

- Discussion

  - CG resource allocation for RACH-less CLTM

Since the UE decides the cell switch timing and target cell based on preconfigured execution conditions in conditional LTM(CLTM), a source cell and candidate cells do not know when the UE will perform a CLTM cell switch or which candidate cell will be selected. This means that all candidate cells must allocate Configured Grant (CG) resources as soon as CLTM preparation is complete to support RACH-less CLTM. Since the UE may choose only one candidate cell, possibly after a long delay or not at all, this leads to significant radio resource waste. From the perspective of candidate cells, many UEs in their neighbouring cells may require RACH-less CLTM, so, more resource waste is expected.

Although the source cell cannot know exactly when the UE will initiate a CLTM cell switch, it can predict when the UE performs CLTM cell switch using factors such as early UL synchronization or measurement reports. These prediction methods do not need to be specified in the standards, allowing flexibility in implementation. However, to reduce radio resource waste, a mechanism for the source cell to share prediction result with candidate cells should be defined in the Rel-19 specifications.

The source cell can notify the tentative target cell of a predicted CLTM cell switch. Upon receiving this notification, the tentative target cell begins allocating CG resources for RACH-less CLTM. This approach significantly reduces resource waste by limiting CG allocation to the tentative target cell and delaying it until a cell switch is probable.

RACH-less CLTM is an effective technique for minimizing handover interruption. However, too much radio resource waste makes it impractical. To make it work, resource waste needs to be kept low. The source cell can assist candidate cells in determining the appropriate timing for CG resource allocation.

Proposal 1: A source cell notifies a candidate cell of a tentative cell switch, and the candidate cell starts allocating CG resources for RACH-less CLTM upon receiving the notification.

  - (MAC-14) Network triggered LTM cell switch to a CLTM candidate cell

Since an open issue MAC-14 was discussed but the cconsensus was not made in the last meeting, MAC Rapporteur suggested not to discuss this issue and consider it based on companies’ contribution in the future meeting.

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM cell switch Command MAC CE is received - For case 1: LTM cell switch target cell is CLTM candidate cell Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM cell switch Command MAC CE is received | Issue Type: Not essential but important How to address it: based on companies’ contribution |

A UE may receive an (Enhanced) LTM CSC MAC CE from the network while it is performing CLTM execution conditions evaluation. In this case, the UE should treat it as a network-triggered LTM cell switch command, not a CLTM cell switch. So, the steps after receiving the (Enhanced) LTM CSC MAC CE should follow the same rules as the LTM cell switch procedure.

If the UE receives the (Enhanced) LTM CSC MAC CE containing a valid TA, the UE performs a RACH-less LTM cell switch procedure. If value “FFF” is in the (Enhanced) LTM CSC MAC CE and the UE has a valid TA acquired by using UE-based TA measurement, the UE can perform a RACH-less LTM cell switch procedure, which matches the Rel-18 LTM procedure. The case where CLTM TAT timer is running is the same as when the UE is configured with UE-based TA measurement. So, if the UE receives the (Enhanced) LTM CSC MAC CE with value “FFF” for the TA value, and a CLTM candidate TAT timer for the target cell indicated by the (Enhanced) LTM CSC MAC CE is still running, then the UE performs a RACH-less LTM cell switch procedure.

Furthermore, if the network sends an (Enhanced) LTM Cell switch Command MAC CE with valid TA for the target cell, the UE should store the TA value for the target cell and the CLTM TAT timer for the target cell should be restarted.

Proposal 2: When the UE receives an (Enhanced) LTM CSC MAC CE with a valid TA requesting a cell switch to a CLTM candidate cell, it stores the TA value for the target cell, starts or restarts the CLTM TAT timer for that cell, and performs a RACH-less LTM switch using the TA from the (Enhanced) LTM CSC MAC CE.

Proposal 3: When the UE receives an (Enhanced) LTM cell switch MAC CE with value “FFF” for the TA value requesting a cell switch to a CLTM candidate cell, the UE performs a RACH-less LTM cell switch with the indicated cell if it has a valid TA (i.e., CLTM TAT timer for the target cell is still running or the UE is configured with UE-based TA measurement). Otherwise, it performs a RACH-based LTM cell switch procedure.

- Conclusions

In this paper, we discuss CLTM related issue. Following are proposals made in this paper:

Proposal 1: A source cell notifies a candidate cell of a tentative cell switch, and the candidate cell starts allocating CG resources for RACH-less CLTM upon receiving the notification.

Proposal 2: When the UE receives an (Enhanced) LTM CSC MAC CE with a valid TA requesting a cell switch to a CLTM candidate cell, it stores the TA value for the target cell, starts or restarts the CLTM TAT timer for that cell, and performs a RACH-less LTM switch using the TA from the (Enhanced) LTM CSC MAC CE.

Proposal 3: When the UE receives an (Enhanced) LTM cell switch MAC CE with value “FFF” for the TA value requesting a cell switch to a CLTM candidate cell, the UE performs a RACH-less LTM cell switch with the indicated cell if it has a valid TA (i.e., CLTM TAT timer for the target cell is still running or the UE is configured with UE-based TA measurement). Otherwise, it performs a RACH-based LTM cell switch procedure.

- References

- Chair notes RAN2#130

- Discussion summary and list of MAC open issues for Mob Ph4

- Running MAC CR for enhanced mobility Ph4