3GPP TSG-RAN WG2 Meeting #131bis	R2-2505329

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.7.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	Rapporteur Inputs

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This document contains the following rapporteur’s input to this meeting:

-	RAN2 agreements;

-	Workplan;

-	A short overview of the status from other RAN groups.

# 2	List of RAN2 Agreements

## 2.1	RAN2#125bis

Regarding the organisation of the work:

-	RAN2 tries to identify impacts on other WGs as soon as possible, e.g. before study phase ends.

Regarding multi-modality (study phase):

-	For the purpose of study, RAN2 assumes that UE and gNB have some kind of multi-modal information;

-	FFS what information is needed/useful, e.g. just multi-modal ID, association between the flow, synchronization requirement etc.

-	RAN2 will study both UL and DL directions based on the assumption of multi-modality association knowledge at RAN/UE;

-	RAN2 will focus on analysing potential usage and benefits (e.g. in terms of capacity and power saving) of multi-modal association knowledge;

-	Areas to study include: synchronization between the flows, FFS impact on QoS insurance and other areas;

-	RAN2 assumes that traffic of different modals having different QoS requirements is mapped to different QoS flows;

-	For different XR traffic flows belonging to the same Multi-modal service and having different QoS requirements, it should be possible to provide differentiated QoS handling over the air. RAN2 should study if that is possible with current mechanism or new ones are needed;

-	Existing QoS flow to DRB mapping framework is used as a baseline, i.e. up to gNB how to map QoS flows to DRBs.

Regarding Scheduling Enhancements (study phase):

-	RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied:

-	Alternative 1: Enhance LCP restrictions/LCH selection;

-	Alternative 2: Enhance LCH prioritization;

-	RAN2 should consider potential impact on traffic from SRBs.

-	RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance - FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

Regarding RLC Enhancements:

-	We focus on RLC AM;

-	RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR;

-	RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets).

## 2.2	RAN2#126

Regarding multi-modality (study phase):

-	Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

Regarding scheduling enhancements (study phase):

-	For LCP enhancements, LCP Prioritisation:

-	Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR;

-	The solution should consider impact on UE complexity (as already indicated in SI objective description);

-	For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

-	Use additional priority configured to LCHs in case of these LCHs with delay-critical data.

-	FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH.

-	For LCP enhancements, LCH Restrictions:

-	We try to avoid RAN1 impacts;

-	RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way;

-	For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial;

-	The solutions should not disallow non-delay critical data from using an UL grant.

-	For LCP enhancements, Granularity:

-	LCP prioritization within a logical channel will not be considered in RAN2 discussions;

-	FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

-	For DSR enhancements:

-	Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG;

-	FFS whether DSR triggering is impacted;

-	FFS whether PDU set importance needs to be included.

Regarding RLC enhancements:

-	For avoiding unnecessary retransmissions:

-	For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

1.	Rx initiated approach

2.	Tx initiated approach

-	RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

-	For Tx initiated approach:

-	The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs;

-	Tx side stops retransmit obsolete SDUs;

-	Rx side updates state variables according to the information from Tx side.

-	For Rx initiated approach:

-	For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter;

-	Tx side just processes the status report as in legacy;

-	FFS how Rx side determines that an SDU should be abandoned.

-	For autonomous retransmissions:

-	To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

-	Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered);

-	Retransmission based on enhanced status report;

-	Retransmission based on enhanced polling;

-	FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism.

-	Impact on capacity should be considered;

-	RAN2 focuses on the enhancements for UL traffic.

In addition, a few LSs were sent to SA2:

-	R2-2405779 Reply LS to SA2 on FS_XRM PH2

-	R2-2405781 Reply LS on Application-Layer FEC Awareness at RAN

-	R2-2405782 LS on multi-modality awareness at RAN

## 2.3	RAN2#127

Regarding an incoming LS from SA2 [R2-2406433], RAN2 agreed:

-	From RAN2 periodicity can already be provided to gNB via TSCAI and/or UAI, which is sufficient for infrequent periodicity changes;

-	RAN2 does not have consensus on whether additional indication for dynamic changes of the periodicity are needed;

-	RAN2 thinks TTNB may be useful for the NW scheduling for DL, provided it is provided in advance and is reliable and accurate at RAN.

Then, RAN2 agreed the following regarding multi-modality (study phase):

-	Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB.

-	FFS whether this can be applied to DL

-	RAN2 considers that based on multi-modal information:

-	The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose.

-	The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

-	For UL, RAN2 does not intend to perform LCP enhancements due to complexity vs gains concerns.

-	For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN.

-	For PDU set discard enhancements:

-	RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

-	RAN2 thinks in case this is feasible, it should be limited to intra-DRB case.

-	For DRX enhancements:

-	Not support multiple active DRX configurations

Regarding scheduling enhancements (also study phase), RAN2 agreed:

-	RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR

-	Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG.

-	For enhanced DSR:

-	There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds

-	FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR

-	FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

-	FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

Regarding RLC enhancements:

-	For Unnecessary retransmissions:

-	Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

-	In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.

-	In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where

-	Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach

-	Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

## 2.4	RAN2#127bis

Agreements for RRM measurement gap skipping:

1.	RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

2.	No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features

3.	RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear.

4.	RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4.

5.	RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution.

Agreements on LCP enhancements:

1.	As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round.

2.	FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation.

3.	Introduce an independent per-LCH remaining time threshold for applying delay-critical priority.

4.	We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

Agreements on DSR enhancements:

1.	We do not change the definition of delay-critical data

2.	For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s)

3.	Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc.

4.	For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0:

•	Buffer size of data volume in each portion

•	Shortest remaining time among PDCP SDUs buffered in each portion.

5.	There is no need to include PSI in the enhanced DSR MAC CE.

6.	A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

7.	FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Agreements on RLC timely retransmissions:

1.	RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

2.	Exclude enhanced status reporting.

3.	Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

Agreements on avoiding unnecessary retransmissions:

1.	RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does.

2.	RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e.

•	TX side stops transmissions of an outdated SDU

•	RX side abandons the SDU based on a local timer

•	Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

•	FFS if some C-PDU handling is needed to avoid C-PDU discard

•	FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

Agreements on XR rate control

1.	FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

2.	RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications:

-	Extend the Bit Rate field

-	Define a new bit rate table to provide sufficient granularity for XR traffic

-	Introduce new values for the bitRateMultiplier

3.	Send LS to SA4 asking about range/granularity which is required

## 2.5	RAN2#128

Agreement for the reply to SA2 on PDU set information: RAN2 confirms that it can be useful for gNB to have PDU Set Information marking without PDU Set QoS parameters.

Agreements on RRM measurement gaps impacts:

-	No MG-specific enhancements is needed on DSR operation.

-	RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped

-	No MG-specific enhancements is needed for DRX operation.

Agreements on LCP prioritization:

-	As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round.

-	As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

Agreements on DSR enhancements:

-	Let the network configure the triggering and reporting thresholds without constraints.

-	RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR

-	The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

-	The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

Agreements on AL-FEC (related to LS from SA2):

-	There is no consensus in RAN2 that AL-FEC ratio information is useful for the gNB for both RLC AM and RLC UM.

-	RAN2 understanding is that in case this information would be provided to the gNB, it is up to gNB how/whether to consider it, i.e. no impact on RAN2 specifications

Agreements on unnecessary RLC retransmissions:

-	There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer.

-	Unless critical issue is identified, no Tx to Rx indication will be introduced.

-	Special handling to avoid PDCP control PDU discard is not needed.

-	A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.

-	The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Agreements on timely RLC retransmissions:

-	Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

Agreements on XR rate control:

-	RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level.

-	gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

-	Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

-	RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP.

-	RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE

-	FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

## 2.6	RAN2#129

Agreements for RRM measurement gap skipping:

-	From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion.

-	“Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC.RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

-	RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1.

Agreements on LCP enhancements:

-	Only one additional priority is configured to an LCH for LCP enhancement.

-	We keep an existing agreement (remaining time threshold is configured per LCH)

-	There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority.

-	Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact

-	FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

-	No additional PBR is needed for priority adjusted data

-	FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Agreements on DSR enhancements:

-	One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE.

-	FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds.

-	We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size.

-	Different LCGs may be configured with different number of reporting thresholds.

-	If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

-	Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold).

-	Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

Agreements on RLC timely retransmissions:

-	Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

-	Only a single autonomous retransmission will be triggered per RLC SDU.

-	There is no dynamic activation/deactivation of the autonomous retransmission mechanism.

-	We have separate thresholds for autonomous reTx and for polling

Agreements on avoiding unnecessary retransmissions:

-	When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment.

-	A new RLC timer at the TX is not introduced to determine outdated RLC SDUs.

-	The new RLC timer at the RX is per RLC entity

-	The duration of the new RLC timer is not lower than that of t-reassembly

-	Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

Agreements on XR rate control

-	RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

NOTE:	We may revisit UAI option based on SA2/RAN3 reply

-	Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc.

-	We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed

-	Working assumption:

-	Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE.

-	The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy).

NOTE: Companies to check with their SA4 colleagues whether there are any issues with this.

## 2.7	RAN2#129bis

Agreements related to PDCP CR:

1.	In PDCP specifications, use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set.

2.	Keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the RRC specification.

3.	Specify delay-reporting indication in the PDCP specification.

4.	Do not specify explicitly an update of delay-reporting indication in the PDCP. FFS (when discussing the CR) whether some explanation/note can be added to clarify that the UE should ensure the indicated data volume is up to date.

Agreements related to RRC CR:

1.	Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

2.	The maximum number of entries in the dsr-ReportingThresList is 4.

3.	For the available data rate query MAC CE, confirm on the following:

•	Introduce a prohibit timer for the UL transmission of the data rate query MAC CE

•	Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration

•	FFS the details of prohibit timer, e.g. granularity.

Agreements on UE Capabilities:

1.	RAN2 assumes that we define separate UE capabilities for different XR features (as in Rel-18 XR).

2.	RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19.

3.	An optional UE capability with signalling (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. No dependency on support of delayStatusReport-r18.

4.	An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.

5.	An optional UE capability with signalling (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites.

6.	An optional UE capability with signalling (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites.

7.	An optional UE capability with signalling (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites. FFS whether there is a separate UE capability for UL rate query.

8.	For the above Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

Agreements on LCP Enhancements:

1.	Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

2.	It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

3.	We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

4.	It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP.

5.	For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

6.	Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

7.	All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

Agreements on DSR Enhancements:

1.	To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

2.	During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

3.	Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review)

4.	We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

Agreements on RLC Enhancements:

1.	When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

2.	For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

3.	Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

4.	Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

Agreements on XR Rate Control:

1.	Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

2.	We specify a single table.

3.	As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered.

4.	The working assumption is changed to agreements on rate query MAC CE.

5.	From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this.

6.	FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

## 2.8	RAN2#130

Agreements related running CRs:

MAC

1.	We go with option 1, i.e. Single and multiple entry DSR MAC CE

2.	In MAC CR, FFS whether some changes are needed due to using single/multiple entry DSR MAC CE terminology

UE capabilities

3.	An optional UE capability with signalling (e.g. delayStatusReportNonDelayReportingData-r19) is introduced to indicate the support of including non-delay-reporting data ahead of delay-reporting data in the buffer size calculation for enhanced delay status report. A UE supporting this feature shall also indicate support of enhanced delay status report (enhancedDelayStatusReport-r19). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

4.	An optional UE capability with signalling (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message (in UL Rate Control MAC CE) from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE (ul-RateControl-r19). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

RLC

5.	Use the term of “discard” for avoiding unnecessary retransmission, e.g. “stopReTxDiscardedSDU”, “t-RxDiscard”.

6.	Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

7.	The term “remaining time based retransmission” is used for autonomous retransmission in RLC.

8.	Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

Stage-2 (on PDU set handling clarification)

9.	We will update 38.300 in line with SA2 updated specs

Agreements on measurement gap skipping

1.	(RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received (and the UE has preference) or when the assistance information changes since the last report.

2.	(RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes.

3.	(RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio. FFS the granularity of prohibit timer

4.	Consider RAN4 progress on the granularity for the recommended gap cancellation ratio when drafting the RRC CR

5.	(UE capability-03) RAN2 defines a per UE capability for reporting the ratio of gap occasions. And its prerequisite condition is that it supports enabling TX/RX during measurement gap scheduling restrictions by DCI.

Agreements for scheduling enhancements

1.	[MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

2.	(MAC-02) When PSI-based discarding is activated for a DRB, priority adjustment is applied only if there is at least one high importance packet in the buffer has a remaining time smaller than the threshold. (MAC spec is already aligned with this agreement).

3.	(MAC-13) When pdu-SetDiscard is configured, PDU sets should be treated as a whole in the LCP procedure with adjusted priority.

4.	Confirm the working assumption “No Bj enhancement is introduced” as an agreement.

Agreements for DSR enhancements

1.	(PDCP-1, RLC-5) Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold.

2.	(PDCP-1, RLC-5) The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG.

3.	(PDCP-1, RLC-5) An understanding is that there will be no DSR with no data indication (i.e. indicating only volume of C-PDU and/or retransmissions for any LCG)

4.	(RRC-7) If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold.

5.	New LCID is used for R19 DSR

6.	(MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR.

7.	(MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU.

8.	(MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

Agreements on RLC timely retransmissions

1.	(RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this

2.	(RLC-9) No additional conditions are needed for the polling enhancement.

3.	(RLC-7) RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

4.	(PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

5.	(RRC-3, RRC-4) Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance

6.	(RRC-8) The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Agreements for unnecessary retransmission avoidance in RLC

1.	(RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission.

2.	Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

a.	Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

3.	(UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

4.	(UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

Agreements for XR rate control

1.	(MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify.

2.	(MAC-04) We rule out explicit signalling of PDU session ID + QFI

3.	(MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

a.	Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

b.	Implicit, e.g. index or mapping

4.	(RRC-1) The granularity of bit rate query prohibit timer is QoS flow.

a.	FFS The value of the prohibit timer is the same for all flows

5.	(MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

6.	(MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit.

7.	(MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate.

8.	(MAC-09) No optimization is needed to address DC case for XR rate control MAC CE

9.	(MAC-09) We capture in stage-2 that XR rate indication is for application and not for MAC entity.

10.	(MAC-11) RAN2 will not specify DL rate control and DL rate control query

# 3	Workplan

The Rel-19 XR Phase 3 objectives are as follows [RP-250107]:

-	Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

-	Specify the following user plane enhancements [RAN2]:

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

-	Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]:

-	PDU set based handling;

-	ECN marking;

-	Burst Arrival Time reporting, if needed;

-	PSI Discard coordination, if needed;

NOTE:	No RAN2 impact from above items.

-	Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-	Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-	QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-	Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

The corresponding update to the workplan is given below, now also updating the completion rates to factor the additional RAN3 TUs in.

Table 3-1: Workplan for the XR WI

| Date | RAN1 | RAN2 | RAN3 | RAN4 | RAN |
| --- | --- | --- | --- | --- | --- |
| 2024-02 | RAN1#116 Start discussion on a design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. |  |  |  |  |
| 2024-03 |  |  |  |  | RAN#103 Status Report (target of 2% corresponding to 0.5 TU spent over an initial allocation of 24.8) |
| 2024-04 | RAN1#116bis Continue the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Start agreeing on solution(s). | RAN2#125bis Endorse workplan Start the work on all 4 objectives except measurement gaps, for which RAN1 conclusions first need to be reached. For multi-modality and scheduling enhancements, start the study phase. | RAN3#123bis Start discussion on RAN3 related objectives on NR-DC, e.g. PDU set based handling, ECN marking, etc. Agree the DC bearer types to be supported. Designate responsible companies for BL Stage-2 & 3 CRs. |  |  |
| 2024-05 | RAN1#117 Continue developing the solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#126 Continue the work on all 3 objectives. | RAN3#124 Finalize the solution on required enhancements. Finalize Stage-2 CR(s) Agree the skeleton of Stage-3 CR(s). |  |  |
| 2024-06 |  |  |  |  | RAN#104 Status Report (target of 18% corresponding to 4.5 TUs spent over an initial allocation of 24.8) Take into account outcome of SA2 study if available. |
| 2024-08 | RAN1#118 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#127 Continue the work on all 3 objectives. For multi-modality and scheduling enhancements, conclude the study phase and convey the outcome to RAN#105. | RAN3#125 Agree Stage-2 CR(s) and Stage-3 CR(s) | RAN4#112 Approval of workplan Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |  |
| 2024-09 |  |  |  |  | RAN#105 Status Report (target of 30% corresponding to 7.5 TUs spent over an initial allocation of 24.8) Checkpoints: 1. Multi-modality (RAN2) 2. Scheduling enhancements (RAN2) |
| 2024-10 | RAN1#118bis Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Check if RAN1 involvement related to objective on multi-modality is needed. | RAN2#127bis Check with RAN1 the progress on measurement gaps and start discussing in RAN2. Continue the work on UL scheduling enhancements according to the refined objective. Start the work on uplink congestion signalling. | RAN3#125bis Discuss multi-modality when handling incoming LS (no additional TU allocated to the new objective). | RAN4#112bis Conclusions on scenarios for measurement skipping based on RAN1 conclusions. Discussion on the need for UE assistance information regarding measurements occasions needed. |  |
| 2024-11 | RAN1#119 Continue the work on the details of the agreed solution(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. | RAN2#128 Narrow down possible solutions. First version of Stage 2 CR | RAN3#126 Discuss LS on PDU Set Information Marking Support without QoS parameters, and usage of multi-modality information (no additional TU allocated to the new objective). | RAN4#113 Discussion on RRM requirements with measurement skipping. Discussion on the necessary UE assistance information. |  |
| 2024-12 |  |  |  |  | RAN#106 Status Report (target of 55% corresponding to 13.5 TUs spent over an initial allocation of 24.8) Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) |
| 2025-02 | RAN1#120 Finalize, if not already, the discussion on the design of the enhancement feature(s) to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. Provide RAN1 input to RAN2 on RRC configuration parameters, UE capabilities, and Stage 2 TPs. | RAN2#129 Continue the work on the details of the agreed solutions. First versions of Stage 2 & 3 CR. | RAN3#127 Discuss LS on UL bit rate control, available data rate reporting and multi-modality awareness (no additional TU allocated to the new objective). | RAN4#114 Conclusion on RRM requirements with measurement skipping. Discussion on the need for UE assistance information regarding measurement skipping. Agreement of CR work split. |  |
| 2025-03 |  |  |  |  | RAN#107 Checkpoint on multi-modality (including possible allocation of additional TUs for RAN3) Status Report (target of 67% corresponding to 16.5 TUs spent over an initial allocation of 24.8) |
| 2025-04 | RAN1#120bis XR will not be discussed. | RAN2#129bis Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. | RAN3#127bis Start discussion on RAN3 related objectives on uplink rate control, PDU set based QoS handling enhancement, TTNB and burst size, exposure of available data rate etc. Designate responsible companies for Stage-2 & 3 BL CRs. | RAN4#114bis Conclusion on the necessary UE assistance information. First Draft CRs for RRM core requirements (draft CRs for 38.133 expected) First big Draft CR. |  |
| 2025-05 | RAN1#121 Continue the work on remaining details such as interaction with measurement gap skipping with valid DL slot for CSI reporting and definition of indication consistency for Option 2. Reflect new agreements in stage 3 CRs. | RAN2#130 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in running Stage 2 & 3 CRs. | RAN3#128 Continue the work on the details of the agreed solutions for all objectives. Reflect new agreements in Stage 2 & 3 BL CRs. | RAN4#115 Discussion on remaining open issues for RRM core. |  |
| 2025-06 |  |  |  |  | RAN#108 Status Report (target of 90% corresponding to 25 TUs spent over an initial allocation of 24.8) |
| 2025-08 |  | RAN2#131 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs | RAN3#129 Finalise Stage 2 & 3 CRs Agree Stage 2 & 3 CRs | RAN4#116 Discussion on remaining open issues for RRM core. Revision of CRs for RRM core requirements. Planning of RRM performance. Initial discussion on RRM test cases. |  |
| 2025-09 |  |  |  |  | RAN#109 Status Report (target of 100% corresponding to 27.75 TUs spent over an initial allocation of 27.75.8) CRs received for approval. |

# 4	Status from other groups

## 4.1	RAN1

During RAN1#121 following was agreements were made:

Valid downlink slot for CSI reference resource definition is only determined based the configured measurement gap pattern and not affected by dynamic measurement gap occasion cancellation.

Adopt following TP to draft CR of TS38.213 in Section 10.6:

## 10.6 Indication for cancelation of RRM measurement gaps An XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133] as described below. A UE can be indicated by a value ‘1’ of an XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception to cancel an earliest associated RRM measurement gap occasion where the UE would perform RRM measurements on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. The earliest RRM measurement gap occasion is at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. If a UE indicates “ValueOption 1” for FG 64-1-ABC, the UE ignores a value ‘0’ of the XYZ measurement gap cancellation field in a DCI format. If a UE indicates “ValueOption 2” for FG 64-1-ABC, a value ‘0’ of the XYZ measurement gap cancellation field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest associated RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the XYZmeasurement gap cancellation field and are associated with a same earliest associated RRM measurement gap occasion where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the more than one DCI formats, the UE applies the indication provided by the XYZmeasurement gap cancellation field of a last DCI format, from the more than one DCI formats, that is in a PDCCH reception that ends at least 3 msec or 5 msec, as determined by a reported UE capability, before the start of that earliest RRM measurement gap occasion.

Note: Above TP is not intended to impact future discussions related to UE features for XR.

Also following conclusions were made in RAN1#121:

-	RAN1 does not discuss the measurement gap occasion cancellation for concurrent measurement gaps further unless triggered by RAN4.

-	UE does not expect different values in ‘Measurement gap cancellation’ fields in DCI formats that are associated with the same measurement gap occasion and are received in same PDCCH monitoring occasion.

## 4.2	RAN3

For uplink congestion signalling:

-	RAN3 agreed to introduce a per QoS flow indication in F1AP, similar to NGAP Indication of Bitrate Adaptation IE, to inform the gNB-DU which QoS flow(s) are subject to uplink rate control, and the same indication is transferred to target gNB during Xn handover preparation procedure.  TS 38.300 TP agreed in R3-253925, TS 38.473 TP agreed in R3-253833.

-	RAN3 agreed LS to RAN2 R3-253927 regarding RAN3 decision to support the per QoS flow indication in F1AP.

For Support of exposure of available bitrate:

-	RAN3 agreed LS to SA2 R3-253929, asking whether the gNB need to report the status of Available Bitrate monitoring.

-	RAN3 agreed Stage-3 TPs to fix the Editor’s Note, e.g. define available bitrate and threshold as a 32-bit integer with unit kbps, use value “8” for maxnoofThresholds.  TS 38.413 TP agreed in R3-253928, TS 38.423 TP agreed in R3-253839, TS38.473 TP agreed in R3-253835, TS 37.483 TP agreed in R3-253840.

-	The TS 38.413 TP R3-253928 also includes the update of the semantics text for support of PDU Set AQP, i.e. for the PDU Set Delay Budget Downlink, add such clarification in semantics : This IE is included if the PDU Set Packet Delay Budget IE in the DL PDU Set QoS Information IE is present and is ignored otherwise.

For Burst Size and TTNB:

-	RAN3 agreed to fix the Editor’s Notes in TS 38.415 BL CR and TS 38.425 BL CR. TS 38.415 TP agreed in R3-253837, TS 38.425 TP agreed in R3-253930.

-	RAN3 agreed the forwarding of BSSize and TTNB in DC case. TS 37.340 TP agreed in R3-253836.

RAN3 agreed the MMSID is defined as OCTET STRING (SIZE (1)).

For RLC retransmission enhancements, RAN3 agreed to reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS how to capture it in spec.

## 4.3	RAN4

Way forward on RRM requirements for XR_Ph3 agreed in (R4-2508285):

-	Types of measurement gaps:

-	Type 2 gaps:

-	MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

-	The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collisionhandling rules.

-	DCI skipping cannot be applied if any gap is configured with gapAssociationPRS.

-	NCSG: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17.

-	Preconfigured measurement gaps: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17.

-	Rel-18 concurrent gaps: MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps.

-	Regarding UE assistance information, the following is agreed:

-	The UE may provide the UAI related information, for the UAI in the agreement captured in RAN4 #114bis, per MG configuration:

-	per-FR gap if gap is configured with gapConfig with per-FR gaps;

-	per gap configuration if gap is configured with gapConfig-r17.

-	Regarding semi-static signalling schemes, the following is agreed:

-	From RAN4 perspective, no consensus to support semi-static solutions in Rel-19.

-	Regarding skipping indication on frequency domain, the following is agreed:

-	It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs.

-	Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

-	Regarding CR drafting and naming convention, the following is agreed

-	Use “Measurement gap occasion cancelation” to refer to measurement gaps that were skipped or cancelled by DCI;

-	Use Lcancel for number of skipping occasions;

-	Indication of which formula/table to use: For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met (new);

-	Use RAN2 pseudo code approach for new clause 9.1.y;

-	Regarding UE capabilities, the following is agreed: define optional UE feature for support of UAI with per UE capability, [and no FR1/FR2 differentiation].

# 5	Conclusion

This contribution has provided:

-	RAN2 agreements;

-	Workplan;

-	A short overview of the status from other RAN groups.