3GPP TSG-RAN WG2 Meeting #131	R2-2505348

Bengaluru, India Aug. 25th – 29th, 2025

Agenda Item:	8.6.3

Source: 	Fujitsu

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion and Decision

- Introduction

At previous RAN2 meetings, some agreements on L1 event triggered measurement reporting are reached. However, there are still some essential or important remaining open issues [1].

This contribution will discuss the following issues:

- MAC-25: Handling of the activated SP CSI-RS for target cell after LTM cell switch

- MAC-8: FFS the coexistence between event triggered report and mTRP ([POST130][117][MOB])

- MAC-24: working assumption on the details for truncated L1 MR MAC CE

- Discussions

2.1 (MAC-25) SP CSI-RS activation/deactivation for LTM candidate

After the cell switch, the UE needs to measure SP CSI-RS resources of the serving cell. To implement it, there are two options:

- Option 1: the UE can continue to measure the activated resources for the target cell according to the received SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE before the LTM cell switch

- Option 2: the UE can measure the resources when the configured resources included in the LTM candidate configuration are activated by the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE after the LTM cell switch (legacy mechanism)

If option 1 applies, one SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE to activate the same resources can be saved if the resources activated by the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE are used after the LTM cell switch. If the resources activated by the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE will not be used after the LTM cell switch, the network can send the SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE or reconfigure the resources. With this, option 1 can be implemented at no additional cost.

Proposal 1 (MAC-25): The UE continues to measure SP CSI-RS resources of target cell after the cell switch.

2.2 (MAC-8) Coexistence between event triggered MR and mTRP

In post email discussion [2], issues of coexistence between L1 event triggered MR and mTRP (source cell) are discussed. The first issue is how to define the “current beam in MR MAC CE”. Companies are asked to select from the options listed below:

- UE should use the best beam of the two “current beams” for event evaluation

- It is up to the UE which of the two beams it uses for L1 event evaluation

- Network indicates which beam the UE shall use

- Specify to use one of the two TCI states (e.g., just use the first indicated TCI state)

- Other

If option a), i.e. a better beam is applied, event e.g. LTM2/3/5 may not be satisfied since the quality of beam of serving cell used for event evaluation is good. Because event-triggered L1 measurement report is not triggered, the LTM cell switch based on the event-triggered L1 measurement report cannot be initiated by the network even if the network configures the event LTM3. The issue can be avoided with option c), i.e. the network can indicate a suitable beam.

Proposal 2 (MAC-8): Network indicates which beam the UE shall include in the MR MAC CE at least for inter-cell mTRP operation.

Alternatively, both current beams can be appliable beam for L1 measurement.

However, option a) is proposed in the email discussion. In case of mTRP operation and if above option a) is adopted, the network does not know what the reported RSRP value corresponds to. Same as rapporteur’s view, such indication could be included in MR MAC CE. Such information can be used by the network for cell switch decisions. For example, the network may determine whether to initiate cell switch or beam switching based on the current serving beam.

Proposal 3 (MAC-8): The UE indicates which TCI state is associated with the reported RSRP value for the serving/current beam in the MR MAC CE if the best beam is used for L1 event evaluation (i.e. option a is adopted).

2.3 (MAC-24) Details for truncated L1 MR MAC CE

Regarding the truncated MR MAC CE, it is up to network to configure whether the current serving beam is included in the MR. It means that the network can indicate that no current serving beam to be included in the MR if it has concerns on UL grant size. So, we suggest RAN2 to confirm the working assumption.

Proposal 4 (MAC-24): RAN2 to confirm the WA as an agreement:

If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

- Conclusion

This document discussed the remaining open issues for the L1 event triggered measurement report and made the following proposals:

Proposal 1 (MAC-25): The UE continues to measure SP CSI-RS resources of target cell after the cell switch.

Proposal 2 (MAC-8): Network indicates which beam the UE shall include in the MR MAC CE at least for inter-cell mTRP operation.

Proposal 3 (MAC-8): The UE indicates which TCI state is associated with the reported RSRP value for the serving/current beam in the MR MAC CE if the best beam is used for L1 event evaluation (i.e. option a is adopted).

Proposal 4 (MAC-24): RAN2 to confirm the WA as an agreement:

If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

# Reference

- R2-2505398, “Discussion summary and list of MAC open issue for Mob Ph4”, vivo, RAN2#131

- R2-2505719, “Report from [POST130][117][MOB] (Nokia)”, Nokia, RAN2#131