3GPP TSG-RAN WG2 Meeting #131	R2-2505350

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.7.6

Source: 	Fujitsu

Title: 	Discussions on XR rate control

Document for:	Discussion and decision

Introduction

On XR rate control, there are the following agreements in the last meeting [1]:

Agreements for XR rate control - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - (MAC-04) We rule out explicit signalling of PDU session ID + QFI - (MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. - (MAC-09) No optimization is needed to address DC case for XR rate control MAC CE - (MAC-09) We capture in stage-2 that XR rate indication is for application and not for MAC entity. - (MAC-11) RAN2 will not specify DL rate control and DL rate control query

In the post-meeting email discussions on the MAC running CR [2], there are following open issues on UL rate control:

[MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE. [MAC-4] Format of the UL Rate Control MAC CE. [MAC-5] Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

In this contribution, we would like to discuss the above open issues on XR rate control.

Discussion

(MAC-3) ID for QoS flows in the UL Rate Control MAC CE

In RAN2#130 meeting, there are following agreements on ID for QoS flows:

We rule out explicit signalling of PDU session ID + QFI

We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

    - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

    - Implicit, e.g. index or mapping

From our perspective, the maximum number of flows we want to be able to indicate can be 8. In order to save the signalling overhead, DRB ID + QFI for each indicated QoS flow do not need to be indicated explicitly. Instead, the index (indices) of the QoS flows can be used in the UL Rate Control MAC CE to identify the QoS flows. The index (indices) of the QoS flows can be determined by one of the following options:

- Option 1: the index of the QoS flow is configured by the network based on the QFI and the PDU session ID

- Option 2: the index of the QoS flow is determined by the mapping between the index and the QFI and the PDU Session ID. The mapping can be predefined by some rules. For example, the indices of QoS flows can be ordered based on the ascending order of PDU session ID and QFI.

Proposal 1: (MAC-3) ID for QoS flows in UL Rate Control MAC CE is the index of the QoS flow.

In our understanding, either option 1 or option 2 can work. Option 1 introduces some signalling overhead in the RRC message for configuring the index for each QoS flow. On the other hand, the mapping rule needs be specified in option 2, and the UE and the gNB should have the same understanding on the mapping rule. We slightly prefer option 2 for its less signalling overhead.

Proposal 2: (MAC-3) The mapping between the index in the rate control MAC CE and the QFI and the PDU session ID can be pre-defined, e.g., based on the ascending order of PDU session ID and QFI.

(MAC-4) Format of the UL Rate Control MAC CE

In RAN2#130 meeting, there are following agreements on the format of UL Rate Control MAC CE:

(MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify.

For the format of UL Rate Control MAC CE, to further save the signalling overhead and to be simple, a bitmap can be used at the beginning of the MAC CE, with each bit in the bitmap indicating the index of the QoS flow for which the rate control information is indicated. In the running RRC CR, the maximum number of QoS flows supporting bit rate control and/or bit rate query is 8, so the length of the bitmap can be 8 bits. In the example in Figure 1, Qi field can indicate whether the bit rate field for QoS flow index i is present in the MAC CE. If Qi is set to 1, the bit rate field for the QoS flow with index i is present in the MAC CE. If Qi is set to 0, the bit rate field for the QoS flow with index i is absent in the MAC CE. The bit rate field indicates the preferred bit rate for the indicated QoS flow and the length of this field is 8 bits. The need for other fields in the MAC CE can be determined based on further RAN2 discussions.

Figure 1 Potential format of rate control MAC CE

Proposal 3: (MAC-4) In the rate control MAC CE, a bitmap indicates the index (indices) of the QoS flows for rate control/query.

Proposal 3a: (MAC-4) The length of the bitmap is 8 bits.

Proposal 3b: (MAC-4) Each bit in the bitmap indicates whether the bit rate field for the corresponding QoS flow is present or not.

(MAC-5) UE behaviour if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE

For the generation of UL Rate Control MAC CE, it is FFS whether a UL Rate Control MAC CE is transmitted only if the available UL-SCH resources can accommodate all the entries in the MAC entity’s list of rate queries. There could be two alternatives if the available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE.

- Alt 1: UL Rate Control MAC CE is generated including part of the pending bit rate queries. In this alternative, the remaining pending bit rate queries (e.g., entries in the MAC entity’s list of rate queries) can be transmitted in the following UL-SCH resources.

- Alt 2: UL Rate Control MAC CE is not generated.

In the above two alternatives, Alt 2 is simpler, and it has less specification impact. On the other hand, since the pending UL rate query does not trigger an SR, the network would not know when/whether the UE has an UL rate query to be transmitted. As the UL Rate Control MAC CE has lower LCP priority than UL data and MAC CEs other than padding BSR/SL-BSR, it may happen frequently that the UL-SCH resources cannot accommodate all the pending rate queries. From this perspective, Alt 1 is better to transmit the UL rate queries for some QoS flows earlier to the network. Therefore, we slightly prefer Alt 1. The selection of the pending bit rate queries to be included in the UL Rate Control MAC CE can be up to the UE implementation. The pending rate queries not included in the UL Rate Control MAC CE are still pending based on the current MAC running CR, i.e., there is no specification impact on current MAC running CR.

Proposal 4: (MAC-5) If the available UL-SCH resources cannot accommodate all the pending rate queries in a single UL Rate Control, the UL Rate Control MAC CE is generated including part of the pending bit rate queries.

Conclusion

In this contribution, we have discussed the open issues for UL rate control. We have the following proposals:

Proposal 1: (MAC-3 ID for QoS flows in UL Rate Control MAC CE is the index of the QoS flow.

Proposal 2: (MAC-3) The mapping between the index in the rate control MAC CE and the QFI and the PDU session ID can be pre-defined, e.g., based on the ascending order of PDU session ID and QFI.

Proposal 3: (MAC-4) In the rate control MAC CE, a bitmap indicates the index (indices) of the QoS flows for rate control/query.

Proposal 3a: (MAC-4) The length of the bitmap is 8 bits.

Proposal 3b: (MAC-4) Each bit in the bitmap indicates whether the bit rate field for the corresponding QoS flow is present or not.

Proposal 4: (MAC-5) If the available UL-SCH resources cannot accommodate all the pending rate queries in a single UL Rate Control, the UL Rate Control MAC CE is generated including part of the pending bit rate queries.

References

- Report from session R19 XR,	Session chair (Huawei), RAN2#130 meeting

- R2-250xxxx List of open issues in MAC_summary_r1, Qualcomm