3GPP TSG-RAN WG2 Meeting #131                                   R2-2505359

Bangalore, India,  Aug 25th -29th, 2025

Agenda item:		8.3.3

Source:				Samsung

Title:					Discussion on Network-sided model

Document for:		Discussion & Decision

1	Introduction

In this paper, we would like to discuss the configuration and report of inference input for NW-side model.

2	Discussion

For NW-side model inference, UE can provide the input data for inference at NW-side. Then, the NW can perform the inference with the input data and use the outcome of the inference for HO. How to use the outcome for HO is totally up to NW implementation. Therefore, for the spec. impact, we can focus on the configuration and report of inference input data between UE and NW.

Also, at the last RAN2#129bis, RAN2 agreed the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. In our view, the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be easily extended/enhanced to support the use cases of AI for mobility. Therefore, in this part, we propose some potential enhancement based on the existing RRM measurement configuration and report.

Common part:

For the common part in the use cases (i.e., temporal/frequency/spatial domain RRM measurement prediction), the UE should provide the input data for NW every time NW performs the inference. Even though it is unclear how often NW will perform the inference, it is evident that there should be the concern on the signalling overhead from the input data report. For example, for temporal domain Case A, the UE should report the input data (i.e., the measurement results for the multiple time instances within OW) which is quite a lot compared to the legacy MR including only one latest measurement result. Moreover, when we consider the L3 beam level prediction, the signaling overhead will be further increased in proportion to the number of beams. However, considering that the motivation is to trigger the HO in proactive manner, NW does not need to perform the inference continuously. Instead, NW can perform the inference only when HO is likely to happen (e.g., when UE moves fast or moves to cell edge area), which can allow UE not to report the input data continuously/periodically. Thus, we need to study ways to reduce the signalling overhead (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Proposal. 1: For NW-side model inference, RAN2 should consider the ways to reduce the signalling overhead for the report of inference input (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Also, at the last RAN2#130, the following agreement was made regarding NW-side RRM measurement prediction.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

For the FFS part on whether there is any potential spec. impact to subcase 1 and 3 for cell level prediction, we think this issue can be discussed per use case as part of the following discussion.

Temporal domain Case A:

For temporal domain Case A, the input data would be the historical L3 Cell/beam-level measurement results of multiple time instances within OW (Observation Window). However, with the legacy measurement report, the UE can report only one latest measurement result for each cell. Therefore, there should be some enhancement on measurement configuration and report to allow UE to log the historical measurement results within OW and report them to network. Regarding this, at the last RAN2#130, the following agreement was made.

To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

In our view, the number of results for multiple time instances above should be equal to the OW of NW-sided model to minimize the signalling overhead. For that, NW should be able to configure the number of time instances for which UE reports the results (i.e., OW length).

Proposal. 2: For temporal domain Case A, NW can configure the number of time instances for which UE reports the results (i.e., OW length).

Regarding the spec. impact to support subcase 1 and 3, if we want to support them for temporal domain Case A, expected enhancement will be reporting beam-level RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model. The beam-level reporting will increase the signalling overhead a lot.

Observation. 1: To support subcase 1 and 3 for cell-level temporal domain Case A, the expected spec. impact is to report “beam-level” RRM measurement results per cell at multiple time instances in one measurement report.

Observation. 2: To support subcase 1 and 3 for cell-level temporal domain Case A, the signalling overhead will increase quite a lot (in proportion to # of beams) for UE to report beam-level measurement results at multiple time instances.

RAN2 already observe that subcase 2 using cell-level input data shows better accuracy than the subcases 1 and 3 using beam-level input. Also, beam-level input data will increase the signalling overhead for inference input report for NW-side model. Thus, in our view, the beam-level inference input report is not necessary.

Proposal. 3: For cell-level temporal domain Case A, RAN2 support only subcase 2 for NW-sided model. I.e., UE can report only “cell-level” RRM measurement results at multiple time instances in one measurement report.

For beam-level temporal domain Case A, we understand that the motivation of beam-level temporal domain Case A prediction is mainly to optimize the dedicated RACH resources allocation, rather than mobility performance. More specifically, it is to enable efficient dedicated RACH resource allocation during handovers by allocating the resources only for the promising beams based on the beam-level prediction results, instead of allocating dedicated RACH resources for more beams based on the current measurement results.

Observation. 3: For beam-level temporal domain Case A, the motivation is to enable efficient dedicated RACH resource allocation during HO.

However, to support NW-sided model for beam-level temporal domain Case A, the signalling overhead will increase quite a lot because UE should report beam-level measurement results at multiple time instances as input data of NW-sided model.

Observation. 4: For beam-level temporal domain Case A, NW-sided model would lead to additional wastage of resources, which contradicts the original motivation of the beam-level prediction.

Proposal. 4: For beam-level temporal domain Case A, for NW-sided model, RAN2 do not introduce any enhancement that may introduce additional signalling overhead (e.g., beam-level measurement results at multiple time instances in one measurement report).

Temporal domain Case B:

For temporal domain Case B, the motivation is to reduce UE’s measurement effort (for saving energy). Specifically, with 50% reduction rate, UE can skip measurement once out of every two instances and predict the measurement values at the skipped time instance. In our evaluation in FR1, we assume that UE measures SSB every 40 msec as baseline and thus for Case B with MRRT 50%, UE performs the measurement every 80 msec. Then, we evaluate HO performance with the UE-side model case where the UE initiates the event-triggered MR based on the actual and predicted measurement results. The evaluation result shows that the performance degradation is marginal even with the measurement reduction at UE-side. As we can see from the evaluation scenario, for UE-side model case, the event-triggered measurement reporting can be the baseline, which means the UE can report the measurement report only when it detects some event.

Observation. 5: For temporal domain Case B, the motivation is to reduce UE’s measurement effort for energy saving.

Observation. 6: For temporal domain Case B, for UE-sided model, the event-triggered measurement reporting can be used. I.e., UE can report the measurement report only when it detects some event.

On the other hands, if we consider the case with NW-side model, the UE should report the measurement results every 80 msec continuously to provide the inference input data for NW. Thus, for NW-side model case, the event-triggered reporting can not be used and the periodical measurement reporting is essential. Considering that Case B is for goal 1(i.e., UE’s measurement effort reduction for energy saving) the frequent measurement reporting from UE with very small interval does not make sense. Therefore, for the Case B prediction, the use of NW-side model can be excluded.

Observation. 7: For temporal domain Case B, for NW-sided model, only the periodical measurement reporting can be used. I.e., UE should periodically report the measurement report with very short interval (e.g., 80 ms), which can increase the energy consumption of UE.

In RRC, the minimum value of periodic reporting interval is 120 ms now. Therefore, to support NW-side temporal domain Case B prediction, we may need to have the enhancement to support shorter interval. However, this will increase the energy consumption of UE, which contradict the original motivation of having Case B prediction.

Based on the discussion above, we would like to propose the following.

Proposal. 5: For temporal domain Case B, for NW-sided model, RAN2 do not introduce any enhancement that may introduce additional signalling overhead (e.g., smaller periodicity for periodic reporting).

Frequency domain:

For frequency domain prediction, the input data would be the L3 Cell-level (for subcase 2)/beam-level (for subcase 1 and 3) measurement results at a certain time instance for input cell(s). With the legacy measurement report, the UE can already report one latest measurement result for each cell/beam. Therefore, we do not see any need of enhancement on the legacy measurement reporting to support the inference input report for frequency domain prediction for NW-side model.

Proposal. 6: For frequency domain prediction, for NW-sided model, UE can report one latest measurement result for each cell/beam to NW as in the legacy. No further enhancement is needed.

Spatial domain:

For spatial domain prediction, the input data would be the beam-level measurement results for a certain group of beams at a certain time instance. With the legacy measurement report, the NW can already configure UE to report measurement results for specific set of beams (i.e., for specific SSB indexes) based on ssb-ToMeasure, maxNrofRS-IndexesToReport, absThreshSS-BlocksConsolidation. Therefore, we do not see any need of enhancement on the legacy measurement reporting to support the inference input report for spatial domain prediction for NW-side model.

Proposal. 7: For spatial domain prediction, for NW-sided model, UE can report one latest measurement result for each of specific group of beams (i.e., SSB indexes) to NW as in the legacy. No further enhancement is needed.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Common part:

Proposal. 1: For NW-side model inference, RAN2 should consider the ways to reduce the signalling overhead for the report of inference input (e.g., event-triggered reporting, adaptive reporting based on mobility status, …).

Temporal domain Case A:

Proposal. 2: For temporal domain Case A, NW can configure the number of time instances for which UE reports the results (i.e., OW length).

Observation. 1: To support subcase 1 and 3 for cell-level temporal domain Case A, the expected spec. impact is to report “beam-level” RRM measurement results per cell at multiple time instances in one measurement report.

Observation. 2: To support subcase 1 and 3 for cell-level temporal domain Case A, the signalling overhead will increase quite a lot (in proportion to # of beams) for UE to report beam-level measurement results at multiple time instances.

Proposal. 3: For cell-level temporal domain Case A, RAN2 support only subcase 2 for NW-sided model. I.e., UE can report only “cell-level” RRM measurement results at multiple time instances in one measurement report.

Observation. 3: For beam-level temporal domain Case A, the motivation is to enable efficient dedicated RACH resource allocation during HO.

Observation. 4: For beam-level temporal domain Case A, NW-sided model would lead to additional wastage of resources, which contradicts the original motivation of the beam-level prediction.

Proposal. 4: For beam-level temporal domain Case A, for NW-sided model, RAN2 do not introduce any enhancement that may introduce additional signalling overhead (e.g., beam-level measurement results at multiple time instances in one measurement report).

Temporal domain Case B:

Observation. 5: For temporal domain Case B, the motivation is to reduce UE’s measurement effort for energy saving.

Observation. 6: For temporal domain Case B, for UE-sided model, the event-triggered measurement reporting can be used. I.e., UE can report the measurement report only when it detects some event.

Observation. 7: For temporal domain Case B, for NW-sided model, only the periodical measurement reporting can be used. I.e., UE should periodically report the measurement report with very short interval (e.g., 80 ms), which can increase the energy consumption of UE.

Proposal. 5: For temporal domain Case B, for NW-sided model, RAN2 do not introduce any enhancement that may introduce additional signalling overhead (e.g., smaller periodicity for periodic reporting).

Frequency domain:

Proposal. 6: For frequency domain prediction, for NW-sided model, UE can report one latest measurement result for each cell/beam to NW as in the legacy. No further enhancement is needed.

Spatial domain:

Proposal. 7: For spatial domain prediction, for NW-sided model, UE can report one latest measurement result for each of specific group of beams (i.e., SSB indexes) to NW as in the legacy. No further enhancement is needed.