3GPP TSG-RAN WG2 Meeting #131	                                                                                   R2-2505361

Bangalore, India Aug 25th – 29th, 2025

Source:	CATT

Title:	Discussion on Asymmetric DL sTRP UL mTRP

Agenda Item:    8.12.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the suggested open issues by the rapporteur related to the asymmetric DL sTRP UL mTRP. In details, the following issues are covered in Section 2,

- Clarification on the RRC parameter singleDCI-MultiTRP-2TA

Our proposals are summarized in Section 3.

Discussion

Clarification on the RRC parameter singleDCI-MultiTRP-2TA [RRC, Issue-4]

According to the online discussion in last meeting [1], to support the forward-compatibility for the tag2 configuration, a new RRC parameter singleDCI-MultiTRP-2TA was introduced per UL BWP to explicitly enable the R19 sDCI mTRP with 2TA,

Introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter] is configured.”;

Regarding this parameter, one open issue is to clarify it can be applied for both intra-cell and inter-cell sDCI mTRP (including asymmetric TRP) with two-TA,

| Asymmetric DL sTRP/UL mTRP | In the last meeting, we have agreed to introduce a new parameter to enable Rel-19 2TA configuraiton, currently named singleDCI-MultiTRP-2TA. According to RAN1 UE feature, Rel-19 2TA is introduced for both intra-cell 2TA with no mDCI mTRP and inter-cell 2TA with no mDCI mTRP. The new parameter should be applied to both and LS to RAN1 may be needed to inform this. | This could be discussed based on company contributions. |
| --- | --- | --- |

According to the latest RRC running CR, it captures this parameter as follows,

BWP-UplinkDedicated ::= SEQUENCE { … [[ singleDCI-MultiTRP-2TA-r19 ENUMERATED {enabled} OPTIONAL -- Need R ]] } singleDCI-MultiTRP-2TA Enables two TA for single DCI multi-TRP. This field is used in clause x.x in TS 38.214 [19]..

As per the current field description, it only states the enable of two-TA for sDCI mTRP but not limited to intra-cell or inter-cell mTRP. So there is nothing blocking this parameter to enable the intra-cell or inter-cell sDCI mTRP.

For example, according to the latest RAN1 UE feature list, the UE may report the capability of intra-cell or inter-cell sDCI mTRP with two-TA,

| 59. NR_MIMO_Ph5 | 59-4-4a | Support two TAs enhancement for intra-cell beam management | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management | 23-1-1 | … | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management | 23-1-1a | … | Optional with capability signalling |

Then up to the NW, it can enable UE by singleDCI-MultiTRP-2TA along with the candidate configurations as follows,

MIMOParam-r17 ::= SEQUENCE { additionalPCI-ToAddModList-r17 SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF SSB-MTC-AdditionalPCI-r17 OPTIONAL, -- Need N additionalPCI-ToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17 OPTIONAL, -- Need N unifiedTCI-StateType-r17 ENUMERATED {separate, joint} OPTIONAL, -- Need R … } SSB-MTC-AdditionalPCI-r17 ::= SEQUENCE { additionalPCIIndex-r17 AdditionalPCIIndex-r17, additionalPCI-r17 PhysCellId, periodicity-r17 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }, ssb-PositionsInBurst-r17 CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) }, ss-PBCH-BlockPower-r17 INTEGER (-60..50) }

That means if UE is configured with at least one different additional PCI than the serving cell, it is configured with inter-cell sDCI mTRP with two-TA. Otherwise, it is intra-cell sDCI mTRP with two-TA.

Based on above analysis, we think the current RRC parameter singleDCI-MultiTRP-2TA is enough to comply with the intended UE feature list from RAN1, and thus we propose that,

Proposal 1a (RRC, Issue-4): RAN2 to confirm the current RRC parameter singleDCI-MultiTRP-2TA can enable UE to configure either intra-cell or inter-cell sDCI mTRP with two-TA. No further RRC spec impact.

Regarding whether we need LS to RAN1 for this information, according to the RAN1 spec description related to the sDCI mTRP with two-TA [2],

TS 38.214: ## 6.1 UE procedure for transmitting the physical uplink shared channel … If a UE is not configured with coresetPoolIndex for the active BWP of a serving cell, and if the UE is configured with tag2-Id and is configured with dl-OrJointTCI-StateList or TCI-UL-State for a serving cell, each TCI-State or TCI-UL-State is associated with a tag-Id-ptr for determining timing adjustment for a corresponding UL transmission as described in Clause 4.2 of [6, TS 38.213].

It still uses the implicit way to express the sDCI mTRP with two-TA. However, the RRC parameter singleDCI-MultiTRP-2TA can facilitate RAN1 to capture the corresponding description for this feature. Therefore the LS to RAN1 is suggested.

Proposal 1b (RRC, Issue-4): LS to RAN1 that RAN2 introduced a new RRC parameter singleDCI-MultiTRP-2TA per UL BWP to enable the UE configured with intra-cell or inter-cell sDCI mTRP with two-TA. Adopt Annex 1 for the draft LS.

Conclusion

Based on the previous analysis in Section 2, our contributions are summarized as follows:

Proposal 1a (RRC, Issue-4): RAN2 to confirm the current RRC parameter singleDCI-MultiTRP-2TA can enable UE to configure either intra-cell or inter-cell sDCI mTRP with two-TA. No further RRC spec impact.

Proposal 1b (RRC, Issue-4): LS to RAN1 that RAN2 introduced a new RRC parameter singleDCI-MultiTRP-2TA per UL BWP to enable the UE configured with intra-cell or inter-cell sDCI mTRP with two-TA. Adopt Annex 1 for the draft LS.

Reference

[1] R2-2504672	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	Vice Chairman (CATT)	Report

[2] 3GPP TS 38.214: "NR; Physical layer procedures for data"

Annex

3GPP TSG-RAN WG2 Meeting #131                                                                                   R2-250xxxx

Bangalore, India Aug 25th – 29th, 2025

Title:	LS on RRC parameter singleDCI-MultiTRP-2TA for sDCI mTRP with two-TA

Response to:

Release:	Rel-19

Work Item:	NR_MIMO_Ph5-Core

Source:	RAN2

To:	RAN1

Cc:

Contact person:	Lei Wang

wanglei24@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

During the discussion in RAN2 131 meeting, RAN2 reached the following agreement,

- RAN2 to confirm the current RRC parameter singleDCI-MultiTRP-2TA can enable UE to configure either intra-cell or inter-cell sDCI mTRP with two-TA.

RAN2 respectfully asks RAN1 to take above into account when stating sDCI mTRP with two-TA in RAN1 spec.

2	Actions

To RAN1 group

ACTION: 	RAN2 respectfully asks RAN1 to take above into account when stating sDCI mTRP with two-TA in RAN1 spec.

3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #131bis	13th – 17rd Oct 2025	Prague , CZ

TSG-RAN2 Meeting #132	17th – 21rd Nov 2025	Dallas , US