3GPP TSG-RAN WG2 Meeting #131	                                                                                   R2-2505362

Bangalore, India Aug 25th – 29th, 2025

Source:	CATT

Title:	Discussion on UE initiated beam reporting

Agenda Item:    8.12.3

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the suggested open issues by the rapporteur related to the UE initiated beam reporting. In details, the following issues are covered in Section 2,

- Spec impact for UCI multiplexing in PUSCH with Rel-15 UL skipping

- Whether to initiate RACH when UEI report is triggered but TAT is expired

- Whether UEI BM parameters should be moved to CSI-MeasConfig

Our proposals are summarized in Section 3.

Discussion

Spec impact for UCI multiplexing in PUSCH with Rel-15 UL skipping (MAC Editor Note)

According to the discussion in last meeting [1], it was agreed that,

For Rel-16 UL skipping (enhancedSkipUplinkTxDynamic), the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated), there is no MAC impact. For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH. Exact MAC spec can be discussed further.

For Rel-16 UL skipping, companies share the same view that the UCI for mode-A UE initiated CSI reporting can be multiplexed with the MAC PDU, i.e., following the current MAC spec. Then for the Rel-15 UL skipping, the guidance is to follow the same principle as the legacy aperiodic CSI reporting, but the controversy is whether we need to capture a separated description for this case,

1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if there is no mode-A UE-initiated CSI reporting triggered for this PUSCH transmission as specified in TS 38.214 [7]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. Editor’s Note: FFS if the change is needed for the agreement: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

Recalling the evolution of UL grant skipping, the Rel-15 feature only considers the case of the aperiodic CSI triggered by DCI to be multiplexed in the PUSCH. In that way, if UE is enabled with UCI multiplexed with PUSCH and is configured with Rel-15 UL skipping, and if the UCI to be transmitted is HARQ feedback but not aperiodic CSI reporting, the gNB needs to blindly decode the TB with the candidate format of UCI only or UCI + MAC PDU. Because according to the current MAC spec as mentioned above, whether to generate the MAC PDU depends on whether there is remaining data to be transmitted. However, this is un-predictable by the NW.

To solve this issue, the Rel-16 UL skipping is introduced with consideration of other potential UCI type to be multiplexed by PUSCH. By this way, the expected NW implementation is to configure the Rel-16 UL skipping if it wants to multiplex UCI (for example, the UE initiated CSI reporting) with MAC PDU transmitted in the PUSCH. So from this perspective, we prefer not making change to the Rel-15 feature.

Proposal 1 (MAC Editor Note): RAN2 to confirm there is no MAC impact to reflect the following RAN2 agreement,

- For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

Whether to initiate RACH when UEI report is triggered but TAT is expired [MAC, Issue-2]

For UE initiated CSI reporting, it only considers the single TRP case. If the UE initiated CSI reporting is triggered and to be transmitted but the TAT of the corresponding TAG is expired, the UL resource will be cleared by UE including the PUCCH for first channel and the CG PUSCH for the second channel. In this situation, whether to trigger RACH remains to be an open issue, with the candidate options on the table,

Proposal 2: Regarding whether UE initiate RACH when UEI report is triggered but TAT is expired, select one of the following options: - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts. - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3).

In legacy, upon TAT expires, UE does not make the SP-CSI reporting on PUSCH/PUCCH.

1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or 2> if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else: 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

We think for the Mode-B UE initiated CSI reporting with type-1 CG, it is similar to the SP-CSI reporting, so the similar principle should be followed.

Besides, from the NW point of view, the TAT expiry is aware by the NW, so if the NW wants to get the UE initiated CSI reporting, it can trigger PDCCH ordered RACH and then re-configure the PUCCH resource and the type-1 CG resource. That means it can be left to NW implementation to handle this scenario.

Proposal 2 (MAC, Issue-2): UE does not initiate RACH when UE initiated CSI reporting is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

Whether UEI BM parameters should be moved to CSI-MeasConfig [RRC, Issue-1]

Now the RAN1 RRC parameter is stable, and RAN2 captures the UE initiated beam reporting-related parameters in CSI-ReportConfig as follows,

CSI-ReportConfig ::= SEQUENCE { reportConfigId CSI-ReportConfigId, … [[ … eventTypeUE-IBR-r19 CHOICE { event1-r19 SEQUENCE { eventThreshold-r19 RSRP-Range }, event2-r19 SEQUENCE { eventThreshold-r19 INTEGER (0..31) }, event7-r19 SEQUENCE { eventThreshold-r19 INTEGER (0..31), valueOfQ-r19 INTEGER (1..8) } }, OPTIONAL, -- Need R csi-ReportUE-IBR-r19 CSI-ReportUE-IBR-r19 OPTIONAL, -- Cond UE-IBR linkedCJTC-Report-r19 CSI-ReportConfigID OPTIONAL -- Need R ]] }

Then there is an open issue on whether the UE initiated beam reporting parameters should be moved to CSI-MeasConfig,

| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | Since now most of the parameters are stable this could be discussed based on company contributions. |
| --- | --- | --- |

As can be seen from the structure, each reportConfigId is associated with a certain L1 event by the CHOICE structure, and thus it aligns with the RAN1 intention that the DCI triggers a UE initiated beam reporting by mapping the corresponding code-point of CSI request field to the reportConfigId that is specific for UE initiated beam reporting. Besides, the current structure shares the similar way as the event configuration for conditional LTM. So we prefer to keep the current structure.

Proposal 3 (RRC, Issue-1): RAN2 to keep the UE initiated beam reporting-related parameters in CSI-ReportConfig.

Conclusion

Based on the previous analysis in Section 2, our contributions are summarized as follows:

Spec impact for UCI multiplexing in PUSCH with Rel-15 UL skipping (MAC Editor Note)

Proposal 1 (MAC Editor Note): RAN2 to confirm there is no MAC impact to reflect the following RAN2 agreement,

- For Rel-15 UL skipping (skipUplinkTxDynamic is configured), same principle as legacy aperiodic CSI applies for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

Whether to initiate RACH when UEI report is triggered but TAT is expired [MAC, Issue-2]

Proposal 2 (MAC, Issue-2): UE does not initiate RACH when UE initiated CSI reporting is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

Whether UEI BM parameters should be moved to CSI-MeasConfig [RRC, Issue-1]

Proposal 3 (RRC, Issue-1): RAN2 to keep the UE initiated beam reporting-related parameters in CSI-ReportConfig.

Reference

[1] R2-2504672	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others	Vice Chairman (CATT)	Report