3GPP TSG-RAN WG2 #131	R2-2505382

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.3.4 Network sided model

Source:	Ericsson

Title:	Network sided model

Document for:	Discussion

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

This contribution discusses aspects related to the inference configuration, data collection and performance monitoring for NETWORK sided AI/ML model, as part of the LCM framework.

# Discussion

## Supported use case, configurations and UE report

Concerning the supported use cases for the network side models the following agreements are made in the RAN2#130 meeting.

Agreements 1 For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. 2 To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NETWORK-sided model.

As shown subcase 1 and sub case 3 are FFS. The definition of the sub cases captured in the latest version of TR 38.744 are provided below for convenience.

3 sub-use cases are considered for cell-level RRM measurement prediction: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated; - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s); - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s). 3 sub-use cases are considered for beam-level RRM measurement prediction: - Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated; - Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s); - Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

For cell level RRM predictions at the network side, a potential input can be L1 beam level measurements reported by the UE. In the previous meeting, it was noted that the network can obtain the latest L1 filtered measurement results as part of the L3 measurement report by setting the beam level L3 filtering co-efficient to zero.

In the following, the configuration used for L3 filtering coefficients for the cell and beam level measurements is shown. As shown, the network configures a list of potential filtering coefficient sets (quantityConfigNR-List which is a list including a set of cell and beam filtering coefficients) in measConfig. Then the network in every configured measObject configures a pointer/index to an entry in quantityConfigNR-List as shown below.

MeasObjectNR ::=                    SEQUENCE {

[…]

quantityConfigIndex                 INTEGER (1..maxNrofQuantityConfig),

QuantityConfig ::=                  SEQUENCE {

quantityConfigNR-List               SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF QuantityConfigNR          OPTIONAL,   -- Need M

QuantityConfigNR::=                 SEQUENCE {

quantityConfigCell                  QuantityConfigRS,

quantityConfigRS-Index              QuantityConfigRS                                                        OPTIONAL    -- Need M

}

However, we would like to note that the latest L1 measurement result (denoted as Mn) is always available at the RRC layer as highlighted below as the L1 measurement result is used as input to the layer 3 filtering.

TS38.331 V18.5.1 (2025-03) 5.5.3.2 Layer 3 filtering The UE shall: … 2> filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, by the following formula: Fn = (1 – a)*Fn-1 + a*Mn - Where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

The latest L1 filtered measurement results are always available at the RRC layer.

Therefore, enabling sub-case 1 and 3, which require transmitting the L1 filtered RSRP measurements, does not require any extra processing for the UE except including the latest L1 filtered measurements available at the UE in the report, instead of including the L3 filtered beam level measurements, in case the intention of the measurement report is network side inference for sub-case 1 and 3. Such request can be configurable by the network e.g., as part of measReportNR.

Setting the filtering coefficient to zero is not preferred since it may create impairment on normal mobility related operations.

No extra processing for the UE is required to include the L1 measurements even if L3 filtering is not set to zero for mobility purpose.

In addition, we would like to note that there is no need for the network to receive the L3 and L1 beam measurements at the same time. Hence, we don’t see any need to create additional fields in measurement reports for the L1 beam measurements as the fields for the L3 beam measurements can be reused. The network will know what measurements it receives based on the measId. Therefore, the specification impact of supporting sub case 1 and sub case 3 is minimal.

The specification impact of supporting sub case 1 and sub case 3 is minimal, by

- Reusing the existing L3 beam measurement fields in Measurement Report (no need for new fields in Measurement Report).

- The network indicating to the UE whether it should include the latest L1 beam measurements or L3 beam measurement (feasible by a flag in reportConfigNR).

To enable sub case 1 and 3 for the network side models, the UE includes the latest received L1 measurement result from the physical layer in the measurement report, if requested by network (e.g., via reportConfigNR).

## Inference input for network sided model

### Cluster-based prediction

In previous RAN2 meetings, cluster-based approach was studied both for temporal, spatial and frequency prediction cases and potential gain on improving the prediction accuracy. Last meeting it was agreed that the legacy RRM measurement configuration and RRM measurement reporting framework can be used for network side RRM measurement predictions.

RAN2#130 Configuration and report of inference input to network sided model Inference input (measurements from UE) - For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level, RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

However, effectively leveraging measurements of multiple cells/frequencies for accurate cluster-based prediction at the network side requires timely and coordinated input data. The current RRM measurement reporting mechanisms present limitations in providing optimal data characteristics for cluster-based prediction at the network side.

Specifically, for periodic reporting, it lacks adaptability because the reports are sent periodically irrespective of the prediction need or the UE mobility state. It potentially leads to unnecessary signaling overhead, especially during periods where network predictions are inactive or the channel is stable. This inefficient resource utilization concerns the case of scaling AI/ML predictions across numerous UEs and frequencies.

Event-triggered reporting (e.g., A3/A4/A5) is designed to react to specific radio conditions. However, its asynchronous nature across different carrier frequencies poses a significant challenge. It is practically difficult to guarantee that measurement reports of multiple cells/frequencies would be triggered simultaneously or within a sufficiently synchronized time window. The lack of temporal coordination and potential data skew (where reports for different frequency reflect different moments in time) hinders the network’s ability to form a coherent, holistic snapshot of the multi-cell/multi-frequency radio environment at a specific point in time for effective cluster-based prediction.

Therefore, we observe:

The inflexibility of periodic reporting and the asynchronous nature of event-triggered reporting make it challenging to obtain measurements for multiple cells/frequencies with the necessary temporal correlation to serve as reliable input for network-sided cluster-based prediction.

To address the identified challenges, we propose,

Multiple cells/multiple frequencies can be included in one RRM measurement report for network sided model.

## Data Collection for network sided model

At the previous meeting, the following agreement was reached for data collection for NETWORK side model.

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found

We would like to note that it is valuable to log RLF occurrences, even though RLF prediction is out of scope of this WI and SI. RLF information could provide valuable insights for the network to understand when predictions are inaccurate or how predictions could help improving the performance. This aligns with the discussion regarding the retention of logged data during RLFs. If data is to be retained, then logging the RLF occurrence is necessary to ensure transparency for the network.

UE logs an indication of RLF in logged data, if RLF occurs during the data collection for the network side model.

## Performance Monitoring for network sided model

Regarding performance monitoring in AI for mobility, RAN2#129bis agreed that, for UE side model, performance monitoring in AI/ML air interface can be studied as a baseline for AI/ML aided mobility, i.e. network side and UE-side performance monitoring. In the following RAN2#130 meeting, it has been agreed that

Agreements (network-side model) 1 RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case 2 For performance monitoring of NETWORK-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement 3 FFS on UE awareness and preference for NETWORK sided model

Concerning UE awareness, one company during the online discussion mentioned similarities of this feature and the immediate MDT feature, claiming that the UE is aware of MDT measurements motivating this by user consent and the necessity of chipset being aware of such measurement.

We would like to highlight that the relevant MDT measurements (radio measurements) is M1 measurement. M1 measurement over-the-air is performed using the RRM measurement framework at the UE. If the network configures several different A2 reports and only one of them is associated with the MDT related M1 report, then the UE has no idea which of the measurement that is used for MDT and which other ones that are used for normal operational purposes. Therefore, according to the current RRC specification, there is no way for the UE to be aware whether the measurement is performed for the RRM purpose or for the MDT purpose.

MDT M1 measurement (e.g., RSRP measurement) is performed at the UE transparently using the normal RRM measurement framework i.e., chipset is not aware whether the measurement is performed for the MDT purpose or for the mobility purpose.

Another argument from supporting company of the UE awareness was the user consent, arguing that the M1 measurement is known to the UE because of the user consent. We would like to note that the user consent is associated with the end-user/subscriber and stored at the UDM. RAN node retrieves and checks the user consent information before configuring the UE with any measurements without informing the UE. However, the UE might become aware that the measurement is for the sake of immediate MDT or other SON report purpose if the network configures obtainCommonLocation in the otherConfig of RRCReconfiguration message

TS38.331 V18.5.1 (2025-03) 5.3.5 RRC reconfiguration … 5.3.5.9 Other configuration … 1> if the received otherConfig includes the obtainCommonLocation: 2> include available detailed location information for any subsequent measurement report or any subsequent RLF report, SCGFailureInformation, successful handover report, and successful PSCell change or addition report (if received for the associated cell group);

There is no explicit indication in MDT procedure to make the UE aware that the M1 measurement collection is for the sake of MDT.

ObtainCommonLocation which is configured as part of SON/MDT procedures for the sake of attempting to collect UE location information is the only implicit way for the UE to be aware the measurement is for the sake of SON/MDT purpose.

In the light of the above analysis of the SON/MDT procedure, we would like to highlight that there is no precedence of the UE(chipset) awareness of the measurements performed for the SON/MDT purpose.

Similar to the SON/MDT procedures, the UE (chipset) would become aware of the network side AIML related measurements only if there is a specific configuration for such measurements.

There is no need for UE (chipset) awareness of network side AI/ML related measurements.

RAN2 abandons the discussion on UE awareness and leaves the user consent discussion to the relevant working groups (SA3) to discuss it on a need basis.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The latest L1 filtered measurement results are always available at the RRC layer.

Observation 2	Setting the filtering coefficient to zero is not preferred since it may create impairment on normal mobility related operations.

Observation 3	No extra processing for the UE is required to include the L1 measurements even if L3 filtering is not set to zero for mobility purpose.

Observation 4	The specification impact of supporting sub case 1 and sub case 3 is minimal, by

	Reusing the existing L3 beam measurement fields in Measurement Report (no need for new fields in Measurement Report).

	The network indicating to the UE whether it should include the latest L1 beam measurements or L3 beam measurement (feasible by a flag in reportConfigNR).

Observation 5	The inflexibility of periodic reporting and the asynchronous nature of event-triggered reporting make it challenging to obtain measurements for multiple cells/frequencies with the necessary temporal correlation to serve as reliable input for network-sided cluster-based prediction.

Observation 6	MDT M1 measurement (e.g., RSRP measurement) is performed at the UE transparently using the normal RRM measurement framework i.e., chipset is not aware whether the measurement is performed for the MDT purpose or for the mobility purpose.

Observation 7	There is no explicit indication in MDT procedure to make the UE aware that the M1 measurement collection is for the sake of MDT.

Observation 8	ObtainCommonLocation which is configured as part of SON/MDT procedures for the sake of attempting to collect UE location information is the only implicit way for the UE to be aware the measurement is for the sake of SON/MDT purpose.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To enable sub case 1 and 3 for the network side models, the UE includes the latest received L1 measurement result from the physical layer in the measurement report, if requested by network (e.g., via reportConfigNR).

Proposal 2	Multiple cells/multiple frequencies can be included in one RRM measurement report for network sided model.

Proposal 3	UE logs an indication of RLF in logged data, if RLF occurs during the data collection for the network side model.

Proposal 4	Similar to the SON/MDT procedures, the UE (chipset) would become aware of the network side AIML related measurements only if there is a specific configuration for such measurements.

Proposal 5	There is no need for UE (chipset) awareness of network side AI/ML related measurements.

Proposal 6	RAN2 abandons the discussion on UE awareness and leaves the user consent discussion to the relevant working groups (SA3) to discuss it on a need basis.

# References

RP-234055, SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR