3GPP TSG-RAN WG2 Meeting #131		R2-2505401

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:		Discussion on conditional LTM

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #130 meeting, conditional LTM was discussed and most remaining open issues were resolved.

In this contribution, we would like to discuss the remaining open issue MAC-14, as well as some other issues:

- Whether to deactivate the activated Candidate TCI state(s) upon CLTM execution: the similar mechanism was agreed for LTM in Rel-18 and we understand it should be applied to CLTM to have a unified design;

- DC and CLTM co-existence: this issue was discussed in last RAN2 #130 meeting and was postponed to this meeting, it is a stage-2 open issue;

- Max number of maintained TA values exceeds the UE capability: this is closely related to UE behaviour and we think clarification is needed on this issue.

- Discussion

  - MAC-14: Co-existence of LTM and CLTM

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |

For this issue, we propose discussing the following two cases:

- Case 1: Reception of an LTM Cell switch Command MAC CE without valid TA

- Case 2: Reception of an LTM Cell switch Command MAC CE with valid TA

Discussion on case 1

According to the current MAC specification, if a UE receives an LTM Cell Switch Command MAC CE without valid TA, it performs RACH-based LTM. However, since we agreed that the network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration, we wonder whether the case could occur in which the LTM Cell Switch Command MAC CE is sent without valid TA while the CLTM candidate TAT timer for the target cell is still running.

If the above case could occur, the UE should leverage its maintained TA to execute RACH-less LTM instead. This avoids unnecessary handover latency.

Proposal 1: (MAC-14) If a UE receives an (Enhanced) LTM Cell Switch Command MAC CE without valid TA and CFRA resource while the CLTM candidate TAT timer for the target cell is running, it shall perform RACH-less LTM.

Discussion on case 2

As agreed in the previous meeting[1], the C-LTM TAT timer for the target cell will not stop upon C-LTM cell switch execution (if running).

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

In our view, the C-LTM TAT for the target cell should also not be stopped upon LTM cell switch execution triggered by the LTM Cell Switch Command MAC CE while it is running. In this case, it needs to be discussed whether the TA maintained by the UE for the target cell should be updated according to the TA indicated in the LTM Cell Switch Command MAC CE.

For Case 2, one scenario involves the network triggering the UE to perform early-RACH to the target cell. The network receives the TA value and directly triggers the LTM cell switch, including the TA value in the LTM Cell switch Command MAC CE, without transmitting the LTM Candidate Timing Advance Command MAC CE to update the maintained TA on the UE side for the target cell. At this point, the TA sent by the network is more up-to-date than the TA maintained on the UE side (or the UE has no stored TA for the target cell). Therefore, UE should store the TA value indicated in the LTM Cell switch Command MAC CE for the target cell, and the C-LTM TAT for the target cell should be started or restarted. This ensures that the target cell maintains an up-to-date TA value for the subsequent CLTM procedures.

Proposal 2: (MAC-14) If an LTM Cell switch Command MAC CE with valid TA is received, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

  - Stage 2 open issue: DC and CLTM co-existence

In RAN2 #130 meeting, the following issues was discussed[2]:

R2-2503733 Report of [POST129b][119][Mob] Stage-2 CR 38.300 (Apple) Apple discussion Rel-19 NR_Mob_Ph4-Core Proposal 1: RAN2 to discuss and confirm if it is valid to have the UE configured with DC, but the CLTM configuration does not have any CLTM candidate cells with DC configured (i.e., explicit configuration for each of the CLTM candidates to inform the UE to release DC). [Apple]: Will have unofficial offline with interested companies. [Ericsson]: Support the proposal and understand there is no additional stage-3 spec impact to allow it. - Comeback in CB session. - Postpone it to the next meeting.

In our understanding, the understanding that ‘network will ensure that DC will be released in candidate configurations and when CLTM is executed, the UE would release DC’, is correct. Although we agreed that DC should not co-exist with CLTM (i.e., Limit specifying the conditional LTM to the scenario where the UE is in non-DC[3]), to allow the above case can make the network configuration more flexible and to make the UE in DC able to receive CLTM configuration and perform CLTM evaluation.

Therefore, we propose to confirm it can be supported for above scenario.

Proposal 3: RAN2 to confirm that it is valid to have the UE configured with DC, but the CLTM configuration does not have any CLTM candidate cells with DC configured (i.e., explicit configuration for each of the CLTM candidates to inform the UE to release DC).

  - Candidate TCI state(s) handling upon CLTM execution

RAN2#128 meeting agreement - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. RAN2#130 meeting agreement - If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed.

According to the above agreements, early TCI state activation on candidate cell(s) has been supported for CLTM.

Therefore, upon CLTM procedure is triggered, UE only needs to monitor the RS corresponding the selected beam to maintain downlink synchronization and ensure data transmission in the target cell. For the activated TCI state other that the one associated with the selected beam, if no deactivation behavior, UE may still monitor corresponding QCL RS to maintain downlink synchronization. However, it is meaningless and introduces large UE measurement overhead and computation complexity.

Observation 1: Upon CLTM procedure is triggered, UE may still monitor corresponding QCL RS of activated TCI state other than the one associated with the selected beam, which introduces large UE measurement overhead and computation complexity.

Furthermore, deactivating the activated TCI state(s) other than the TCI state after the reception of cell switch command has been supported in Rel-18 LTM[4].

# 21 L1/L2-triggered mobility procedures <Omitted…>After reception of the LTM Cell Switch Command MAC CE, activated TCI states that are not indicated by the MAC CE are deactivated.

Hence, it is reasonable to reuse the same mechanism for CLTM, i.e., the activated TCI state(s) other than the TCI state associated with the selected beam are deactivated upon CLTM procedure being triggered.

Proposal 4: Activated Candidate TCI state(s), other than the TCI state associated with the selected beam, should be deactivated upon CLTM procedure being triggered.

Proposal 5: If Proposal 4 is agreed, send an LS to RAN1 to request them to update corresponding specification.

  - Max number of maintained TA values exceeds the UE capability

As agreed in the previous meeting[1], the max number of TA values a UE can maintain for CLTM is defined as a UE capability, with a value range of 1~8.

- Define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8).

As we know, up to 8 LTM candidate cells can be configured for a UE. Thus, the max number of TA values that a UE can maintain may be lower than the number of configured LTM candidate cells.

If the UE has already reached its maximum capability for maintaining TA values but receives an early TA MAC CE for a CLTM candidate cell whose CLTM TAT is not running, it will be unable to maintain this new TA value. RAN2 needs to discuss and resolve this issue to ensure that there is no mismatch between the UE and the network.

Proposal 6: RAN2 discusses how to handle the case where the maintained TA value number has reached the upper capability limit but an LTM Candidate Timing Advance Command MAC CE for a candidate cell whose TAT is not running is received from the network.

- Conclusion

In this contribution, we discussed open issue MAC-14, and some other issues such as open issue for stage-2, Candidate TCI state(s) handling in CLTM and the UE capability for maintaining TA in CLTM.

We have the following Observations and Proposals:

MAC-14:

Observation 1: Upon CLTM procedure is triggered, UE may still monitor corresponding QCL RS of activated TCI state other than the one associated with the selected beam, which introduces large UE measurement overhead and computation complexity.

Proposal 1: (MAC-14) If a UE receives an (Enhanced) LTM Cell Switch Command MAC CE without valid TA and CFRA resource while the CLTM candidate TAT timer for the target cell is running, it shall perform RACH-less LTM.

Proposal 2: (MAC-14) If an LTM Cell switch Command MAC CE with valid TA is received, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

Stage-2 open issue:

Proposal 3: RAN2 to confirm that it is valid to have the UE configured with DC, but the CLTM configuration does not have any CLTM candidate cells with DC configured (i.e., explicit configuration for each of the CLTM candidates to inform the UE to release DC).

Candidate TCI state(s) handling

Proposal 4: Activated Candidate TCI state(s), other than the TCI state associated with the selected beam, should be deactivated upon CLTM procedure being triggered.

Proposal 5: If Proposal 4 is agreed, send an LS to RAN1 to request them to update corresponding specification.

UE capability on max number of maintained TA

Proposal 6: RAN2 discusses how to handle the case where the maintained TA value number has reached the upper capability limit but an LTM Candidate Timing Advance Command MAC CE for a candidate cell whose TAT is not running is received from the network.

- References

- RAN2 #129bis meeting chair notes;

- RAN2 #130 meeting chair notes;

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4;

- 3GPP TS 38.213 V19.0.0 (2025-06);