3GPP TSG-RAN WG2 Meeting #131		R2-2505407

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on MAC and RRC open issues for UEI BMR

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson), one RRC open issues related to UEI BMR is listed as below [1]:

| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | Since now most of the parameters are stable this could be discussed based on company contributions. |
| --- | --- | --- |

In the email discussion on [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung), some MAC open issues are listed in [2].

Issue 2: Does UE initiate RACH when UEI report is triggered and to be transmitted but TAT is expired?

In this contribution, we will discuss the above open issues related to UE-initiated/event-driven beam management, including RRC and MAC impacts.

- Discussion

  - RRC open issues

For the below open issue:

| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | Since now most of the parameters are stable this could be discussed based on company contributions. |
| --- | --- | --- |

First, the motivation of the FFS needs further clarification. If it means to introduce a new RRC parameter, like UEI-CSI-ReportConfigToAddModList, to provide the configuration of the UE-initiated/event-driven beam management, it violates the RAN1 specification. According to TS38.214[3], the following agreement achieved in RAN1#121 meeting [4] has been captured as below.

RAN1#121 meeting agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH - UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH Note-1: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. Note-2: How to capture the above is up to Editor.

TS38.214 v19.0.0 5.2.5 Priority rules for CSI reports For two overlapping PUSCHs, the priority rules in this clause are applied for physical channels with same priority index according to clause 9 in [6, TS 38.213] if a UE is not configured with sTx-2Panel or a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSet in the active DL BWP and the UE is configured with sTx-2Panel and the two overlapping PUSCHs are associated with same value of coresetPoolIndex. CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeA’, y =1 for aperiodic CSI reports to be carried on PUSCH, y = 2 for CSI reporting with CSI-ReportConfig with eventType and with reportTransmissionMode set to ‘ModeB’,y = 3 for semi-persistent CSI reports to be carried on PUSCH, y = 4 for semi-persistent CSI reports to be carried on PUCCH and y = 5 for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR and for CSI reports not carrying L1-RSRP, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations A first CSI report is said to have priority over second CSI report if the associated value is lower for the first report than for the second report.

According to the above RAN1 specification, RAN1 has assumed that the configuration of UE-initiated/event-driven beam management is achieved based on the legacy CSI report configuration framework. The report configuration of UE-initiated/event-driven beam management is included in the csi-ReportConfigToAddModList and sorted with the legacy CSI report configuration.  Therefore, if UEI-CSI-ReportConfigToAddModList is introduced, it will introduce great impact on the RAN1 specification. Firstly, the definition of s and Ms should be revised to UEI-CSI-ReportConfigId and maxNrofUEI-CSI-ReportConfigurations for a CSI report configured with UEI-CSI-ReportConfig. Furthermore, CSI reports for beam and CSI are included in the legacy CSI report configuration framework. However, the UEI report configuration is only applicable to beam report. Thus, the value of maxNrofCSI-ReportConfigurations is larger than that of maxNrofUEI-CSI-ReportConfigurations. In this case, the priority rule based on the legacy priority formula is not aligned with the corresponding agreement. As listed in the following table, if the priority formula is reused, the UEIBMR with Mode B has a higher priority than the aperiodic CSI report, which violates the agreement. Therefore, further discussion is needed to determine how to capture the agreement in RAN1 if a new RRC parameter, like UEI-CSI-ReportConfigToAddModList, is introduced to provide the configuration of the UE-initiated/event-driven beam management.

|  | Legacy CSI report | UEIBMR |
| --- | --- | --- |
| Ncells | 8 | 8 |
|  | 48 | 10 |
| y | 1 | 2 |
| k | 0 | 0 |
| c | 1 | 1 |
| s | 6 | 6 |
| priority value | 822 | 336 |

It has been assumed that the configuration of UE-initiated/event-driven beam management is achieved based on the legacy CSI report configuration framework in the RAN1 specification.

To avoid a large RAN1 specification impact, it is reasonable to support UE-initiated/event-driven beam management by extending the legacy CSI report configuration framework. Similar to the event-triggered reporting configuration for LTM, a new report type, i.e., UEI-BM can be introduced in the CSI-ReportConfig. Under the parameter of the new report type, the parameters related to UE-initiated/event-driven beam management, including event-related parameters, reporting uplink resource configuration and report content configuration can be included. In summary, compared to moving UEI BM parameters to CSI-MeasConfig, extending the legacy CSI report configuration framework to support UE-initiated/event-driven beam management is preferred.

(RRC-x) RAN2 supports to extend the legacy CSI report configuration framework to support UE-initiated/event-driven beam management, i.e. not moving UEI BM parameters to CSI-MeasConfig.

(RRC-x) Similar to the event-triggered report configuration framework for LTM, a new report type, i.e., UEI-BM could be introduced in the CSI-ReportConfig and the UEI-BM corresponding parameters can be replaced under the parameter of the new report type.

  - MAC open issue

Regarding the open issue:

Issue 2: Does UE initiate RACH when UEI report is triggered and to be transmitted but TAT is expired?

According to the discussion in RAN2#130 meeting, below options are identified and discussed in the post meeting email discussion [2], but no consensus has been achieved.

Option 1: UE does not initiate RACH when UEI report is triggered and to be transmitted but TAT (associated to the UEI report resource) is expired. Option 2: UE initiates RACH when UEI report is triggered and to be transmitted but there is no PUCCH or type-1 CG as the associated TAT is expired, and UE waits for NW to reconfigure new resource for UEI report. Option 3: UE initiates RACH when UEI report is triggered and to be transmitted but there is no PUCCH or type-1 CG as the associated TAT is expired, and UE indicates the cause of RACH (e.g., in Msg3). Option 4: UE initiates RACH when UEI report is triggered and to be transmitted but there is no PUCCH or type-1 CG as the associated TAT is expired, and UE transmits UCI for the report in Msg3 or using the uplink grant scheduled by PDCCH addressed to C-RNTI based on which RA procedure is completed.

According to the current specification, UE does not initiate RACH when TAT is expired, except for the demand on data transmission. As UEI report is a type of UCI transmission rather than data transmission, following the legacy behavior is sufficient, i.e. option 1. From the other perspective, the options for optimization, i.e., Option 2~4, should not be supported with the following considerations:

- Option 2 is useless as the UEI report may be outdated when the resource reconfiguration is provided.

- Compared to Option 2, only the cause of RACH is introduced in Option 3. However, this information has no benefits on the UEI reporting latency, as the new PUCCH resource or type-1 CG for UEI report is provided by RRC reconfiguration. Therefore, Option 3 is worthless.

- Compared to Option 2 and 3, Option 4 has less reporting latency for UEI report than Option 3. However, according to our RAN1’s colleagues, Option 4 can introduce much RAN1’s specification impact, such as report content and CPU occupation time. According to the following agreement achieved in RAN1 [5][6], the PUCCH resource carrying the 1-bit indication for UEI report implicitly indicates the CC on which the report configuration is configured and is associated with the value determination of the CSI report configuration indication included in the report content. If the design of Msg3 or the uplink grant scheduled by PDCCH addressed to C-RNTI to carry the 1-bit indication cannot achieve the same functionality as the PUCCH resource, the report content included in the Msg3 or the uplink grant scheduled by PDCCH addressed to C-RNTI needs to be redefined and discussed in RAN1.  Furthermore, if Option 4 is adopted, the CPU occupation time of UEI report needs to be redesigned with consideration of the RACH procedure.

RAN1#119 agreement Confirm the following working assumption in RAN1#117: On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. RAN1#120 agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19

With this, we think the option 1 is preferred and enough.

(MAC-2) UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts, i.e. option 1.

- Conclusion

In this contribution, we discuss the RRC and MAC open issues on UE-initiated/event-driven beam management, identified in the post meeting email discussion. Based on the above discussion, we have the following proposals.

RRC open issues:

- (RRC-x) RAN2 supports to extend the legacy CSI report configuration framework to support UE-initiated/event-driven beam management, i.e. not moving UEI BM parameters to CSI-MeasConfig.

- (RRC-x) Similar to the event-triggered report configuration framework for LTM, a new report type, i.e., UEI-BM could be introduced in the CSI-ReportConfig and the UEI-BM corresponding parameters can be replaced under the parameter of the new report type.

MAC open issue:

(MAC-2) UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts, i.e. option 1.

- References

- R2-2505807, Open issues for MIMO on 38.331, Ericsson

- R2-2505424, Report of MAC open issues for MIMO, Sumsung

- RAN1#121 Chairman's notes.

- 3GPP TS 38.214: "NR; Physical layer procedures for data"

- RAN1#119 Chairman's notes.

- RAN1#120 Chairman's notes.