3GPP TSG-RAN WG2 Meeting #131	R2-2505425

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.12.2

Source:	Samsung

Title:	RRC parameters for two-TA operation

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the RRC parameters for Rel-19 two-TA operation.

# Discussion

Based on the Rel-19 UE feature list (R1-2504675), Rel-19 two-TA is supported for 1) intra-cell beam management with no mDCI mTRP and 2) inter-cell beam management with no mDCI mTRP.

| 59. NR_MIMO_Ph5 | 59-4-4a | Support two TAs enhancement for intra-cell beam management | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management | 23-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management is not supported | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-4b | Support two TAs enhancement for inter-cell beam management operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management | 23-1-1a | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported | Per FS | No | No | n/a |  | Optional with capability signalling |

And RAN1 has the following agreement regarding Rel-19 two-TA operation.

Agreement RAN1#120bis - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

For intra-cell beam management 2TA with no mDCI mTRP, asymmetric DL sTRP/UL mTRP is one use case scenario.

For inter-cell beam management 2TA with no mDCI mTRP, “PRACH association indicator” in DCI indicates the DL reference timing associated to one of the two TAGs. According to the RAN1 RRC parameters list (R2-2505000), prachAssociationDCI-1-0 is introduced to enable the PRACH association indicator in DCI.

prachAssociationDCI-1-0 Enables the presence of 1-bit DCI field "PRACH association indicator" in DCI format 1_0, which can be present in DCI format 1_0 when this RRC parameter and SSB-MTC-AdditionalPCI are configured and the UE is not configured with multi-DCI based multi-TRP (see TS 38.212 [17], clause 7.3.1).

For Rel-18 mDCI mTRP with two TAs, tag2-r18 in ServingCellConfig is introduced. It is also applied to Rel-19 2TA.

Tag2-r18 ::=                        SEQUENCE {

tag2-Id-r18                         TAG-Id,

tag2-flag-r18                       BOOLEAN,

n-TimingAdvanceOffset2-r18          ENUMERATED { n0, n25600, n39936, spare1 }                           OPTIONAL    -- Need S

}

tag2 This field is used to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if coresetPoolIndex for a BWP is configured with more than one value or if singleDCI-MultiTRP-2TA is configured for a BWP.

In the last meeting, we have agreed to introduce a new parameter to enable Rel-19 sDCI mTRP two-TA to avoid impact to Rel-18 mDCI mTRP two-TA configuration.

Introduce a new RRC parameter per BWP that explicitly enables the Rel-19 sDCI-mTRP 2TA, and clarify in FD of tag2 to include all cases where tag2 is configured that “it is optionally configured in a serving cell for mDCI mTRP 2TA if coresetPoolIndex for a BWP is configured with more than one value, and for sDCI mTRP 2TA if [the new parameter] is configured.”

[[

singleDCI-MultiTRP-2TA-r19             ENUMERATED {enabled}                                                OPTIONAL   -- Need R

]]

singleDCI-MultiTRP-2TA This field is used in clause x.x in TS 38.214 [19].

Since Rel-19 two-TA feature is supported for both intra-cell and inter-cell operation without mDCI mTRP, the new parameter (singleDCI-MultiTRP-2TA) should be applied for both intra-cell and inter-cell 2TA without mDCI mTRP. The field description should be updated to capture this clarification, and a LS to RAN1 may be needed.

Proposal 1: RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA and inter-cell 2TA for which mDCI mTRP is not configured. Update the field description to capture this and informs RAN1 by LS.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA and inter-cell 2TA for which mDCI mTRP is not configured. Update the field description to capture this and informs RAN1 by LS.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.

- R2-2504672 Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)

# Appendix

# Title:	LS on RRC parameter for Rel-19 two-TA

# Response to:	-

# Release:	Rel-19

# Work Item:	NR _MIMO_Ph5

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact Person:

Name:	xxxx

E-mail Address:	xxxx

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 have agreed to introduce a new RRC parameter to enable the configuration for Rel-19 two-TA operation which includes both the intra-cell 2TA without mDCI mTRP and the inter-cell 2TA without mDCI mTRP.

2. Actions:

To RAN1

ACTION: RAN2 respectfully asks RAN1 to take into account the above information for Rel-19.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG2 Meeting #131bis	13th – 17th October 2025	Prague, Czech Republic

TSG RAN WG2 Meeting #132	17th – 21st November 2025	Dallas, USA