3GPP TSG-RAN WG2 Meeting #131	R2-2505426

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.12.3

Source:	Samsung

Title:	Open issues on UE-initiated CSI Reporting

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues on UE-initiated report.

# Discussion

## UE-initiated CSI reporting when TAT expired

Regarding the issue when UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report due to TAT expired, UE behaviour has been discussed.

It has been agreed UE releases the PUCCH and type-1 CG resource for UEI report when TAT expires.

When the TAT of the pTAG expires, UE releases PUCCH resource for mode-A/B UEI report and clears type-1 CG for mode-B UEI report. FFS for the case when the TAT expires on the sTAG.

Regarding whether to trigger RACH to transmit the triggered UEI report, the following proposals are considered based on the post#130 offline discussion [3].

Proposal 2: Regarding whether to initiate RACH when UEI report is triggered but TAT is expired, select one of the following options: - Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts. - Option 3: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and UE indicates the cause of RACH. Discuss how to indicate the cause of RACH (e.g., MAC CE in Msg3). Proposal 3 (6/10): For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

In our view, it would be helpful to trigger RA procedure when UEI report is triggered but no resource is available due to TAT expiry. This can at least make UL resynchronization. RA triggered for UEI report is similar to the RA triggered for SR. In the current procedure for SR, UE initiates RACH if there is no valid PUCCH resource for pending SR. For UE-initiated report the UCI in PUCCH is similar to SR.

However, the difference to SR is that for SR, UE can directly transmit UL data using the UL grant in RAR, while for UEI report, without the UEI report indicator NW cannot know it is the UEI report that triggers the RA, so may not reconfigure new PUCCH/PUSCH resource for UEI report. In this case, one way is to totally wait for the NW to reconfigure but the UEI report can become stale or useless. For better performance, UE can send a UEI report indicator in Msg3, this is to serve the same purpose of UEI report indicator by UCI in PUCCH.

For RA in RRC connected mode, only C-RNTI MAC CE is transmitted in Msg3 using the UL grant in RAR, which has 8 bits for subheader and 16 bits for payload. The UL grant in RAR can support at least 56 bits in order to transmit CCCH or 72 bits in order to transmit CCCH1. The remaining bits can be used to include a MAC CE indicating the UEI report and the MAC CE can have zero bits payload.

Proposal 1: UE initiates RA procedure when UEI report is triggered but no PUCCH or type-1 CG is available due to TAT expiry.

Proposal 2: Introduce a MAC CE with zero-bit payload to indicate the UEI report indicator in Msg3. The MAC CE is identified by a LCID.

For mode-A UEI report, if TAT is expired after transmitting UEI report indicator in PUCCH and before transmitting UEI report in PUSCH, since PUSCH is scheduled by DCI and NW can initiate PDCCH-order RACH to recover UL synchronization for UE to transmit the DCI-scheduled UEI report.

Proposal 3: For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: UE initiates RA procedure when UEI report is triggered but no PUCCH or type-1 CG is available due to TAT expiry.

Proposal 2: Introduce a MAC CE with zero-bit payload to indicate the UEI report indicator in Msg3. The MAC CE is identified by a LCID.

Proposal 3: For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

# References

- RP‑242356	Revised Work Item: NR mobility enhancements Phase 4, RAN#105.

- R2-2504672	Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)

- R2-250xxxx	Report of MAC open issues for MIMO, Samsung