3GPP TSG-RAN WG2 Meeting#131        			                                                               R2-2505430

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.2.4

Source:	ETRI

Title:	Remaining open issues on data transmission

Document for:	Discussion and Decision

# Introduction

In last RAN2#130 meeting, followings are agreed for A-IoT data transmission[1].

Agreements on Segmentation 1. R2D message scheduling non-first segment (re)transmission does not include upper layer command. 2. For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present. 3. This implies that the R2D message will either have command or offset (but not both). FFS whether we define two message types or one message type with optional fields.

Agreements on Unsuccessful Command 1. The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion. If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed. 2. Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response. RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command). Ask if this can be handled by CT1

Agreements on MAC PDU format 1. A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is 7-bit in bytes. 2. The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes. Segmented SDUs are also byte aligned. 3. FFS D2R message type. Current running CR will capture no message type, but we can revisit this next meeting and also consider if any other bits are needed for the MAC header 4. The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed. FFS can come back if padding is needed depending on granularity of TBS (only if needed)

Agreements on AS ID For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not. FFS for CFRA FFS for need for release message

In this paper, we discuss remaining issues for A-IoT data transmission and propose our views.

# Discussions

## Issue 3-3: AS ID release

An AS ID is the AS layer identifier used to address a specific A-IoT device for R2D reception and D2R scheduling. An AS ID is configured for a service using either a random ID message for CBRA or an R2D upper layer data transfer message for CFA. The AS ID can be released after a service is finished, or in cases where an error occurs and the service is terminated.

The basic mechanism for AS ID release is as follows. The AS ID is released when the device receives a paging message. Upon receiving this message, the device recognizes that a new service has been triggered and releases the stored AS ID. The AS ID is also released when the device receives a NACK feedback message from the reader. In this case, the device releases the AS ID stored in memory and performs a re-access.

During an email discussion on remaining A-IoT MAC running issue, some companies proposed introducing explicit signaling for AS ID release. Unicast, broadcast, or multiplexing with a list of AS IDs were discussed as mechanisms for explict signalling. Using an explicit release message offers some advantages. It allows for faster AS ID release, which is particularly beneficial given the potentially long periods between subsequent paging messages. This early indication of AS ID release also helps manage the limited number of AS IDs that can be configured for devices, especially in dense deployment scenarios. However, this mechanism has some negative impacts. The main issue is the overhead caused by explicit signaling. The release message consumes resources on the R2D channel and increases device complexity and power consumption due to the message processing. Considering unicast signalling, the overhead would occur due to frequent message transmission, as a separate message would be needed for each device. If multiplexing with a list of AS IDs is used, the message size would be larger, as it must contaion multiple identifiers.

In our view, an explicit message for AS ID release is an optimization. Therefore, we do not believe it is necessary to introduce an explicit message for this purpose.

Proposal 1. An explicit message is not required for AS ID release.

## Issue 3-5: D2R message type

The core issue is whether D2R message type is necessary.

There are two primary D2R messages: the Random ID message and the D2R Upper Layer Data Transfer message. Since both are transmitted on resources configured by a reader, the reader can already anticipate which message to expect. For this reason, we believe a D2R message type is not essential for Rel-19 A-IoT.

However, some companies have argued that a D2R message type would be beneficial for Rel-20 use cases, such as the DO-A scenario. While DO-A traffic is being introduced in Rel-20, a message type may not be strictly mandatory, as resources can be pre-configured for specific type of traffic. Nevertheless, from a protocol design perspective, a D2R message type would be a clear way for the reader to identify the message being received. Additionally, because it does not introduce significant complexity, adding a D2R message type is a viable consideration.

Therefore, a D2R message type is not critical for Rel-19 A-IoT, but it may become a necessary component for future releases.

Proposal 2. A D2R message type is not critical for Rel-19 A-IoT, but it may become necessary for future releases.

## Issue 3-7: More data indication

When the More Data Indicator (MDI) is configured in a Device-to-Reader (D2R) transmission, it signals to the reader that additional data is exist and may be transmitted. The MDI can be applied in two scenarios: segmenated data transmission and delayed response to commands.

In segmationation case, the device MAC can determine how many segments are buffered for transmission. Upon receiving a D2R transmission with MDI set to ‘1’, the reader interprets this as an indication of upcoming segments and proceeds to allocate more D2R transmission resources accordingly. This allows efficient scheduling and resource preparation on the reader to handle segmented data without interruption.

However, in the case of delayed response to a command, the device MAC cannot accurately determine the data volume that will be sent, as it does not receive that information from the upper layer in advance. Even if the MDI is set in the D2R transmission, the reader has no way to identify the time when the corresponding D2R resources should be scheduled.

As a result, while MDI configuration is useful in segmentation scenario, its utility is limited in situations where data availability is unpredictable, such as with delayed command responses. Therefore, configuring the MDI field is not essential in these cases.

Proposal 3. The More Data Indicator (MDI) is not used to indicate delayed response to a command.

## Issue 3-8: R2D TBS

R2D messages can have either a fixed length or a variable length. The Access Trigger message is the only fixed-size message, while other R2D messages, such as A-IoT paging, Random ID response, R2D Upper Layer Data Transfer, and NACK feedback message, may have a variable size.

From a protocol design perspective, a unified header format is preferable. However, the Access Trigger message contains only 3 bits information and is transmitted very frequently during A-IoT protocol operations. If we were to use a 1-Byte TBS field with 5bits for padding, approximately 81.25% of the transmission resource (i.e., 13 out of 16 bits) would be wasted, which brings a significant inefficiency. Additionally, since a device can identify the message type by decoding the R2D message type field in the MAC PDU, the TBS field is not essential for the device to determine the message size.

Proposal 4. A separate R2D TBS field is not needed for fixed-size R2D messages.

For a variable size MAC PDU, a TBS field is mandatory because the device needs a mechanism to identify the MAC PDU size. The TBS field should be designed to accommodate all possible MAC PDU sizes. According to the RAN1 decision [2], the maximum TBS size is 1,000 bits, meaning a 7-bit length would be sufficient. However, considering byte alignment, an 8-bit TBS field is also a valid consideration.

Proposal 5. A 7-bit or 8-bit R2D TBS field is needed for variable-size R2D messages.

# Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1. An explicit message is not required for AS ID release.

Proposal 2. A D2R message type is not critical for Rel-19 A-IoT, but it may become necessary for future releases.

Proposal 3. The More Data Indicator (MDI) is not used to indicate delayed response to a command.

Proposal 4. A separate R2D TBS field is not needed for fixed-size R2D messages.

Proposal 5. A 7-bit or 8-bit R2D TBS field is needed for variable-size R2D messages.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #130”, May. 2025

- 3GPP TR 38.769, Study on solutions for ambient IoT (Internet of Things), Dec., 2024