3GPP TSG-RAN WG2 Meeting #131	R2-2505434

Bengaluru, India, Aug. 25th – 29th, 2025

Agenda Item:	 8.13.3

Source:	 Huawei, HiSilicon

Title:	 Control Plane Procedures for Multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

In RAN2 #130 meeting, good progress was made on the control plan procedures for multi-hop relay, and the following agreements were reached as shown below.

RAN2#130 meeting agreements [1]:

Agreements on baseline Control Plane design: Do not pursue fast forwarding. Continue during this meeting to discuss including the RRC state in relay discovery, including: - Spec impact to capture it in the RRC container - Implications for (re)selection procedure Continue discussion of whether/how to extend timers. The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container. As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed. No SA2 spec impact is expected. Companies are asked to confirm with SA2 colleagues if there is any concern with a candidate first relay UE sending the RRC container with its own information. Extend the T300, T301 and T319 timers for multi-hop U2N relay operation. The UE and gNB multiply the signalled timer value by the hop count. Agreements on SRAP: Do not implement reflective bearer mapping in the running CR(s) now; interested companies are asked to converge a single detailed TP for final go/no-go decision next meeting. Agreements on system information: There is no remote UE ID in SIB request or SIB delivery, i.e. RemoteUEInformationSidelink and UuMessageTransferSidelink. In-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire SIB broadcast by the last relay UE’s serving cell via Uu interface directly for it’s own concerned SIB. For the SIB requested by child UE, if it is also the concerned SIB of intermediate relay UE, in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly. For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, regarding in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly. Below b), c), and e) are applied also in IDLE/INACTIVE states for intermediate relay UE without additional specification change. The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node: b) Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) c) Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) e) Upon receiving updated SIB1 from the network (as in Rel17) For the SIB requested by child node, if the SIB has not been included in SIB request in parent link, if IDLE/INACTIVE intermediate relay UE obtain the requested SIB via Uu interface or unsolicited SIB1 forwarding and CAN respond SIB request of child UE, adopt legacy condition(“if the UE has SIB request information to provide”) to determine SIB request in parent link is needed or not. Applicability of this legacy language to the multihop case to be documented in RRC. Legacy condition “if the UE has SIB request information to provide” is enough, no additional specification change is needed to support “when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu” (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

Since the RAN2 131 meeting is the last meeting for this work item, in this paper, we discuss all the left open issues and try to close them.

# 2	Discussion

## Convert the working assumption to agreement

In RAN2 #130, the following working assumption was made:

Working assumption: The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container. As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed Companies are asked to confirm with SA2 colleagues if there is any concern with a candidate first relay UE sending the RRC container with its own information.

After consultation with SA2 colleagues, it was confirmed that there are no restrictions on either the candidate first relay UE or the candidate intermediate relay UE sending the RRC container with its own information for both Model A and Model B Discovery Procedures.

Proposal 1 – RAN2 confirms the working assumption from RAN2 #130, namely that the Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container.

## 2.2 RRC-15 - Guidance for Remote UE to Select or Reselect a Suitable Relay UE in RRC_CONNECTED to Ensure Low Latency

In multi-hop scenarios, it is advantageous for the Remote UE to select or reselect a suitable NR sidelink U2N Relay UE that satisfies both AS-level and higher-layer criteria (together covering all parameters required for relay reselection) and is in the RRC_CONNECTED state. This helps to ensure lower latency during connection establishment.

To address this, a note has been added to the running CR in section 5.8.15.2 providing such guidance to UEs, using “may” to indicate its optional nature.

NOTE X:	The L2 U2N Remote UE may prioritize the selection or reselection of suitable NR sidelink U2N Relay UE that is in RRC_CONNECTED state, based on the RRC state information included in the Discovery Message container.

Without such optional guidance, relying solely on implementation-specific behavior may result in consistently high system latency, with no guarantee of improvement

Proposal 2 – RAN2 confirms that the existing note, stating that the Remote UE may prioritize selecting/reselecting a suitable Relay UE that is in the RRC_CONNECTED state based on RRC state information included in the Discovery Message container, is retained in the running CR.

## Authorised Relay UEs remaining in RRC_CONNECTED for longer duration

Considering main use case for multi hop relay is for emergency or disaster recovery scenario, certain Relay UE(s) can be authorised to remain in RRC Connected State for longer duration after establishing the connection with the network, mainly to serve other intermediate and remote UEs while  the recovery operation is ongoing. This authorisation information can be provided to the UE at the NAS layer and gNB can receive the authorization for these relay UEs from the CN when the connection is established in NGAP : INITIAL CONTEXT SETUP REQUEST message.

After the emergency or disaster recovery operation is completed, the Relay UEs can request the RRC Connection Release from the network through UE Assistance Information message which will trigger the RRC Connection release for these UEs.

In summary this solution would require the following specification impacts

- Authorisation for Relay UEs to remain in RRC_CONNECTED state to be included in TS 23.304 and TS 38.413

- Indication in UE Assistance Information message to the network to initiate the release RRC Connection when there are no connected UEs for certain time.

Considering the benefits and minimal specification impact of this the solutions described above, we propose the this solution be considered for R19

Proposal 3: Given that the primary use case for multi-hop relay is in emergency or disaster recovery scenarios, it is proposed that certain Relay UEs be authorized by the CN to remain in RRC_CONNECTED for an extended period after initial connection establishment. This approach would enhance their availability to provide low-latency relay services.

Proposal 4: If RAN2 agrees to support such a solution, it is proposed to send an LS to RAN3 and SA2 requesting the necessary authorization mechanisms for such Relay UEs. Furthermore, it is proposed to include an indication in the UE Assistance Information message allowing the network to trigger RRC connection release when no connected UEs are present for a specified duration.

## 2.4 RRC-13: Fine Tuning of Extended Timer Values Covering Both Uu and PC5 Hops

Figure 1. multi-hop relay with one Uu hop and three PC5 hops

We had discussed to extend the timers for the connection establishment /reestablishment/resume with multi-hop since the legacy timer is only for the single-hop. It was agreed “the UE and gNB multiply the signalled timer value by the hop count”.

In the current running CR, the Remote UE timer values, UE-TimersAndConstantsRemoteUE, which include components from both the Uu and PC5 hops, were considered to be multiplied by the hop count. However with this approach the value of the timer for the multi hop remote UE might be quite large as the Remote UE timer already considers one PC5 hop and one Uu hop and multiplying it with the hop count will result in higher values.

As shown in figure 1, for an n-hop relaying path, there is one Uu hop and n PC5 hops. We think the timer should be extended by a combination of two parts, the Uu hop and PC5 hops. In legacy, we already specified  the timers for Uu hop, i.e., t300/t301/t319 as below, which is used by a normal UE to connected to the network via Uu link.

One straight forward option is that instead of multiplying the timers in UE-TimersAndConstantsRemoteUE  with the hop count we multiply the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count.

The other option is that for multi-hop relay, we can introduce a timer for single PC5 hop, e.g., t300-PC5/t301-PC5/t319-PC5. For a remote UE connecting with a n-hop relaying path, i.e., with one Uu hop and n PC5 hops, the overall timer for connection establishment /reestablishment/resume can be calculated as

t300 for n-hop relay = t300 + t300-PC5*n

t301 for n-hop relay = t301 + t301-PC5*n

t319 for n-hop relay = t319 + t319-PC5*n

-------------------------------------------------------------TS 38.331 V18.6.0-----------------------------------------------------------

#### –	UE-TimersAndConstants

The IE UE-TimersAndConstants contains timers and constants used by the UE in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE.

UE-TimersAndConstants information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTS-START

UE-TimersAndConstants ::=           SEQUENCE {

t300                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t301                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

t310                                ENUMERATED {ms0, ms50, ms100, ms200, ms500, ms1000, ms2000},

n310                                ENUMERATED {n1, n2, n3, n4, n6, n8, n10, n20},

t311                                ENUMERATED {ms1000, ms3000, ms5000, ms10000, ms15000, ms20000, ms30000},

n311                                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10},

t319                                ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000},

...

}

------------------------------------------------------------------------------------------------------------------------------------------------

Proposal 5: Extend the T300, T301 and T319 timer for multi-hop U2N relay operation in accordance with the hop count for connection establishment/ re-establishment and resume procedure by either

- Multiplying the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count; or

- Introducing PC5-specific timers (t300-PC5/t301-PC5/t319-PC5) and calculating the timer value as t300 + t300-PC5*n, where n is the number of PC5 hops and one Uu hop is included.

## 2.5 SRAP configuration

For SRAP configuration and bearer mapping, we have the following FFS.

Do not implement reflective bearer mapping in the running CR(s) now; interested companies are asked to converge a single detailed TP for final go/no-go decision next meeting.

In RAN2 #129 meeting, it was agreed that the remote UE ID and bearer ID can be included in the SRAP PDU header for data forwarding. For a relay UE, it will receive the SRB/DRB data from the ingress relay RLC channel of the ingress link, and forward the data to the egress relay RLC channel of the egress link.

In the UL, when the relay UE receives the data from its child UE (a relay UE or a remote UE), it will forward the data to its parent relay UE according to the SRAP PDU header and the bearer mapping configuration. The bearer mapping configuration contains the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

In the DL, the relay UE can also forward the data based on the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel of the egress link, and ingress PC5 relay RLC channel between the relay UE and its parent UE.

Besides, since the relay UE can connect to several child UEs if it is not the first relay UE, the path information corresponding to the egress link is needed.

The SRAP configuration can explicitly provide the path information through the identity (L2 ID) of the child relay UE for the intermediate and last relay UE.

Therefore, the following proposal is proposed

Proposal 6: For handling the Multi hop relay the path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

## Error handling

In single-hop relay scenario, the relay UE and remote UE do error handling by comparing the bearer ID and local ID information and the mapping information, except the following case for the relay UE:

- the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID

The above error handling is for path switching case when an IDLE/INACTVIE relay UE is selected and the remote UE sends the SRB1 message via the IDLE/INACTVIE relay UE, but the mapping configuration on the relay UE side cannot be ready.

Figure 2. Connection establishment for multi-path relay

Figure 2 shows the connection establishment procedure for multi-path relay. In step 1 and step 2, the remote UE selects a proper path and establishes the PC5 connection with the first relay UE. The first relay UE triggers the SUI message in step 3. After receiving the SUI message form the first relay UE, which indicates the newly connected remote UE identity, the gNB need configure the SRB0 mapping information for all relay UEs. In the meantime, the remote UE sends the SRB0 message to the first relay UE without SRPA header. Therefore, the first relay UE will not do the error handling for the SRB0 message, and the first relay UE will forward after receiving the SRB0 mapping information and contain the bearer ID and local ID in the SRAP header.

To be more effective, the gNB can configure all the relay UEs in parallel in steps 4-1, 4-2, and 4-3, since each relay UE is configured through the RRC reconfiguration procedure of itself. The configuration latency is uncontrollable, so as the order of mapping configuration for the relay UEs. As a result, when the first relay UE forward the SRAP PDU which carrying the SRB0 message to the intermediate relay UE, the SRB0 mapping configuration may be not available. In this case, if the intermediate relay UE do the error handling, it will discard this SRB0 message from the remote UE, which can cause the connection establishment failure. Therefore, we think for the first SRB0 message from a remote UE, the relay UE should also be forbidden from perform the error handling.

For the SRB1 message forwarding, the similar problem exists in steps 7 to 9. For example, the remote UE will respond the RRCSetupComplete message after receiving the SRB1 mapping configuration and send the SRAP PDU carrying SRB1 message in the UL link. It is possible that the SRB1 mapping configuration is not ready at the relay UE side. As a result, the SRB1 massage will be discard if the relay UE do the error handling in this case. Similarly, the error handling for the first SRB1 message from a remote UE should also be forbidden.

Proposal 7. The relay UE should not do the error handling for the first SRB0 message and the first SRB1 message form the child UE.

# 3	Conclusion

Convert the working assumption to agreement

Proposal 1 – RAN2 confirms the working assumption from RAN2 #130, namely that the Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container.

RRC-15 - Guidance for Remote UE to Select or Reselect a Suitable Relay UE in RRC_CONNECTED to Ensure Low Latency

Proposal 2 – RAN2 confirms that the existing note, stating that the Remote UE may prioritize selecting/reselecting a suitable Relay UE that is in the RRC_CONNECTED state based on RRC state information included in the Discovery Message container, is retained in the running CR.

Authorised Relay UEs remaining in RRC_CONNECTED for longer duration

Proposal 3: Given that the primary use case for multi-hop relay is in emergency or disaster recovery scenarios, it is proposed that certain Relay UEs be authorized by the CN to remain in RRC_CONNECTED for an extended period after initial connection establishment. This approach would enhance their availability to provide low-latency relay services.

Proposal 4: If RAN2 agrees to support such a solution, it is proposed to send an LS to RAN3 and SA2 requesting the necessary authorization mechanisms for such Relay UEs. Furthermore, it is proposed to include an indication in the UE Assistance Information message allowing the network to trigger RRC connection release when no connected UEs are present for a specified duration.

RRC-13: Fine Tuning of Extended Timer Values Covering Both Uu and PC5 Hops

Proposal 5: Extend the T300, T301 and T319 timer for multi-hop U2N relay operation in accordance with the hop count for connection establishment/ re-establishment and resume procedure by either

- Multiplying the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count; or

- Introducing PC5-specific timers (t300-PC5/t301-PC5/t319-PC5) and calculating the timer value as t300 + t300-PC5*n, where n is the number of PC5 hops and one Uu hop is included.

SRAP configuration

Proposal 6: For handling the Multi hop relay the path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

Error handling

Proposal 7. The relay UE should not do the error handling for the first SRB0 message and the first SRB1 message form the child UE.

# 4	Reference

- Session Chair Notes on Relay and Positioning RAN2 #130;