3GPP TSG-RAN WG2 Meeting #131	R2-2505437

Bengaluru, India, 25-29 August, 2025

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon, China Telecom

Title:	Further consideration on Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were made regarding the Store & Forward (S&F) satellite operation [1]:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Based on the current progress in RAN2, this contribution focuses on the remaining issues without conclusions for Store and Forward satellite operation, including paging optimization, access control and the satellite ID issue.

# 2	Discussion

## 2.1	Paging in S&F satellite operation

In the legacy paging procedure, the arrival of MT data triggers the network to page UE. In the S&F scenario, when feeder link is available, the CN sends MT data to satellite, which receives and stores it. When service link is available, the satellite pages UE or the UE initiates RRC connection, at which point the network sends MT data to UE. However, if the satellite does not receive any MT data when feeder link is available, there is no need for the satellite to page UE when service link is available. In this case, the UE doesn’t need to monitor paging or initiate RRC connection.

Therefore, if UE is informed when the NW does not have any MT data buffered for an area, the UE does not need to monitor the paging or initiate RRC connection, which helps to save UE power.

This issue is discussed in the open issue list for 36.304 CR and the following proposal is made by the CR rapporteur:

Proposal 4: Paging monitoring relaxation for cell operating in SF mode without any stored DL data can be considered for Rel-19 based on assessment of required spec changes. Required spec changes are evaluated based on text proposal from interested companies on the above issue

We believe the main require change is to introduce an indication in the system information to indicate whether UE should skip monitoring paging and initiating RRC connection for MT data reception. It is totally up to NW to not include this in case the NW thinks the UE should monitor paging, e.g., due to potential SI change or possible MT data reception.

Proposal 1:An indication is introduced in the System information to indicate that the UE could skip monitoring paging and initiating RRC connection for MT data reception.

## 2.2	UE behaviour related to the Satellite ID list

During RAN2#129 meeting, RAN2 received the reply LS on satellite IDs from SA2 as follows [2]:

- RAN2 understanding: The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

[SA2 response]: SA2 has discussed the scenario and clarified this in the TS 23.401 clause 4.13.9.

Question: If a satellite is included in the satellite list signalled by the MME to a UE, does it mean one of the below options, or possibly other option(s):

- the satellite has the UE context but does not necessarily support store-and-forward operation; or

- the satellite has the UE context and must support store-and-forward operation (but may be operating in normal IoT NTN mode or in store-and-forward mode when the satellite serves the area where the UE is located)?

[SA2 Answer]: SA2 confirms that option 2 is the correct understanding.

According to the LS content and the updated TS 23.401, the following RAN2 understanding is confirmed by SA2: “the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.”

This UE behaviour related to the Satellite ID list is discussed in the open issue list for 36.304 CR and the following proposals are made by the CR rapporteur:

SF Mode parameter in neighour cell and UE Procedure changes Proposal 2: Working assumption on introducing SF mode for neighbour cell is confirmed. No additional specification changes needed. Usage of this parameter is left to UE implementation. NAS Configured List for DC Operation Proposal 3: On the FFS related to consideration of NAS configured sateliite list for DC operation, RAN2 confirms that NAS configured list is not taken into account for UE behaviour related to DC operation.

Firstly, we agree to confirm the working assumption on introducing SF mode for neighbour cell. Then we further discuss the UE behavior as follows:

In order to camp on the satellites that are not included in the NAS satellite ID list, the UE may need to continuously perform measurement for cell selection evaluation after being released to RRC_IDLE. This is to ensure that the UE will not miss any passing satellites. However, in this case, the NAS wait timer becomes ineffective, leading to increased power consumption for the UE. To alleviate this issue, when determine whether to perform measurements, the UE should also take into account the information for the subsequent satellites in SIB32, which was introduced for discontinuous coverage in Rel-17. In other words, UE if the t-ServiceStart for a satellite in SIB32 occurs earlier than the NAS wait timer, UE need to start performing measurement based on the t-ServiceStart to ensure UE can access that satellite.

Proposal 2: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris). The text proposal for capturing this in TS 36.304 is provided in the Annex.

However, if the UE detects that the satellite indicated in SIB32 (but not contained in NAS satellite ID list) is operating in SF mode, it should not access this satellite, as there would be no UE context on this satellite. If there were, the CN would have included this satellite into the NAS satellite ID list.

Proposal 3: If a satellite indicated in SIB32 but not included in the NAS satellite ID list is operating in SF operation, the UE should not access it.

## 2.3	Access control

For both S&F architectures, the UE’s DL NAS data can be stored at the MME on board the satellite serving the UE at next pass. In this case, if the UE is unable to access the intended satellite, the stored data will be lost. Given that the current access control mechanism may lead to rejection of the UE’s RRC connection request due to the load status, it is beneficial for the eNB to prioritize the access of the UE, if it is subsequently re-attempting the access to the NW, to avoid potential data loss. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

Proposal 4: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Paging in S&F satellite operation

Proposal 1:An indication is introduced in the System information to indicate that the UE could skip monitoring paging and initiating RRC connection for MT data reception.

UE behaviour related to the Satellite ID list

Proposal 2: When determining whether to perform measurement in IDLE, the UE should consider both NAS information (i.e., Satellite list and NAS wait timer) and the information in SIB32 (i.e., t-ServiceStart and ephemeris). The text proposal for capturing this in TS 36.304 is provided in the Annex.

Proposal 3: If a satellite indicated in SIB32 but not included in the NAS satellite ID list is operating in SF operation, the UE should not access it.

Access control

Proposal 4: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW. The re-attempt from the UE can be indicated to the network while performing RRC Connection Establishment procedure.

# 4	References

- R2-2504673, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 130.

- S2-2502450, Reply LS on Satellite IDs for store-and-forward operation, SA2.

# 4	Annex – Text Proposal for TS 36.304

## 4.1	Overview

The idle mode tasks can be subdivided into four processes:

-	PLMN selection;

-	Cell selection and reselection;

-	Location registration;

-	Support for manual CSG selection.

The relationship between these processes is illustrated in Figure 4.1-1.

Figure 4.1-1: Overall Idle Mode process

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set TS 23.122 [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

For E-UTRA a cell may be associated with more than one CN type (EPC and/or 5GC) and hence the selected cell can be suitable for more than one CN type. The CN type(s) for which the selected cell is suitable are reported to NAS which selects a CN type to be used for camping and for the NAS registration procedure (see below). Note that CN type selection is only applicabe for UE supporting E-UTRA connected to 5GC.

For E-UTRA a cell may be associated with more than one tracking area. The UE reports all the broadcasted tracking area codes in the selected cell to NAS for registration procedure.

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN TS 23.122 [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. Similar to cell selection procedure, if the reselected cell is an E-UTRA cell and the UE supports E-UTRA connected to 5GC, the CN type(s) for which the cell is suitable are reported to NAS which selects one of them. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed. In RRC_INACTIVE state, if the new cell does not belong to the configured RNA, a RNA update procedure is performed.

If necessary, the UE shall search for higher priority PLMNs at regular time intervals as described in TS 22.011 [4] and search for a suitable cell if another PLMN has been selected by NAS.

Search of available CSGs may be triggered by NAS to support manual CSG selection.

If the UE loses coverage of the registered PLMN, either a new PLMN is selected automatically (automatic mode), or an indication of which PLMNs are available is given to the user, so that a manual selection can be made (manual mode).

Registration is not performed by UEs only capable of services that need no registration.

The UE may perform sidelink communication or V2X sidelink communication or sidelink discovery or NR sidelink communication while in-coverage or out-of-coverage for sidelink, as specified in clause 11.

The purpose of camping on a cell in idle mode is fivefold:

a)	It enables the UE to receive system information from the PLMN.

b)	When registered and if the UE wishes to establish an RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

c)	If the PLMN receives a call for the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNAs (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in this set of tracking areas. The UE will then receive the paging message because it is tuned to the control channel of a cell in one of the registered tracking areas and the UE can respond on that control channel.

d)	It enables the UE to receive ETWS and CMAS notifications.

e)	It enables the UE to receive MBMS services.

If the UE is unable to find a suitable cell to camp on or if the location registration failed (except for LR rejected with cause #12, cause #14, cause #15 or cause #25, see TS 23.122 [5] and TS 24.301 [16]), it attempts to camp on a cell irrespective of the PLMN identity, and enters a "limited service" state.

When NAS indicates that PSM starts, the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while the UE is in PSM it is up to UE implementation whether it performs the corresponding action immediately or the latest when PSM ends. When NAS indicates that PSM ends, the UE shall perform all idle mode tasks.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. In addition, in a Store and Forward (S&F) cell, if the UE determines that it is out of coverage of the satellites listed in the S&F Monitoring List configured by upper layers [23] and those listed in SystemInformationBlockType32, the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.