3GPP TSG-RAN WG2 Meeting #131													     R2-2505442

Bengaluru, India, August 25 – 29, 2025

Agenda item:	8.3.2.3

Source:	Apple

Title:	UE-sided data collection

Document for:	Discussion and Decision

# 1	Introduction

RAN2#130 have made the following agreements on event prediction:

- For UE sided data collection – use AI/ML PHY request/configuration framework as baseline.  FFS enhancements/or differences

In this contribution we provide our additional inputs to progress this discussion.

# 2	Discussion

It should be obvious, but apparently (since we are having this discussion) worth repeating, that UE-sided data collection will be performed:

- By a sufficiently large number of UEs, and

- Over a sufficiently long period of time.

This is of course because, as everybody knows, AI/ML training generally requires large amounts of data. Furthermore, data collection isn’t time critical – it makes little difference whether it takes a week, a month, or a few months to collect enough data. Supervised training (which is what has been evaluated in this study) happens offline, after training is performed there will be a period of testing and eventually deployment – all those takes time. As said, data collection isn’t time critical.

Observation 0: data collection isn’t time sensitive.

Observation 1: data collection can and will be performed over long enough period of time.

Observation 2: data collection can and will be performed by large enough number of UEs.

Naturally, data collected in such manner will cover all the areas, cells and configuration of interest. This is because that if a particular cell or configuration isn’t used for mobility (by large enough number of UEs over large enough period of time), that’s a pretty strong indication that such rarely used cell or an obscure configuration isn’t statistically important.

In other words, the only data which can’t be collected using legacy measurements configured for mobility are outliers, and outliers shall be avoided in training.

Observation 3: if not enough UEs (out of a large number of UEs selected over a large period of time) have been configured with measurements of a particular cell, this is a pretty good indication the cell is not particularly important.

This leads us to the following rather obvious proposal that all we need to allow UE sided data collection – is nothing. UEs will simply use their measurement configurations (configured for mobility) to collect the data of interest, and what’s not configured for mobility – isn’t interesting.

Proposal 1: UE can (and will) perform data collection using measurement configuration(s) it is provided with for mobility; not clear what else is needed.

The second point is equally obvious, but I guess the same reasoning applies here as well. As we all know, there have been very little progress in both AI/ML PHY and SA2 on the topic of UE-sided data collection. Lacking progress in those, which we depend on, it is not at all clear what can we possibly discuss.

Observation 4: little progress has been achieved on UE-sided data collection in AI/ML PHY.

Observation 5: not a whole lot of progress has been achieved in SA2 either.

Observation 6: it’s not clear whether RAN2 can make any progress on this topic, at least not at the moment.

Proposal 2: postpone the discussion on UE-sided data collection till further progress in AI/ML PHY and SA2.

# 3	Proposals

Observation 0: data collection isn’t time sensitive.

Observation 1: data collection can and will be performed over long enough period of time.

Observation 2: data collection can and will be performed by large enough number of UEs.

Observation 3: if not enough UEs (out of a large number of UEs selected over a large period of time) have been configured with measurements of a particular cell, this is a pretty good indication the cell is not particularly important.

Proposal 1: UE can (and will) perform data collection using measurement configuration(s) it is provided with for mobility; not clear what else is needed.

Observation 4: little progress has been achieved on UE-sided data collection in AI/ML PHY.

Observation 5: not a whole lot of progress has been achieved in SA2 either.

Observation 6: it’s not clear whether RAN2 can make any progress on this topic, at least not at the moment.

Proposal 2: postpone the discussion on UE-sided data collection till further progress in AI/ML PHY and SA2.

# 4	References

RAN2#130 meeting minutes