3GPP TSG-RAN WG2 Meeting #131	R2-2505451

# Bengaluru, India, 25-29 August 2025

Agenda item:	8.13.3

Source:	Apple

Title:	Discussion on remaining UP and CP issues for Multi-hop U2N Relay

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

There are a few basic agreements made in the last few RAN2 meetings [1][2][3] regarding the SRAP, RRC and UE capability, but there are some SRAP and RRC signaling design aspects are not sufficiently discussed yet.

In this paper, we discuss the remaining issues of UP and CP issues for multi-hop U2N relaying, including some ASN.1 design issues.

# 2 Discussion

### 2.1 PC5 Relay RLC Channel configuration

In Legacy Rel-17 U2N relay, the PC5 Relay RLC channel configurations are provided as part of “sl-ConfigDedicatedNR”, and the remote UE and relay UE is supposed to configure all PC5 relay RLC channels based on this configuration via PC5-RRC procedure, as indicated in the text below in TS 38.331 [4].

However, there is one extra issue for L2 relay UE, as it may connect to multiple remote UEs when it receives this configuration via Uu RRC. Note that “sl-RLC-ChannelToAddModList” is not provided per destination layer 2 ID, so there are two different understandings of how the relay UE is supposed to configure:

- One particular PC5 Relay RLC channel (e.g., sl-RLC-ChannelID=x) for only a specific remote UE

- Each PC5 Relay RLC channel (e.g., sl-RLC-ChannelID=x) is to be configured for all connected remote UEs.

In the first understanding, the PC5 Relay RLC Channel is to be established over a single PC5 link towards a particular remote UE. This may only be possible if the SRAP-config has limited the usage of this PC5 relay RLC channel x to be mapped/included in a single entry for a particular remote UE in “sl-L2RelayUE-Config”. However, such a dependency  or restriction of SRAP configuration has not been mentioned anywhere in TS 38.331. Given that the procedure text in 5.8.9.7.2 has no text to support the first understanding either, we think the correct behavior for R17 relay UE is based on the second understanding, that is to preform PC5 Relay RLC channel establishment procedure in every PC5 link, per each sl-RLC-ChannelID. For example, if the relay UE has three remote UEs, and there are five valid RLC channel configurations configured by sl-ConfigDedicatedNR , then there will be 3x5=15 PC5 Relay RLC channels established in total by this L2 U2N relay UE. Also, upon any new remote UE establish PC5-RRC connection towards this relay, a new PC5-RRC procedure to establish/configure those 5 PC5 Relay RLC channels will be initiated. Note that this may create some signaling waste, because it is possible not all those 15 channels will be actually used, but there is no other side effects.

Observation 1 	For Rel-17 U2N relay operation, each PC5 Relay RLC channel configured to a L2 Relay will be established to multiple remote UEs connected to this relay UE via PC5-RRC.

In Rel-19, whether the same understanding is supposed to be applicable to Layer 2 intermediate relay UE, RAN2 need to discuss first if such an understanding is still reasonable in Rel-19 multi-hop case. For an intermediate relay UE, it may have N child node and 1 parent node, with total of N+1 PC5 links, so any additional or modification of a single PC5 relay RLC channel configuration will trigger the PC5-RRC procedure in N+1 links. One extra complication introduced in Rel-19 is that the intermediate relay UE need to establish the RLC channel in both directions (with N links in DL direction, and 1 link in UL direction) with the identical PDB parameters in the SL RLC channel configuration. Not sure if this is really desirable for the gNB-controlled QoS design, as it is usually assumed that the UL and DL hop may be configured with different PDB, as upstream link is usually more congested than the downstream link in a tree topology because traffic are aggregated towards the root relay node (last U2N relay UE).

Observation 2 	If Rel-19 inherits Rel-17 legacy, each PC5 Relay RLC channel configured to an intermediate Relay UE will be established in both UL and DL directions via PC5-RRC.

In lieu of this, RAN2 need first discuss whether this basic understanding can be confirmed or not. And if not, then how RRC can implement an alternative understanding of PC5 relay RLC channel configuration for intermediate relay UEs.

Proposal 1 	RAN2 discuss whether each SL-RLC Channel configuration in sl-ConfigDedicatedNR for Intermediate relay UE is to be established in all N+1 PC5-RRC connections (N child node, 1 parent node). If not, what would be the correct UE behaviour in Rel-19.

### 2.2 Reflective Bearer Mapping

In Legacy Rel-17 U2N relay, each relay UE is configured with a SRAP mapping for a local ID with the remote UE L2 ID and the SRAP mapping per bearer information [4]:

SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE {

sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M

...

}

SL-SRAP-Config-r17 ::= SEQUENCE {

sl-LocalIdentity-r17 INTEGER (0..255) OPTIONAL, -- Need M

sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17 OPTIONAL, -- Need N

sl-MappingToReleaseList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17 OPTIONAL, -- Need N

...

}

SL-MappingToAddMod-r17 ::= SEQUENCE {

sl-RemoteUE-RB-Identity-r17 SL-RemoteUE-RB-Identity-r17,

sl-EgressRLC-ChannelUu-r17 Uu-RelayRLC-ChannelID-r17 OPTIONAL, -- Cond L2RelayUE

sl-EgressRLC-ChannelPC5-r17 SL-RLC-ChannelID-r17 OPTIONAL, -- Need N

...

}

From the signaling design above, the Rel-17 Relay only knows the remote UE’s L2 ID in the single-hop relay case. However, in multi-hop relay UE case, the relay UE need to know the next-hop L2 ID which is different from the remote UE’s L2 ID. The current signaling could be redesigned to introduce this L2 ID hierarchy where a direct child and its indirect children’s L2 ID are all provided to NW. That will make the SRAP configuration of RRC signaling rather complex. This can be avoided by support “reflective mapping”, which allows the relay UE to remember the L2 ID of the ingress link for a local ID, then use it for DL egress PC5 link determination. With this method, the SRAP configuration does not need to have a complex change and keep it largely as same as the legacy design. To be more specific, the new IE such as “sl-SRAP-ConfigRelayList “ in the RRC running CR is not needed.

Proposal 2 	Reflective mapping is used for SRAP entity to derive DL egress PC5 link as it does not require the change of legacy SRAP configuration.

To reuse the same exact legacy Rel-17 SRAP configuration, the L2 ID of an indirect child and L2 ID of direct child are both include in the same flat structure. The relay UE will be able to figure out which entry corresponds to a direct child as the L2 ID of a direct child is known to the relay UE when the PC5 link is established.

Proposal 3 	L2 ID of indirect child is provided in “SL-RemoteUE-ToAddMod-r17” to keep the legacy flat structure of SRAP configuration.

If the current hierarchy structure in RRC running CR is to be used, then we still think there is no need to introduce a new “SL-SRAP-configID-r19” , as shown below.

..., [[ sl-SRAP-ConfigRelay-ToAddModList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19 OPTIONAL, -- Need R sl-SRAP-ConfigRelay-ToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19 OPTIONAL -- Need R ]]

Logically, each child configuration is for traffic of an indirect child UE, it can be simply indexed with the local ID or L2 ID, without introduce a new index value.

Proposal 4 	No need to introduce “SL-SRAP-configID-r19”.

### 2.3  Paging Monitoring

As each relay UE is responsible to submit and cancel the SI/Paging request for all its children UE and itself, it makes sense to allow the relay UE to aggregate the requests and send them in one PC5-RRC message. Logically, this can also be done by the relay UE to send multiple PC5-RRC request one-by-one, but this is a wasteful of SL radio resource and create some redundancy. Also, all those requests maybe triggered simultaneously and sending them one by one will add unnecessary latency. For example, when a PC5-RLF occurs, it no longer makes sense to monitor the paging for the child and grandchildren severed by the failed link. In another case, when a relay UE enters RRC_CONNECTED, it need to cancel the paging monitoring for itself and all its children and grandchildren to avoid duplicate paging monitoring. This is not efficient signalling design as only one event will be trigger multiple similar messages in the same link.

Observation 3	It is inefficient to trigger multiple RemoteUEInformationSidelink messages by a single RLF event.

In those cases, it is much better to just send one signalling to cancel the paging monitoring for all those remote UEs. Adding this “aggregated request” has very limited spec impact, but yields much greater flexibility Therefore, we think it is desirable to change ASN.1 to support it.

Proposal 5 	Extend RemoteUEInformationSidelink message to allow aggregation of multiple paging/SI requests together.

It is also worth noting that the paging records forwarded by relay UE can also be aggregated, especially when the relay UE can intercept Uu paging message which contains more than one paging records matching the paging monitoring requests from the path through one child relay. In this case, it is natural to forward all those matching paging records together to this child relay. Therefore, we think similar changes of

Proposal 6 	Allow UuMessageTransferSidleink to contain a list of PagingRecord.

### 2.4  Relay UE capability

In RAN2#130 meeting [3], the following has been agreed:

As a baseline, implement the capability CR with a new capability for Rel-19 multihop relay UE, e.g., relayUE-MH-Operation-L2 with no distinction between last and intermediate cases.  FFS if there is a need to distinguish last and intermediate (based, e.g., on the need for remote UE functionality in the intermediate relay).

There is no strong justification to distinguish U2N relay UE (e.g., between intermediate relay UE and last relay UE). Rel-19 last relay UE and intermediate relay UE should share the same capability. They just happen to be in different locations of the multi-hop relay topology.

Someone may argue the Uu SRAP functions will not be implemented if the UE is intermediate U2N relay, but:

- The Uu SRAP function is quite a minor function to implement, Uu SRAP header shares the same format as PC5 SRAP header. So, if a UE has implemented the PC5 SRAP, it is not a big undertaking to add Uu SRAP functions.

- As a relay UE may move in out of coverage and change its role frequently, which is quite common in public safety use case, the distinction of two types of UE capability with create service outage in public safety use case.

- It is more challenging to deploy relay scenarios practically with three types of devices instead of only two (relay and remote).

Given the above consideration, we think RAN2 need to confirm the baseline assumption that there is no separate UE capability for “intermediate Layer-2 U2N relay UE”.

Proposal 7 	RAN2 confirm that there is no separate UE capability for “intermediate Layer-2 U2N relay UE”.

# 3 Conclusion

In this contribution, we discuss some remaining UP and CP issues and have the following observations.

Observation 1 	For Rel-17 U2N relay operation, each PC5 Relay RLC channel configured to a L2 Relay will be established to multiple remote UEs connected to this relay UE via PC5-RRC.

Observation 2 	If Rel-19 inherits Rel-17 legacy, each PC5 Relay RLC channel configured to an intermediate Relay UE will be established in both UL and DL directions via PC5-RRC.

Observation 3	It is inefficient to trigger multiple RemoteUEInformationSidelink messages by a single RLF event.

Then, we have the following proposals:

Proposal 1 	RAN2 discuss whether each SL-RLC Channel configuration in sl-ConfigDedicatedNR for Intermediate relay UE is to be established in all N+1 PC5-RRC connections (N child node, 1 parent node). If not, what would be the correct UE behaviour in Rel-19.

Proposal 2 	Reflective mapping is used for SRAP entity to derive DL egress PC5 link as it does not require the change of legacy SRAP configuration.

Proposal 3 	L2 ID of indirect child is provided in “SL-RemoteUE-ToAddMod-r17” to keep the legacy flat structure of SRAP configuration.

Proposal 4 	No need to introduce “SL-SRAP-configID-R19”.

Proposal 5 	Extend RemoteUEInformationSidelink message to allow aggregation of multiple paging/SI requests together.

Proposal 6 	Allow UuMessageTransferSidleink to contain a list of PagingRecord.

Proposal 7 	RAN2 confirm that there is no separate UE capability for “intermediate Layer-2 U2N relay UE”.

# 4 References

[1] Chairman’s notes, RAN2#129

[2] Chairman’s notes, RAN2#129bis

[3] Chairman’s notes, RAN2#130

[4] TS 38.331