3GPP TSG-RAN2 Meeting #131	R2-2505458

# Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.7.4 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Remaining issues on scheduling enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

During the MAC running CR discussion [1], following open issue is left for DSR.

[MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported. [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE. [MAC-6] Whether any spec change is needed to capture the agreement on DSR cancellation in the DC case.

During the RLC and PDCP running CR discussion [2-3], following open issue is left for DSR.

RLC-13 Whether the delay-reporting or non-delay-reporting information is visible or not to RLC? PDCP-1 (Editorial) Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

For the UE capabilities [4], following open issue is left for DSR.

(UE capability-01): Whether a UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

In this document, remaining open issues for DSR enhancement are discussed.

2.	Discussion

[MAC-1] DSR cancellation condition

In the current running MAC CR for DSR operation, the DSR triggering and cancelling operation for Multiple Entry DSR is performed based on the RemainingTimeThreshold, as in Single Entry DSR. On the other hand, during the MAC open issue discussion, one company suggested not to cancel the pending DSR even though there is no delay-critical data if there is delay-reporting data above the RemainingTimeThreshold but below the largest reporting threshold. The intention of this suggestion is to allow the transmission of the Multiple Entry DSR MAC CE when there is delay reporting data above the RemainingTimeThreshold .

Meanwhile, in RAN2#128 meeting [5], it was already discussed on the triggering condition and cancelling condition for Multiple Entry DSR, and agreed to reuse the triggering condition and cancelling condition defined in Rel-18 DSR, i.e., based on RemainingTimeThreshold.

The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

In addition, based on the current triggering condition, if there is only delay-reporting data without any delay-critical data, the DSR is not triggered and the DSR MAC CE will not be transmitted until the buffered data becomes the delay-critical data, i.e., remaining time till PDCP discard timer expiry becomes less than RemainingTimeThreshold. Similarly, DSR is not needed as well when there is no delay-critical data which triggers the DSR or when all the delay-critical data has been reported, regardless of whether there is delay-reporting data or not. Therefore, it would be better to have a consistency to triggering and cancelling the DSR using the same remaining time threshold (i.e., RemainingTimeThreshold) and there is no critical need to re-discuss this issue to modify the canceling condition for Multiple Entry DSR at this stage.

Proposal 1. [MAC-1] For Multiple Entry DSR, reuse the existing cancelling condition defined for Single Entry DSR, i.e., no spec change is needed.

[MAC-2] whether to cancel the BSR based on DSR transmission

In addition, in the MAC open discussion [1], it was suggested to cancel the BSR based on the DSR transmission, e.g., cancel the BSR when all the available data is reported by the DSR MAC CE. However, it should be noted that the same discussion was held in Rel-18 XR, when the Single Entry DSR is defined. Specifically, there was long discussion on whether to associate the DSR procedure and existing BSR procedure. As a result, it was agreed to define DSR operation independently from the existing BSR operation, due to complexity on mixing DSR and BSR procedure, e.g., impact on trigger/cancel BSR based on DSR operation, handling the case for LCGs not configured with DSR, etc. Therefore, since it is not very critical issue to reduce the transmission of BSR MAC CE and it is an optimization issue, there is no need to repeat the same discussion in Rel-19 XR.

Proposal 2. [MAC-2] There is no impact on BSR procedure depending on DSR transmission.

[MAC-6] whether to capture the DSR cancellation on DC case

In RAN2#130 meeting [6], it was discussed in which case triggered DSR should be cancelled in DC case. Based on the discussion, it was agreed that DSR should be cancelled when all the PDCP SDUs associated with the DSR are transmitted in any MAC entity after triggering the DSR, since there would be no ‘PDCP SDUs associated with the DSR’ exists at the time of the DSR MAC CE transmission in this MAC entity.

However, similar to discussion in [AT129][014][XR] regarding Rel-18 DSR procedure, there were different views on whether the current text specifies that DSR is cancelled if the other MAC entity transmits all the PDCP SDUs associated with the Rel-19 DSR. Based on different understanding on the current specification, it was clarified as an agreement as follows:

(MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU. (MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

Given that this issue was already discussed in the last meeting, it is slightly preferred to keep the existing agreement, i.e., no further clarification in the spec. However, if any additional text is needed for better clarity, it would be enough to clarify that DSR is cancelled if all the PDCP SDUs associated with the DSR is transmitted in any MAC entity.

Proposal 3. [MAC-6] No further clarification is needed to specify the cancellation of DSR based on whether the PDCP SDUs associated with the DSR are transmitted on the other MAC entity for DC case.

[RLC-13] delay reporting indication from PDCP layer to RLC layer for non-delay-reporting data

In PDCP layer [7], the delay-reporting PDCP data volume associated with i:th reporting threshold is calculated based on the PDCP SDU which is not submitted to RLC layer. For the PDCP data PDUs which have been submitted to the RLC layer, the delay-reporting data volume calculation with i:th reporting threshold is performed in RLC layer, based on the delay-reporting indication provided from the PDCP layer. Here, the delay-reporting indication is provided from PDCP layer to the RLC layer for delay-reporting PDCP SDU associate with i:th reporting threshold which is submitted to the RLC layer.

On the other hand, in RLC layer [8], the delay-reporting RLC data volume associated with i:th reporting threshold is calculated as follows based on the delay-reporting RLC SDU and non-delay-reporting RLC SDU. Here, delay-reporting RLC SDU is defined based on the delay-reporting indication from the PDCP layer, while the non-delay-reporting RLC SDU is determined in RLC layer, based on whether the RLC SDU will be transmitted prior to delay-reporting RLC SDU associated with reporting threshold:

Delay-reporting RLC SDU: a delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]). Non-delay-reporting RLC SDU: a non-delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU that will be transmitted prior to any of the delay-reporting RLC SDU(s) associated with the i:th dsr-ReportingThreshold.

However, based on the current delay-reporting data volume calculation performed in PDCP layer and RLC layer, it may not be accurate when the pdu-SetDiscard and dsr-ReportNonDelayCriticalData are configured. For example, assume that the data is buffered as in Figure 1, and SDU1, SDU2, and SDU3 are submitted to the RLC layer.

Figure 1.

Then, in PDCP layer, delay-reporting PDCP SDUs and non-delay-reporting PDCP SDUs for each reporting threshold are identified as follow:

- For i=1:

  - Delay-reporting PDCP SDU: SDU2, SDU4, SDU7

  - Non-Delay-reporting PDCP SDU: SDU1, SDU3, SDU5, SDU6

- For i=2:

  - Delay-reporting PDCP SDU: SDU3, SDU6, SDU9

  - Non-Delay-reporting PDCP SDU: SDU1, SDU2, SDU4, SDU5, SDU7, SDU8

Delay-reporting data volume in PDCP layer is calculated as follows:

- Delay-reporting data volume associated with 1st reporting threshold = [SDU4 + SDU5 + SDU6 + SDU7]

- Delay-reporting data volume associated with 2nd reporting threshold = [SDU8 + SDU9]

On the other hand, in RLC layer, delay-reporting data indication is received from PDCP layer as follows:

- For SDU2, delay-reporting indication associated with 1st reporting threshold is provided from the PDCP layer. Therefore, from RLC point of view, SDU2 is considered as delay-reporting RLC SDU associated with 1st reporting threshold.

- For SDU3, delay-reporting indication associated with 2nd reporting threshold is provided from the PDCP layer, since it is delay-reporting PDCP SDU associated with 2nd reporting threshold and it is submitted to the RLC layer. Therefore, from RLC point of view, SDU3 is considered as delay-reporting RLC SDU associated with 2nd reporting threshold.

Based on the delay-reporting indication provided by PDCP layer, the RLC layer calculates the delay-reporting RLC data volume associated with each reporting threshold as follows:

- Delay-reporting data volume associated with 1st reporting threshold: [SDU1 + SDU2]

  - Delay-reporting RLC SDU for i=1: SDU2

  - Non-delay-reporting RLC SDU for i=1: SDU1

- Delay-reporting data volume associated with 2nd reporting threshold: [SDU3]

  - Delay-reporting RLC SDU for i=2: SDU3

Here, SDU3 is calculated as delay-reporting data volume associated with 2nd reporting threshold, since the RLC layer does not calculate the data volume based on the buffer status in PDCP layer.

As a result, based on the current PDCP and RLC operation, SDU3 is reported as the delay-reporting data volume associated with 2nd reporting threshold in Multiple entry DSR MAC CE, even though it is actually transmitted prior to the delay-reporting data associated with 1st reporting threshold, i.e., SDU7. Given that dsr-ReportNonDelayCriticalData is defined in order to report required size of uplink grant to transmit the delay-reporting data associated with each reporting threshold, it is not intended behaviour.

Therefore, if dsr-ReportNonDelayCriticalData is configured, it would be better to indicate the non-delay-reporting PDCP SDU associated with each reporting threshold to the RLC layer as well, in order to calculate delay-reporting data properly in RLC layer. In the RLC layer, delay-reporting RLC data volume would be calculated based on the delay-reporting indication for delay-reporting PDCP SDU and non-delay-reporting indication for non-delay-reporting PDCP SDU, which are provided by PDCP layer.

Proposal 4. [RLC-13] If dsr-ReportNonDelayCriticalData is configured, non-delay-reporting PDCP SDU associated with i:th reporting threshold is also indicated to the RLC layer.

If Proposal 4 is agreeable, TPs in Annex A and Annex B should be adopted.

Proposal 5. Adopt the TPs in Annex A and Annex B.

[PDCP-1] Definition of delay-reporting data/non-delay reporting PDCP SDU

During the discussion on PDCP running CR[3], it was remained as an open issue regarding whether to move “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

PDCP-1 (Editorial) Whether the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” needs to be moved to the definition section for both delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

If Proposal 4 is agreed, based on the current definition of the delay-reporting PDCP SDU and non-delay-reporting PDCP SDU, delay-reporting indication or non-delay-reporting indications could be provided to RLC layer multiple times for the same PDCP SDU. This is because current definition does not consider whether it is already considered as delay-reporting PDCP data volume associated with another reporting threshold.

For example, assume that the buffer status as in Figure 1 and SDU1, SDU2, SDU3 are submitted to RLC layer. Then, when SDU3 is submitted to RLC layer, PDCP layer provides two indications for SDU3 to RLC layer as follows:

- For SDU3, non-delay-reporting indications associated with 1st reporting threshold is provided to RLC layer, since SDU3 is non-delay-reporting PDCP SDU associated with 1st reporting threshold.

- For SDU3, delay-reporting indications associated with 2nd reporting threshold is provided to RLC layer, since SDU3 is delay-reporting PDCP SDU associated with 2nd reporting threshold based on the PDU set remaining time of SDU3.

In other words, even though SDU3 is calculated as delay-reporting RLC data volume associated with 1st reporting threshold based on non-delay-reporting indication associated with 1st reporting threshold, delay-reporting indication associated with 2nd reporting threshold is additionally provided, which is not actually needed.

However, even though the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition part of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU, there are still the case that two indications are provided from PDCP layer to RLC layer for the same PDCP SDU, if the corresponding PDCP data PDU has been submitted to the RLC layer.

For example, when the buffer status is as in Figure 1, two indications will be still provided for SDU3 even though the delay-reporting PDCP SDU and non-delay-reporting PDCP SDU excludes the PDCP SDUs which are already considered as delay-reporting PDCP data volume associated with lower reporting thresholds. In detail, the SDU3 is NOT calculated as delay-reporting PDCP data volume since it has been submitted to the RLC layer. Therefore, SDU3 will be considered as non-delay-reporting PDCP SDU associated with 1st reporting threshold and delay-reporting PDCP SDU associated the 2nd reporting threshold.

Therefore, in order to associate with a single reporting threshold for the same PDCP SDU, moving the text “and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i” to the definition is not sufficient and further consideration is needed on whether the delay-reporting indication or non-delay-reporting indication associated with lower reporting threshold is already provided to RLC layer. An example TP to the correctly associate with a single reporting threshold for the same PDCP SDU is as follows:

Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1), and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i and are not indicated to the lower layer as delay-reporting PDCP SDU or non-delay-reporting PDCP SDU associated with any of the k:th dsr-ReportingThreshold where k < i. If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th dsr-ReportingThreshold and larger than or equal to the i-1:th dsr-ReportingThreshold (if i>1) or larger than zero (if i=1), and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i and are not indicated to the lower layer as delay-reporting PDCP SDU or non-delay-reporting PDCP SDU associated with any of the k:th dsr-ReportingThreshold where k < i. Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and that is not a delay-reporting PDCP SDU, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i and are not indicated to the lower layer as delay-reporting PDCP SDU or non-delay-reporting PDCP SDU associated with any of the k:th dsr-ReportingThreshold where k < i.

In addition, if the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU are changed in PDCP running CR, corresponding changes are also needed in the RLC running CR since the RLC also have similar text in delay-reporting RLC data volume calculation.

In our view, if we start to change the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU, the definition will be more complicated since it needs to further consider two aspects, including 1) whether it is calculated as delay-reporting PDCP SDU associated with lower reporting threshold, and 2) whether it is already indicated to RLC layer as delay-reporting PDCP SDU or non-delay-reporting PDCP SDU associated with lower reporting threshold.

Further, if the definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU is changed based on delay-reporting PDCP data volume calculation, there will be circular reference between the definition part and data volume calculation part, which is not desirable. In detail, in delay-reporting PDCP SDU and non-delay-reporting PDCP SDU are defined based on the delay-reporting data volume calculation defined in clause 5.15, while the delay-reporting data volume calculation is performed based on the delay-reporting PDCP SDU and non-delay-reporting PDCP SDU defined in clause 3.1.

Therefore, given that the current data volume calculation is not broken, it would be safer to keep the current text in PDCP running CR and RLC running CR, i.e., keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

Proposal 6. [PDCP-1] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

[UE capability-01] Pre-requisite of multipleEntryDelayStatusReport-r19

In RAN2#129bis meeting [9], it was agreed to define a UE capability in order to indicate the support of Multiple Entry DSR. However, it was remained as an FFS on whether the UE supporting Multiple Entry DSR shall also support Single Entry DSR defined in Rel-18.

An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.

In our understanding, if a UE supports Multiple Entry DSR, it is also natural to support Single Entry DSR as well. In addition, when the network configures reporting threshold(s) for Multiple Entry DSR, the reporting threshold(s) (i.e., dsr-ReportingThreshold) are configured within the Rel-18 DSR configuration (i.e., within LCG-DSR-Config-r18 IE), so the network always configures Rel-18 DSR parameters including RemainingTimeThreshold when Rel-19 DSR is configured.

Therefore, it would be better to support Rel-18 DSR as well if the UE supports Rel-19 DSR.

Proposal 7. [UE capability-01] A UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

3.	Conclusion

In this contribution, it is discussed for the scheduling enhancement for XR. This document includes following proposals.

Proposal 1. [MAC-1] For Multiple Entry DSR, reuse the existing cancelling condition defined for Single Entry DSR, i.e., no spec change is needed.

Proposal 2. [MAC-2] There is no impact on BSR procedure depending on DSR transmission.

Proposal 3. [MAC-6] No further clarification is needed to specify the cancellation of DSR based on whether the PDCP SDUs associated with the DSR are transmitted on the other MAC entity for DC case.

Proposal 4. [RLC-13] If dsr-ReportNonDelayCriticalData is configured, non-delay-reporting PDCP SDU associated with i:th reporting threshold is also indicated to the RLC layer.

Proposal 5. Adopt the TPs in Annex A and Annex B.

Proposal 6. [PDCP-1] Keep the current definition of delay-reporting PDCP SDU and non-delay-reporting PDCP SDU.

Proposal 7. [UE capability-01] A UE supporting multipleEntryDelayStatusReport-r19 shall also indicate support of delayStatusReport-r18.

4.	Reference

[1] R2-2505070 List of open issues in MAC

[2] R2-2505403 Summary of RLC open issue list for R19 XR

[3] R2-2505439 Summary of [POST130][507][XR] PDCP running CR and open issues (LGE)

[4] R2-2505138 Open issues of Rel-19 XR UE capabilities

[5] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024

[6] Report of 3GPP TSG RAN WG2 meeting #130,St Julian's, Malta, 19 - 23 May, 2025

[7] R2-2505438 Introduction of R19 XR enhancements for PDCP spec.

[8] R2-2505402 Introduction of R19 XR enhancements for RLC spec.

[9] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7 - 11 April, 2025

Annex A.	PDCP TP for proposal 4.

(…unrelated part omitted) The transmitting PDCP entity provides a delay-reporting indication or non-delay-reporting indication associated with the i:th dsr-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold.

Annex B.	RLC TP for proposal 4.

An example TP proposal 4 in RLC specification is as follows:

Delay-reporting RLC SDU: a delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]). Non-delay-reporting RLC SDU: a non-delay-reporting RLC SDU associated with the i:th dsr-ReportingThreshold is an RLC SDU corresponding to a PDCP PDU indicated by PDCP as non-delay-reporting associated with the i:th dsr-ReportingThreshold (see TS 38.323 [4]).