3GPP TSG-RAN WG2 Meeting #131	R2-2505474

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.2.2

Source:	Mediatek Inc.

Title:	Discussion on Functionality Management for RRM measurement event prediction

Document for:	Discussion, Decision

Introduction

In the RAN2 #130 meeting, RAN2 uses the beam management agreements as the baseline and made following agreements on inference configuration and applicable reporting for AI mobility.

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

In this contribution, we focus on the applicability procedure, inference configuration/reporting and performance monitoring for the event prediction.

Discussion

Inference Configuration

Full Inference Configuration

In another contribution [2], we propose the related components for the AI mobility full inference configuration, including the Measurement Object (MO), Report Configuration, Measurement Identities, Predicted Object, and Inference Identity. For measurement event prediction, the predicted output is the event occurrence, so the predicted object is no longer needed. The inference identity is used to identify the inference configuration and provides the mapping of a measurement object, a report configuration.

For the UE side model, the full inference configuration of event prediction should contain the following:

- Measurement object: Describes the information of measurement/observed target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report. The measurement report should include the predicted event and real measurement results.

- Inference identity: Refers to a MeasId, identifying the inference configuration and providing the mapping of a measurement object and a report configuration.

For the full inference configuration of event prediction, we think that the existing RRM measurement configuration consisting of MeasObject/MeasId/ReportConfig can be extended to above full inference configuration to support the use cases of AI for mobility, including the event prediction.

Proposal 1: The existing RRM measurement configuration can be reused and extended with the inference identity element to support event prediction, as detailed below:

- Measurement object: Describes the information of measurement/observed target.

- Report configuration: Describes the measurement event to be predicted, with triggering condition and report quantity for the measurement report. The measurement report should include the predicted event and real measurement results.

- Inference identity: Refers to a MeasId, identifying the inference configuration and providing the mapping of a measurement object and a report configuration.

Inference Parameters

The NW may configure full inference configurations (Option A), or only inference parameters (Option B) in Step 3. For the UE-side model event prediction, regardless of whether considering direct or indirect prediction, or temporal case A or temporal case B, the AI model-related configuration includes OW and PW. The implicit configuration is that PW is related to TTT. Thus, the network needs to indicate the predicted event id to the UE. The RAN configuration (e.g., EventId/TTT/offset...) may change with the HO procedure, so the inference parameters should include the RAN configuration (e.g., EventId/TTT/offset...) to allow model update. RAN2 has agreed that the "inference parameters" can be considered for inference/measurement configuration, which provides the necessary information for the UE to examine the applicability. It may contain the PW length (for temporal case A) and the skipping pattern, MRRT (for temporal case B).

Proposal 2: In addition to the agreed parameters at last meeting, RAN2 consider following “inference parameters” for event prediction, which provides the necessary information for UE to examine the applicability, it may contain：

- Measured EventId

- Event related information (Threshold/Hysteresis/offset/TTT)

HO Modelling Options

Given that RAN2 has agreed on three different HO modeling options for event prediction, each modeling option may have a different impact on the reporting procedure when event prediction is incorporated.

The HO modelling option 1, which requires the UE to report both the legacy measurement report and an additional prediction measurement report, would indeed have significant impacts on the current specifications as follows:

- The existing measurement reporting framework would need to be extended to accommodate the new prediction measurement reports.

- Reporting both real and predicted measurements report would increase the signaling overhead on the air interface.

For HO modeling options 2 and 3, where the UE directly reports the prediction measurement report and the network treats this predicted report as if it is a legacy measurement report, the impact on the specifications would be quite limited.

In addition, during the evaluation stage, the performance of HO modeling option 1 showed only very limited gain.

Observation 1: The HO modelling option 1 has very limited performance gain, with significant impacts on the current specifications.

Proposal 3: To avoid changing the current HO behavior when introducing measurement event prediction, RAN2 adopts HO modeling options 2 and 3 as the basis for specifying the handover procedure in the normative work.

Condition When to Perform Inference

For the UE-side model, the model training is up to UE implementation, such as the training data and label are constructed in a continuous manner or only when the condition is satisfied. Therefore, the condition of when to perform inference is also up to UE implementation. The inference timing for event predictions can occur at several ways:

- Continuous Real-Time Inference: AI continuously measures incoming measurement data and performs inference to predict potential events. This method leads to high computational load and energy consumption on the UE, which will impact battery life, as well as increased processing and signaling overhead on the network.

- Event-Driven Inference: The UE performs inference when certain condition is fulfilled (e.g., the A3 event entering condition is fulfilled).

Based on the above analyse, we have the following proposal:

Proposal 4: For the measurement event prediction, the UE should perform inference when the entering condition of the predicted event is fulfilled.

Inference Reporting

At #129bis meeting, RAN2 has agreed following for event prediction:

- For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

- Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

For the UE-side model of event prediction, the report configuration should include the triggering condition and report quantity for the measurement report. The measurement report can be generated based on both real measurements and/or inference results.

Figure 1 is RAN2's illustration of HO modeling option 2. Based on above discussion, we assume the UE performs inference when the event entering condition is fulfilled. With such assumption, at current time i.e. t0 measurement event e.g. A3 event is predicted at future time t1. And at time t0, the entry condition of the predicted measurement event is already met based on actual measurement result at t0, so network transmits handover command immediately after handover preparation phase. In this way, the HO modeling option 2 and option 3 has same procedure as the legacy HO, which once a predicted measurement event e.g. A3 event is received network can transmit handover command immediately after handover preparation is completed. UE will report predicted measurement event at the time instance it is predicted to be triggered.

Figure 1: Handover model option 2

In this case, with the UE performs inference when the entering entry condition is met, the inference report is included to the measurement report (MR) and include the real measurement result of the current measurement objective whose entering condition is fulfilled, allowing the network to make the handover (HO) decision. In addition, the prediction results can be optionally sent to NW as additional information.

Proposal 5: For measurement event prediction, the measurement report should include real measurement results and optionally the prediction results.

Performance Monitoring

For performance monitoring on UE side model, RAN2 made following agreements at last meeting,

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

For indirect event prediction, if we want to use intermediate KPIs such as RSRP difference or F1 score for performance monitoring, the issue arises in case B, i.e., the UE cannot obtain the reduced measurement instances required to calculate these KPIs.

Observation 2: For indirect event prediction case B, the UE cannot obtain the reduced measurement instances required to calculate these KPIs.

If RSRP differences is used as a KPI for performance monitoring, for indirect event prediction case B, the UE needs to keep the real measurements in PW to evaluate the RSRP difference. For indirect event prediction case A, without the RS/measurement reduction, the real measurement is preserved naturally.

Proposal 6: For indirect event prediction, RSRP differences can be used as the performance metric for monitoring.

For direct event prediction, whether the intermediate KPIs F1 score or system KPIs such as HOF rate, total number of HOF per UE per second, and total number of HO attempts per UE per second, all the KPIs that RAN2 has agreed need a long-term measurement/monitoring.

Meanwhile, just as a person cannot step into two rivers at the same time, similarly, the UE cannot simultaneously execute two handovers (HOs) based on both predicted events and actual measured events. Therefore, it is impossible to compare system KPIs between the inference output and monitoring data at the same time.

Observation 3: For direct event prediction, the UE cannot get the comparison between the inference output and monitoring data of system KPIs at the same time due to the UE cannot simultaneously execute two handovers (HO) based on both predicted events and actual measured events.

To solve this dilemma, we propose following methods to enable the UE to obtain the event prediction accuracy or ground-truth/monitoring data of the measurement object.

Direct event prediction performance monitoring option 1: Performance Monitoring Based on Real Measurements.

With option 1, for performance monitoring, the UE chooses one prediction occasion from several prediction occasions and performs HO based on real measurement results rather than AI prediction. For the PW, the UE perform both measurement and prediction. However, the AI prediction in this prediction occasion is not used to trigger any HO procedures; it is only compared with the actual measurement results. By comparing the predicted event occurrence with the real event timing, we can assess how accurately the AI model predicts the event. This performance monitoring does not need to be done frequently, which helps to minimize the processing and measurement burden on the UE.

Direct event prediction performance monitoring option 2: Performance Monitoring Based on Post-Handover performance.

With option 2, the UE makes handover decisions based on AI-predicted results. After the handover, the UE evaluates the outcome: if the handover based on prediction leads to issues such as HOF, RLF, or the UE quickly switching back to the original cell within a short period, it indicates that the AI prediction was incorrect/inappropriate. This method allows us to judge the appropriateness of the AI prediction based on what happens after the handover.

Based on these methods, for direct event prediction performance monitoring we have the proposal:

Proposal 7: For direct prediction, the performance monitoring can consider following options in the WI and other options are not excluded.

- Option 1: Performance Monitoring Based on Real Measurements. The UE chooses one prediction occasion from several prediction occasions and performs HO based on real measurement results rather than AI prediction. For the PW, the UE perform both measurement and prediction. The AI prediction is not used to trigger any HO procedures. By comparing the predicted event occurrence with the real event timing, we can assess how accurately the AI model predicts the event. Performance monitoring is performed at intervals. FFS periodically or event triggered.

- Option 2: Performance Monitoring Based on Post-Handover performance. The UE makes handover decisions based on AI-predicted results. After the handover, the UE evaluates the outcome performance: if the handover based on prediction leads to issues such as HOF, RLF, or the UE quickly switching back to the original cell within a short period, the AI prediction is considered incorrect/inappropriate.

Conclusion

HO Modelling Options

Observation 1: The HO modelling option 1 has very limited performance gain, with significant impacts on the current specifications.

Performance Monitoring

Observation 2: For indirect event prediction case B, the UE cannot obtain the reduced measurement instances required to calculate these KPIs.

Observation 3: For direct event prediction, the UE cannot get the comparison between the inference output and monitoring data of system KPIs at the same time due to the UE cannot simultaneously execute two handovers (HO) based on both predicted events and actual measured events.

Inference Configuration

Proposal 1: The existing RRM measurement configuration can be reused and extended with the inference identity element to support event prediction, as detailed below:

- Measurement object: Describes the information of measurement/observed target.

- Report configuration: Describes the measurement event to be predicted, with triggering condition and report quantity for the measurement report. The measurement report should include the predicted event and real measurement results.

- Inference identity: Refers to a MeasId, identifying the inference configuration and providing the mapping of a measurement object and a report configuration.

Proposal 2: In addition to the agreed parameters at last meeting, RAN2 consider following “inference parameters” for event prediction, which provides the necessary information for UE to examine the applicability, it may contain：

- Measured EventId

- Event related information (Threshold/Hysteresis/offset/TTT)

HO Modelling Options

Proposal 3: To avoid changing the current HO behavior when introducing measurement event prediction, RAN2 adopts HO modeling options 2 and 3 as the basis for specifying the handover procedure in the normative work.

Condition When to Perform Inference

Proposal 4: For the measurement event prediction, the UE should perform inference when the entering condition of the predicted event is fulfilled.

Inference Reporting

Proposal 5: For measurement event prediction, the measurement report should include real measurement results and optionally the prediction results.

Performance Monitoring

Proposal 6: For indirect event prediction, RSRP differences can be used as the performance metric for monitoring.

Proposal 7: For direct prediction, the performance monitoring can consider following options in the WI and other options are not excluded.

- Option 1: Performance Monitoring Based on Real Measurements. The UE chooses one prediction occasion from several prediction occasions and performs HO based on real measurement results rather than AI prediction. For the PW, the UE perform both measurement and prediction. The AI prediction is not used to trigger any HO procedures. By comparing the predicted event occurrence with the real event timing, we can assess how accurately the AI model predicts the event. Performance monitoring is performed at intervals. FFS periodically or event triggered.

- Option 2: Performance Monitoring Based on Post-Handover performance. The UE makes handover decisions based on AI-predicted results. After the handover, the UE evaluates the outcome performance: if the handover based on prediction leads to issues such as HOF, RLF, or the UE quickly switching back to the original cell within a short period, the AI prediction is considered incorrect/inappropriate.

Reference

[1] 	R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model, RAN1

[2]	R2-2503782_Discussion on Functionality Management for AI Mobility, MediaTek

[3]	TR 38.744