3GPP TSG-RAN WG2 Meeting #131       	                      R2-2505512

Bangalore, India, 25th-29th Aug 2025

Agenda item:	8.3.2.2

Source:	CMCC

Title:	Discussion on functionality management for measurement event prediction

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 had the following agreements on functionality management for UE-sided model:

Agreements on applicability reporting - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). Agreements on inference - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them. Agreements on performance monitoring 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. 3 Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - 4 For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

This contribution will discuss the applicability reporting procedure, inference configuration/reporting and performance monitoring for UE sided model for RRM measurement event predictions.

- Discussion

- 2.1 Applicability reporting

During the discussion on beam management in AI/ML PHY, RAN2 has the following agreements on applicability reporting in last meeting:

Agreements - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)

The above agreements can be reused to AI Mob.

Proposal 1: Reuse the following agreements on beam management in AI/ML PHY to AI Mob:

- - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B

- 2.2 Inference

For indirect measurement event prediction, the model input and output are same as RRM measurement prediction. Therefore, the potential “inference parameters” considered for RRM measurement prediction are also applied for indirect measurement event prediction.

Figure 1. Example of indirect measurement event prediction

Proposal 2: The potential “inference parameters” considered for RRM measurement prediction are also applied for indirect measurement event prediction.

For direct measurement event prediction, we believe that the measurement event configuration (e.g. threshold, TTT) can be considered as inference parameter since the output of the model is the occurrence probability of the measurement event. In addition, the measurement results can be also considered as inference parameter.

Figure 2. Example of direct measurement event prediction

Proposal 3: For direct measurement event prediction, the measurement event configuration (e.g. threshold, TTT) and measurement results can be considered as potential “inference parameter”.

- 2.3 Performance monitoring

For measurement event prediction, RAN2 studied direct prediction and indirect prediction. During the simulation phase, F1 score (for indirect and direct) and time difference between true event and predicted event reporting (for indirect) are used as performance metrics.

RAN2#127bis Agreements - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection.

In last meeting, RAN2 agreed that RSRP differences can be used as the performance metric for monitoring for RRM measurement prediction. For indirect prediction, the RSRP differences can be also applied, i.e. the UE reports actual measurement results and the network calculates the RSRP differences. Furthermore, for the F1 score or time difference, the UE can report missed event detection, false event detection, the true event time to the network for calculating the metrics. For direct prediction, missed event detection and false event detection can be reported to the network for monitoring.

Proposal 4: For NW-side performance monitoring, the UE can report the following information to the network:

- For indirect prediction: actual measurement results, missed event detection, false event detection, true event time

- For direct prediction: missed event detection, false event detection

During the previous meeting discussion, some companies proposed that the calculation of F1 score may be complicated for the UE. In fact, there may not be enough available truth-label for a single UE to calculate the performance metrics in a single cell, for example, the network can handover the UE to target cell once a single measurement event is satisfied. However, the network can obtain multiple truth-labels from different UEs and calculate a valuable F1 score.

Observation 1: It may be difficult for a single UE to calculate F1 score because there may not be enough available truth-label in a single cell.

Therefore, for UE-side performance monitoring, we think the information reported by the UE is similar to which for NW-side performance monitoring. Specifically, for indirect prediction, the UE can report RSRP differences, missed event detection, and false event detection. Furthermore, for time difference between actual event time and predicted event time for indirect prediction, we believe that it is easy for the UE to calculate and report to the network. On the other hand, for direct prediction, the UE can report missed event detection and false event detection to the network.

Proposal 5: For UE-side performance monitoring, the UE can report the following information to the network:

- For indirect prediction: RSRP differences, missed event detection, false event detection, time difference between true event time and predicted event time

- For direct prediction: missed event detection, false event detection

# 3	Conclusion

Here are the proposals on functionality management for measurement event prediction.

Applicability reporting

Proposal 1: Reuse the following agreements on beam management in AI/ML PHY to AI Mob:

- - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration

- Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled.  Assume this applies to Option A and B

Inference

Proposal 2: The potential “inference parameters” considered for RRM measurement prediction are also applied for indirect measurement event prediction.

Proposal 3: For direct measurement event prediction, the measurement event configuration (e.g. threshold, TTT) and measurement results can be considered as potential “inference parameter”.

Performance monitoring

Proposal 4: For NW-side performance monitoring, the UE can report the following information to the network:

- For indirect prediction: actual measurement results, missed event detection, false event detection, true event time

- For direct prediction: missed event detection, false event detection

Observation 1: It may be difficult for a single UE to calculate F1 score because there may not be enough available truth-label in a single cell.

Proposal 5: For UE-side performance monitoring, the UE can report the following information to the network:

- For indirect prediction: RSRP differences, missed event detection, false event detection, time difference between true event time and predicted event time

- For direct prediction: missed event detection, false event detection

# 4	Reference

- Draft_RAN2_130_Meeting_Report_v3