3GPP TSG-RAN WG2 Meeting #131	R2-2505517

Bengaluru, India, August. 25th – 29th, 2025

Agenda Item:	8.6.2

Source: 	OPPO

Title: Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our considerations on the open issues related to inter-CU LTM.

Discussion

MAC-20

Open issue MAC-20 is concerning how to design the MAC CE to activate/deactivate the SP CSI-RS resources for LTM CSI acquisition for candidate cells, i.e., whether the same MAC CE is used for both RRM measurement and early CSI acquisition purpose.

Open issue MAC-20 (essential): How to design the MAC CE to activate/deactivate the SP CSI-RS resource for LTM CSI acquisition for candiate cells based on RAN1 LS

Based on RAN1 inputs, the NZP-CSI-RS resource configuration of candidate cells can be used for both beam management and CSI acquisition. In current RRC running CR, the CSI-RS/CSI-IM resource sets are configured in the same LTM-CSI-ResourceConfig IE, thus the same MAC CE can be also used for SP CSI-RS resources activation/deactivation

The same MAC CE is used for SP CSI-RS activation/deactivation for both RRM measurement and early CSI acquisition purposes.

MAC-25

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-25 | Handling of the activated SP CSI-RS for target cell after LTM cell switch Editor’s NOTE: After reconfiguration with sync that is triggered by LTM, it is FFS how to handle the configured Semi-persistent CSI-RS/CSI-IM resource sets for target cell. Based on the following agreement: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell | Issue Type: Essential How to address it: based on companies’ contribution |

Further discussion is needed on how to handle the activated SP CSI-RS resource of target cell after cell switch since it may have impacts on target cell CSI acquisition. As agreed by RAN1, the measurements for CSI acquisition can be performed as follows.

- After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability.

- After the reception of the cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration.

Based on the second bullet, the UE may still need to measure the CSI-RS resources for CSI acquisition after the reception of CSC. Therefore, the SP CSI-RS set(s) associated with the ltm-CSI-ReportConfig in LTM-Candidate IE of the target cell should be kept as activated after cell switch. The deactivation of the resource can rely on NW indication, i.e., SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for Candidate Cell MAC CE.

The SP CSI-RS resources associated with the ltm-CSI-ReportConfig in ltm-Candidate IE of the target cell keeps activated after cell switch.

After cell switch completion, it is up to NW to deactivate the SP CSI-RS resources which are used for target cell early CSI acquisition.

MAC-26

As one of the open MAC issues, further discussion is needed on how to perform the power ramping during the RACH in case that the candidate/target cell is configured with 2TAGs.

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-26 | How to consider the TAG ID when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1 for an (C)LTM candidate cell which is configured with two TAGs in RACH-less (C)LTM_ Editor’s NOTE: In RACH-less (C)LTM, for an (C)LTM candidate cell which is configured with two TAGs, TAG ID is not considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1. Rapporteur realized the same issue exists for Rel-18 LTM, which was missing. | Issue Type: Essential How to address it: based on companies’ contribution |

There are two cases when the candidate/target cell is configured with mTRP 2TAGs:

Case1: Inter-cell mTRP 2TAGs. In this case, the UE can only perform early UL sync or LTM execution to the TRP with the same PCI as the candidate/target cell, i.e., RACH to the TRP associated to additional PCI is not allowed. So the issue is not valid in this case since only one TAG will be involved in the RACH procedure during early UL sync or LTM execution.

Case2: Intra-cell mTRP 2TAGs. In this case, it is possible there is TAG change during the RACH procedure, e.g., the SSBs selected for different RACH attempts are associate to different TRPs. In R18 MIMO, there is no conclusion whether separate power ramping is required in this case, i.e., the TAG ID is not considered when determining whether to perform power ramping in the same RACH procedure. The principle of MIMO can be simply followed in LTM. In addition, for early UL sync triggered by the NW, the NW can include new transmission indication in the PDCCH order if different power ramping is necessary.

Based on the considerations above, it is proposed that in case that the candidate cell is configured with 2TAGs, the TAG ID is not considered when performing power ramping during the RACH either for early UL sync or LTM execution.

In case that the candidate cell is configured with 2TAGs, the TAG ID is not considered when performing power ramping during the RACH either for early UL sync or LTM execution.

L2 handling during LTM execution

In current RRC running CR, how to handle the MN terminated DRB(s) during inter-CU SCG LTM is specified as follows.

In above text, the UE needs to perform data recovery for all AM DRBs using master key in case of inter-CU SCG LTM. While according to TS38.323, data recovery is required when there is RLC entities release or re-establishment. To align with legacy L2 handling principle, it is proposed that during inter-CU LTM execution, PDCP data recovery is only performed for AM DRBs using master key in case that its associated RLC entities need to be released or re-established due to the triggered LTM.

During inter-CU SCG LTM execution, data recovery is only performed for AM DRB using master key in case that its associated RLC entities need to be released and/or re-established due to the triggered LTM.

Conclusion

Based on the discussion in section2, we have following proposals:

Proposal 1	The same MAC CE is used for SP CSI-RS activation/deactivation for both RRM measurement and early CSI acquisition purposes.

Proposal 2	The SP CSI-RS resources associated with the ltm-CSI-ReportConfig in ltm-Candidate IE of the target cell keeps activated after cell switch.

Proposal 3	After cell switch completion, it is up to NW to deactivate the SP CSI-RS resources which are used for target cell early CSI acquisition.

Proposal 4	In case that the candidate cell is configured with 2TAGs, the TAG ID is not considered when performing power ramping during the RACH either for early UL sync or LTM execution.

Proposal 5	During inter-CU SCG LTM execution, data recovery is only performed for AM DRB using master key in case that its associated RLC entities need to be released and/or re-established due to the triggered LTM.