3GPP TSG-RAN WG2 Meeting #131	                                    R2-2505519

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Open issues for conditional LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the open issues for supporting conditional LTM.

Discussion

MAC-30

| MAC-30 | If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer. [Rapp]: New added based on OPPO’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| --- | --- | --- |

In last meeting, the following agreement regarding the CLTM fallback is agreed:

During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM

One issue related to the agreement is how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer if RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG TAT timer expiration.

Upon RACH-less CLTM cell switch execution, UE select a configured uplink grant for initial uplink transmission, and the HARQ process ID (HARQ process#x) is calculated based on the equation defined in MAC spec. If the PTAG TAT timer expires after initial uplink transmission, UE initiates RACH procedure and considers the RACH-based CLTM cell switch to be ongoing.

In legacy RACH procedure, the MAC entity obtains the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3/MsgA buffer. And HARQ process#0 is used for transmission of the MAC PDU in the Msg A buffer and the Msg 3 buffer.

Upon fallback to RACH-based CLTM, the MAC entity may not be able to obtain the MAC PDU (initial UL transmission) from the multiplexing and assembly entity, since the MAC PDU (initial UL transmission) to be transmitted has been obtained from the multiplexing and assembly entity to the HARQ buffer of HARQ process#x during CG based initial uplink transmission of the RACH-less CLTM execution. If the RRCReconfigruaitonComplete message transmission is postponed, i.e., retx after the competition of the RACH procedure for RRCReconfigruaitonComplete initial transmission. The condition for LTM cell switch completion will be ambiguous since RACH completion does not guarantee the success transmission of RRCReconfigruaitonComplete. And the interruption time of LTM cell switch is also increased due to the postponed transmission of RRCReconfigruaitonComplete.

To solve the issue, the following solutions may be performed:

- Option1: MAC entity obtains the MAC PDU (RRCreconfigurationcomplete message) from the HARQ buffer of HARQ process#x and indicate to the Multiplexing and assembly entity to include MAC subPDU carrying MAC SDU from the obtained MAC PDU. Then the MAC entity could obtain the MAC PDU from the Multiplexing and assembly entity and store it in the Msg3/MsgA buffer for transmission.

- Option 2: The MAC entity could obtain the MAC PDU from upper layer, e.g., RRC re-generate the RRCreconfigurationcomplete message and deliver to MAC entity.

We think option1 is preferred since option1 is simpler and can avoid the complexity of inter-layer interaction.

During the RACH-based CLTM which fallback from RACH-less CLTM, the MAC entity performs the following steps when generating Msg3/MsgA payload:

- obtain the MAC PDU (initial uplink transmission) from the HARQ buffer corresponding to CG based initial uplink transmission

- indicate to the Multiplexing and assembly entity to include MAC subPDU carrying MAC SDU from the obtained MAC PDU

- obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the MsgA/ Msg3 buffer.

MAC-29

| MAC-29 | When CLTM candidate configuration is reconfigured/released, whether/how to handle the stored TA value and the running TAT for the corresponding candidate cell. [Rapp]: New added based on Offinno’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| --- | --- | --- |

For supporting early sync for CLTM, ltm-Candidate-TimeAlignmentTimer is introduced for TA maintenance of CLTM candidate. And ltm-Candidate-TimeAlignmentTimer for target cell and other candidate is not stopped upon C-LTM cell switch execution.

It is observed the stop condition for ltm-Candidate-TimeAlignmentTimer has not been defined, especially for the case that the corresponding candidate configuration has been released and added again. For example, UE performs early sync to CLTM candidate#3 and start ltm-Candidate-TimeAlignmentTimer for CLTM candidate#3 upon reception the LTM candidate TA command MAC CE. Later NW releases the candidate config of CLTM candidate#3 and adds a new CLTM candidate#3 configuration. If ltm-Candidate-TimeAlignmentTimer for old CLTM candidate#3 is not stopped, UE may assume the TA of new CLTM candidate#3 is available and perform RACH-less CLTM execution if satisfied. Then unnecessary CLTM failure may be caused due to applied wrong TA.

The ltm-Candidate-TimeAlignmentTimer is stopped if corresponding LTM candidate config is released.

MAC-14

| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

In last meeting, it has agreed that NW can send an LTM cell switch command MAC CE indicating a CLTM candidate configuration. And no spec change is expected for supporting this agreement.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

As agreed in last meeting, CLTM candidate TAT timer is not stopped upon CLTM execution, and UE starts the PTAG TAT timer using remaining time.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

Based on the agreements above, there are two ways for CLTM TAT maintenance:

- Option 1: follow CLTM way

CLTM TAT for target cell is not stopped upon LTM cell switch command reception if it is running and starts the PTAG using the remaining time from the C-LTM TAT timer. In this case, the spec change is needed to define CLTM TAT timer handling for the NW triggered LTM cell switch.

- Option 2: follow LTM way

CLTM TAT for target cell is not stopped upon LTM cell switch command reception if it is running and starts the PTAG TAT timer of the target cell. And in this case, NW should always include TA in the LTM cell switch command even the CLTM TAT timer is running, which will not cause any spec change.

To align with the agreement in last meeting, i.e., no specification change is expected, we prefer to go option 2.

Upon reception of LTM cell switch command MAC CE indicating a CLTM candidate configuration index, UE only performs RACH-less LTM execution if TA command is included in LTM cell switch command or UE has successfully measured the TA, regardless of CLTM TAT is running or not. (Same as R18 and no spec change is needed)

Execution condition evaluation

Based on current TS38.300, if any HO command/ LTM cell switch command is received before any CLTM execution condition is satisfied, UE shall execute the received HO command/LTM cell switch command, regardless of any previously received CLTM configuration.

Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration) or LTM cell switch command MAC CE, the UE executes the HO procedure as described in clause 9.2.3.2 or LTM cell switch procedure as described in clause 9.2.3.5, regardless of any previously received CHO configuration.

For CLTM, same principle should also be followed. Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Coexistence with L3 HO

The issue on coexistence between different types of HO mechanism should also be studied. As shown below, Rel-18 LTM support the coexistence with all other L3 HO mechanisms except DAPS.

| Coexistence(Y/N) | Normal HO | DAPS | CHO | CPAC | SCPAC |
| --- | --- | --- | --- | --- | --- |
| LTM | Y | N | Y | Y | Y |

For CLTM, we can follow similar agreements made in Rel-18, CLTM can be coexisted with other L3 HO mechanism.

CLTM and L3 HO coexistence is supported.

CLTM and DAPS coexistence is not supported.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	During the RACH-based CLTM which fallback from RACH-less CLTM, the MAC entity performs the following steps when generating Msg3/MsgA payload:

a)	obtain the MAC PDU (initial uplink transmission) from the HARQ buffer corresponding to CG based initial uplink transmission

b)	indicate to the Multiplexing and assembly entity to include MAC subPDU carrying MAC SDU from the obtained MAC PDU

c)	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the MsgA/ Msg3 buffer.

Proposal 2	The ltm-Candidate-TimeAlignmentTimer is stopped if corresponding LTM candidate config is released.

Proposal 3	Upon reception of LTM cell switch command MAC CE indicating a CLTM candidate configuration index, UE only performs RACH-less LTM execution if TA command is included in LTM cell switch command or UE has successfully measured the TA, regardless of CLTM TAT is running or not. (Same as R18 and no spec change is needed)

Proposal 4	Before any CLTM execution condition is satisfied, upon reception of HO command or LTM cell switch command MAC CE, the UE executes the HO procedure or LTM cell switch procedure, regardless of any previously received CLTM configuration.

Proposal 5	CLTM and L3 HO coexistence is supported.

Proposal 6	CLTM and DAPS coexistence is not supported.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx