3GPP TSG-RAN WG2 Meeting #131	R2-2505522

Bengaluru, India, Aug 25th –29th, 2025

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT paging

Document for: Discussion and Decision

# 1	Introduction

This contribution discusses security-related field in A-IoT Paging, how to define the Paging ID length and the unit of paging ID length, how to generate transaction ID and the size of transaction ID, and the paging ID visibility to MAC.

# 2	Discussion

## 2.1	Security-related field in A-IoT Paging

### 2.1.1	RAN LS

SA3 sent security-related LS to TSG RAN and RAN1:

S3-252335 (LS from SA3 to RAN1 and RAN): SA3 therefore seeks any other information besides 1uw peak power consumption clarification on the following: Expected operational patterns (e.g., duty cycles, active time slots) of Device Type 1 and the required portion for non-security functions Power and energy budgets and constraints during operations. Any assumptions RAN1 are making regarding security feature support for this device category in terms of power and energy consumption.

RAN sent a reply in RAN#108:

RP-251886 (LS reply from RAN to SA3 and RAN1): TSG-RAN advises SA3 that this is the extent of information that can be provided to SA3 from TSG RAN and RAN WGs. RAN expects that completion of the Rel-19 WI in RAN and SA WGs will be as planned.

Observation 1:	From RAN LS: “RAN expects that completion of the Rel-19 WI in RAN and SA WGs will be as planned.”

### 2.1.2	SA3 LS

In SA3#122 meeting, LS on security parameter in a paging message was sent from SA3:

S3-252392 (LS from SA3 to RAN2): SA3 is currently working on the authentication procedure. For this procedure, SA3 is assuming the ability to include an additional parameter, a 128bit random number in the paging request message. There may be other security parameters required. Therefore, SA3 would like to request RAN2 to confirm the feasibility of including additional security parameters and their maximum size.

The available bits for a security parameter depends on the maximum bit size of other fields in A-IoT Paging and the maximum TB size of A-IoT Paging message. The maximum bit size of other fields depends on the final A-IoT MAC spec. In the current running CR for A-IoT MAC spec from Email discussions, we can obtain a rough size of an A-IoT Paging message, which helps to determine how this 128-bit security parameter can fit.

A rough maximum size of the fields which are agreed or under discussion in A-IoT Paging is around 300 bits:

- According to the current running CR for A-IoT MAC spec, the size of AS fields is not larger than 53 bits:

  - with agreed sizes of fields: 1-bit "RA type", 1-bit ''Paging ID Presence Indication", and 4-bit "Number of Access Occasions"; and

  - with assumption of 3-bit "Message Type", 1-bit "R field", 8-bit "Paging ID Length", 3-bit "Transaction ID", 7-bit "Fill bits", and 25-bit "D2R Scheduling Info".

- From the CT4 CR (C4-252464), the size of Paging ID field is not larger than 256 bits.

C4-252464 Figure YY.2-1: Structure of AIoT Device Permanent Identifier Figure aa.3.1: Structure of Filtering Information The Filtering Information is composed of the following parts, with a maximum length of 256 bits:

According to the RAN TR, the maximum TB size is around 1000 bits:

TR 38.769: It is studied on the functionality of segmentation. It is understood that segmentation and reassembly would add complexity. Therefore, the simplified segmentation in the A-IoT MAC layer is studied, considering following studies in physical layer: - In both R2D and D2R directions, there is no lower bound on the minimum TB size, and a maximum TB size of around 1000 bits can be supported.

Hence, 128 bits in A-IoT paging message is feasible since there is enough available bits according to the current conclusions.

Proposal 1:	RAN2 replies LS to SA3:

It is feasible to include a 128-bit security parameter in paging message;

The maximum remaining size for additional security parameters can be determined based on the latest A-IoT MAC specification, assuming the upper bound limitation of paging message is 125 bytes. However, less overhead is preferred from RAN2 perspective.

## 2.2	Transaction ID

### 2.2.1	Transaction ID generation (Issue 1-2)

According to the discussion in RAN3/SA2, there is a Correlation Identifier transmitted over NGAP interface. The reader can use this Correlation Identifier to associate the response(s) from devices to a specific service request from CN. If we use this Correlation Identifier as the transaction ID, the length of the transaction ID could be large, since the Correlation Identifier is used to identify a service among all the services from different CN nodes. On the other hand, considering the AS overhead and the buffer size required by the device storage for transaction ID, the transaction ID in the paging message should not have the same size with the Correlation Identifier from CN.

There could be multiple methods for reader implementation to generate the transaction ID.

- A first possible method could be for the transaction ID to use the right-most truncated part of the Correlation Identifier from CN, which is shorter than the Correlation Identifier.

- A second method could be that the reader can try to use different transaction IDs for different CN Correlation Identifiers, according to different implementation manners (i.e. different truncated ways or reader’s mapping table).

- Alternatively, the reader can generate the transaction ID not considering the CN Correlation Identifier on purpose. In such case, the reader may not intend to avoid the redundant responses.

How the reader will generate "transaction ID" can be implemented in different manners mentioned above, which should be left to the network implementation. Therefore, it is suggested to not specify how the reader generates the transaction ID.

Proposal 2:	(Issue 1-2) It is proposed to NOT specify how the reader generates the transaction ID, while some reader implementation manners can be the following: (Already covered by post-email summary)

The transaction ID is the right-most truncated part of CN Correlation Identifier; or

The reader tries to use different transaction IDs for different CN Correlation Identifiers; or

The reader generates the transaction ID without considering the CN Correlation Identifier at all (intentionally).

### 2.2.2	Transaction ID size (Issue 1-2)

Firstly, one main motivation of using a larger size of the transaction ID is to avoid ID collision between two parallel services from different readers which have the interleaved coverage. In RAN2#130 meeting, it was agreed that multi-reader parallel services are not supported in Rel-19:

Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification.

Secondly, transaction ID is only carried in A-IoT paging message, which is not too much overhead-sensitive compared with other messages, e.g., Access Trigger message, which, on the contrary, needs to be transmitted many times in one paging round.

Hence, from the technical perspective, there are not strong concerns or issues to prefer one transaction ID size in the range from 3 to 6 bits.

Observation 2:	Since RAN2 conclusion only needs transaction ID carried in paging message and does not consider multiple readers parallel services, there should be no major concern or technical issue as to the size of transaction ID.

In RAN2#129 meeting, the following agreement was achieved:

The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID

The correlation ID from CN is defined by other WGs and provided by CN. The transaction ID is generated by reader. The content of transaction ID can be based on CN correlation ID, but the size of correlation ID will not have impact on the size of transaction ID. Hence, RAN2 can decide on the size of transaction ID to ensure the WI completion in August meeting without any input from other WGs.

Proposal 3:	(Issue 1-2) RAN2 makes the decision on the size of transaction ID, e.g., 3~6 bits, without the need to ask other WGs on the correlation ID size. (Already covered by post-email summary)

### 2.2.3	Transaction ID in CFRA (Issue 1-5)

RAN2#130 agreement: For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case.

For CBRA command case, subsequent paging with the same transaction ID allows the on-going device(s) to continue data transmission by keeping AS ID(s). By this method, the reader can trigger other devices access before the command of the on-going device(s) arrives at the reader from CN. This can solve the issue that the command from CN needs a delay and the reader needs to wait the command during this delay, with the drawback that other devices cannot access.

However, for CFRA command case, only one device is selected, so the delay caused by command request generation in CN and transmission via CN-RAN interface, should be negligible. Therefore, there is no need to suspend the on-going data transmission and finish the data transmission across the paging messages. Hence, for CFRA command case, the transaction ID is also not needed.

Proposal 4:	(Issue 1-5) RAN2 confirms the baseline: For CFRA, transaction ID is omitted in the paging message (including the command case). (Already covered by post-email summary)

## 2.3	Paging ID

### 2.3.1	Paging ID length (Issue 1-3)

In RAN2#129bis meeting, RAN2 sent LS on paging ID length. SA2 replied as follows:

S2-2505793: To CT4 ACTION: SA2 kindly asks CT4 to provide the maximum length of the Filtering Information.

According to the structures of Paging ID from CT4, which is introduced in Section 2.1.2:

- The maximum size of AIoT Device Permanent Identifier: 228 bits (8 bit for ID type, 24 bits for PLMN ID, 44 bits for NID, 24 bits for Third Party Identifier, and 128 bits for Identification on Information).

- The maximum size of Filtering Information: 256 bits.

So at most 8 bits are needed to indicate the size of Paging ID.

Furthermore, the size of Paging ID is flexible, because:

- In Filter Information, the length of Component Value varies from 1 to 128 bits, and the minimum unit of change is one bit. So, to indicate the 128 different sizes of Component Value, 7 bits are needed.

- Besides, the presence of PLMN ID, NID, Third Party Identifier and Identification Information is optional, and this results in the 16 different sizes of Paging ID. So only 7 bits are not enough to indicate the size of Paging ID, and at least 8 bits are needed to indicate the size of Paging ID.

Hence, 8-bit field for the “Paging ID length” is needed, and the Paging ID varies in the unit of a bit.

Observation 3:	Based on the reply LSs: C4-252466 and S2-2505793, the max size of paging ID is 256 bits, and it is very flexible size for filter information and permanent device ID.

Proposal 5:	(Issue 1-3) The “paging ID length” is defined as one 8-bit field in the unit of bit is used (as captured in the MAC running CR). (Already covered by post-email summary)

The RAN2#129bis meeting agreed that MAC PDU should be byte-aligned, while the variable length of Paging ID may be not byte-aligned according to the above analysis. Hence, a “fill field” is needed to ensure the byte-aligned MAC PDU, i.e. when the paging message is not byte-aligned, a 1~7-bit “fill field” is introduced in the end of A-IoT Paging MAC PDU.

On the other hand, for future extensions, possible Rel-20 content, e.g., multiple Device IDs, can be added as the “fill field” at the end of A-IoT Paging MAC PDU. In this release, the device shall ignore the values of this field (which is considered as unknown content).

Proposal 6:	(Issue 1-3) RAN2 agrees to use one “fill field” in the end to make paging message byte-aligned (1~7bits) (as captured in the MAC running CR).

### 2.3.2	Paging ID visibility (Issue 1-6)

Paging ID visibility is still one open issue:

Remaining A-IoT MAC open issues: Issue 1-6: Paging ID visibility; Whether Paging ID is invisible or visible to MAC. - Relevant agreements: - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - From the previous discussion, there are some motivations to make paging ID visible to MAC: - 1. Reader can operate on the paging ID for further sub-grouping. The Rapporteur understands that this can be considered as an enhancement from reader side for better system efficiency. From device side, since there is an explicit indication for CBRA and CFRA, the device (even in multi-device CFRA) can determine how to perform random access instead of paging ID/group ID. In this case, such visibility is not an essential function. And according to guidance from chairlady, such enhancement can be considered with lower priority. - 2. Reader can associate the paging ID/device ID and AS ID for a given device within a service request. The Rapporteur understands that, according to RAN3 LS R3-252481, the reader will allocate NGAP device ID for each device and maintain the per-session per-device context, via which the reader can associate the command being received from the NG interface with the AS ID assigned for a device, i.e. such device management/association does not rely on the paging ID/device ID. - 3. The Temp ID may have impact on this visibility discussion. The Rapporteur understands that SA3 has not concluded on the solution of Temp ID. But the majority of the companies seems to think that this Temp ID is maintained/managed between CN and device, since they already concluded there is no AS security in A-IoT. Therefore, no RAN2 discussion is needed before SA3 further inputs..

According to the Rapporteur, no MAC spec impacts are foreseen for the last two motivations. The motivation “Reader can operate on the paging ID for further sub-grouping” has not been excluded.

In our understanding, the gain of sub-grouping on reducing collision probability can be achieved by other implementation solutions. According to the optimal theoretical slotted Aloha efficiency value, the same effect can be achieved by allocating more access occasions in a single A-IoT paging. When many devices are paged, more access occasions are allocated and the devices can be randomly divided among the allocated occasions. In addition to this, sub-grouping has no gain on power saving, because the paged devices anyway need to continuously monitor to avoid missing the subsequent A-IoT Paging/Access Trigger messages.

Proposal 7:	(Issue 1-6) Confirm RAN2 assumption that the paging identifier is transparent to the A-IoT MAC Layer and no MAC spec impacts are foreseen.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	From RAN LS: “RAN expects that completion of the Rel-19 WI in RAN and SA WGs will be as planned.”

Observation 2:	Since RAN2 conclusion only needs transaction ID carried in paging message and does not consider multiple readers parallel services, there should be no major concern or technical issue as to the size of transaction ID.

Observation 3:	Based on the reply LSs: C4-252466 and S2-2505793, the max size of paging ID is 256 bits, and it is very flexible size for filter information and permanent device ID.

SA3 LS

Proposal 1:	RAN2 replies LS to SA3:

It is feasible to include a 128-bit security parameter in paging message;

The maximum remaining size for additional security parameters can be determined based on the latest A-IoT MAC specification, assuming the upper bound limitation of paging message is 125 bytes. However, less overhead is preferred from RAN2 perspective.

Transaction ID generation (Issue 1-2)

Proposal 2:	(Issue 1-2) It is proposed to NOT specify how the reader generates the transaction ID, while some reader implementation manners can be the following: (Already covered by post-email summary)

The transaction ID is the right-most truncated part of CN Correlation Identifier; or

The reader tries to use different transaction IDs for different CN Correlation Identifiers; or

The reader generates the transaction ID without considering the CN Correlation Identifier at all (intentionally).

Transaction ID size (Issue 1-2)

Proposal 3:	(Issue 1-2) RAN2 makes the decision on the size of transaction ID, e.g., 3~6 bits, without the need to ask other WGs on the correlation ID size. (Already covered by post-email summary)

Proposal 4:	(Issue 1-5) RAN2 confirms the baseline: For CFRA, transaction ID is omitted in the paging message (including the command case). (Already covered by post-email summary)

Paging ID length (Issue 1-3)

Proposal 5:	(Issue 1-3) The “paging ID length” is defined as one 8-bit field in the unit of bit is used (as captured in the MAC running CR). (Already covered by post-email summary)

Proposal 6:	(Issue 1-3) RAN2 agrees to use one “fill field” in the end to make paging message byte-aligned (1~7bits) (as captured in the MAC running CR).

Paging ID visibility (Issue 1-6)

Proposal 7:	(Issue 1-6) Confirm RAN2 assumption that the paging identifier is transparent to the A-IoT MAC Layer and no MAC spec impacts are foreseen.

# 4	Annex:	TP for draft reply LS

Title:	[Draft] Reply LS on security parameter in paging request message

Response to:	S3-252392

Release:	Rel-19

Work Item:	FS_Ambient_IoT_solutions

Source:	Huawei, HiSilicon [RAN2]

To:	SA3

Cc:	RAN3, SA2

Contact person:	xxx

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN2 thanks SA3 for the LS on security parameter in paging request message.

RAN2 further discussed this issue and confirmed the feasibility of including a 128-bit security parameter in paging message. Based on the agreed fields in A-IoT Paging message and the maximum TB size of A-IoT R2D message (including Paging message), RAN2 concluded:

- It is feasible to include a 128-bit security parameter in paging message;

- The maximum size for security parameters can be determined based on the latest A-IoT MAC spec, assuming the upper bound limitation of paging message is 125 bytes. However, less overhead is preferred from RAN2 perspective.

2	Actions

To SA3

ACTION: 	RAN2 kindly asks SA3 to take above into account.

3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #131bis	13 - 17 Oct 2025			Prague, CZ

TSG-RAN2 Meeting #132				 17 - 21 Nov 2025									Dallas, US