3GPP TSG RAN WG2 #131			R2-2505526

Bengaluru, India, Aug. 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The following agreements have been made in RAN2 so far [2]-[6]. In this contribution, we provide our views on the first scope on on-demand SSB SCell operation.

Agreements on OD-SSB SCell@RAN2#130

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

Agreements on OD-SSB SCell@RAN2#129bis

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

Agreements on OD-SSB SCell@RAN2#129

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Agreements on OD-SSB@RAN2#127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell@RAN2#127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

- Discussion

2.1	Issue 1 of RRC OD-SSB

In the TS38.331 CR rapporteur’s summary, the following remaining issues were identified [7].

IE CellGroupConfig Editor’s note: FFS value for maxNrofOD-SSB Q9: Suggest value for this parameter or indicate if another WG needs to be liaised?

The value for the parameter of maxNrofOD-SSB should be decided. This parameter is to define the maximum number of configurations OD-SSB that are simultaneously configured per one SCell. The OD-SSB activation and adaptation based on MAC CE can be done only across RRC-configured OD-SSBs. Therefore, we think the necessary value for the above parameter highly depends on how many parameters and their values under the parent IE (OD-SSB-config) can be taken, particularly those that NW wants to indicate dynamically.

From our perspective, since the following parameters should be flexible via MAC CE, at least the value of 16 should be supported. We are open to supporting larger values.

- OD-SSB periodicity: 6 values are defined in total, and we think about 4 variations should be dynamically indicated in practice

- Number of bursts: 8 values are defined in total, and we think about 4 variations should be dynamically indicated in practice

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

Proposal 1: (Issue 1 of RRC OD-SSB)

- Support at least the value of “16” for the maxNroOD-SSB

2.2	Issue 2 of RRC OD-SSB

od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is indicated.. Agreement (RAN1 #121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

According to the above RAN1’s conclusion, RAN2 has to conclude whether one RRC signaling can configure an OD-SSB with ‘od-ssb-nrofBurst’ (meaning that this OD-SSB is to be deactivated implicitly) and simultaneously activate the OD-SSB. In other words, it should be decided whether OD-SSB which are activated directly via RRC can be deactivated based on the counter.

Regarding RRC-based OD-SSB activation, we think that a typical use case is to incorporate it together with direct SCell activation. Through this procedure, one RRC signaling indicates an SCell configuration with activated and a corresponding OD-SSB configuration with activated. For case#2 where OD-SSB is added onto always-on SSB in a cell, the main motivation to activate OD-SSB is to speed up SCell activation procedure. This is because always-on SSBs are basically enough to perform normal SCell operation while SCell is activated. So, it is better to use a finite number of (OD-)SSB just during processing direct SCell activation, which means it is beneficial that OD-SSB to be implicitly deactivated can be activated via RRC.

Proposal 2: (Issue 2 of RRC OD-SSB)

- Support RRC-based activation of an OD-SSB transmission with configuring od-ssb-nrofBurst for the OD-SSB.

2.3	Issue 3 RRC OD-SSB

– OD-SSB-Config RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.; Field servingCellMO is introduced in IE OD-SSB-Config. In this way it is possible to keep the same field name an hence all the procedural text intact..

It should be decided whether servingCellMO is configured directly under Serving Cell config or under each OD-SSB-config. In our undestanding, some parameters configured in servingCellMO, e.g., subcarrier spacing or smtc, might be different across multiple OD-SSBs in case#2 which can be dynamically adapted via MAC CE. If one common servingCellMO is configured under directly ServingCellConfig and shared across these OD-SSBs, there would be undesirrable mismatch between parameters in measobject and OD-SSB opon switching OD-SSBs in a SCell.

Proposal 3: (Issue 3 of RRC OD-SSB)

- We prefer a new servingCellMO (associated with OD-SSB having different frequency from AO-SSB in case#2) to be configured in OD-SSB-Config.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SSB SCell operation for network energy saving.

Proposal 1: (Issue 1 of RRC OD-SSB)

- Support at least the value of “16” for the maxNroOD-SSB

Proposal 2: (Issue 2 of RRC OD-SSB)

- Support RRC-based activation of an OD-SSB transmission with configuring od-ssb-nrofBurst for the OD-SSB.

Proposal 3: (Issue 3 of RRC OD-SSB)

- We prefer a new servingCellMO (associated with OD-SSB having different frequency from AO-SSB in case#2) to be configured in OD-SSB-Config.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127 meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes

- 3GPP, RAN2#129bis meeting, Chairman’s notes

- 3GPP, RAN2#130 meeting, Chairman’s notes

- Report of [POST130][107][NES] (Ericsson), Ericsson