3GPP TSG RAN WG2 #131			R2-2505527

Bengaluru, India, Aug. 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The following agreements have been made in RAN2 so far [2]-[7]. In this contribution, we provide our views on remaining issues of on-demand SIB1 for idle/inactive mode UEs.

Agreements on OD-SIB1@RAN2#127

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

Agreements on OD-SIB1@RAN2#127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Agreements on OD-SIB1@RAN2#128

1.	NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

2.	New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

3.	Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

4.	Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

5.	The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

6.	The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

7.	If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

8.	A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no cor-responding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

9.	UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE ac-quired WUS configuration. FFS on SI validity area rather than cell level.

10.	Working assumption: UL-WUS configuration in RRC release is not supported.

Agreements on OD-SIB1@RAN2#129

1.	There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

2.	Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

3.	The UE follows the legacy validity principle of stored SIB.

4.	For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

5.	SIB-x can be cell specific or area specific, as legacy.

6.	Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

Agreements on OD-SIB1@RAN2#129bis

1.	NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

2.	When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

3.	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

4.	SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

5.	Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

6.	The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

7.	Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

8.	Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

9.	If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

10.	WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

11.	Area id will be used as legacy.

12.	We do not need a separate new triggering condition of OD-SIB1 acquisition.

13.	When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

Agreements on OD-SIB1@RAN2#130

1.	Not to include access related information of NES into WUS configuration.

2.	Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

3.	TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

4.	Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

5.	If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

6.	Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

- Discussion

2.1	Issue 1 of RRC OD-SIB1

In the TS38.331 CR rapporteur’s summary, the following remaining issues were identified [8].

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting. FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1. E.g. “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1.” Q12: Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS?

On whether emphasize the UE behaviour in RRC connected regarding si-BroadcastStatus, we do not think this is necessary because the similar descriptions are already captured in the normative text.

Proposal 1: (Issue 1 of RRC OD-SIB1)

- No need for adding the following description in the field description.

  - “a UE supporting OD-SIB1 in RRC_CONNECTED applies “si-BroadcastStatus” in the stored SIB1.

2.2	Issue 2 of RRC OD-SIB1

Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update? Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this? We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion.

The issue was about how UE reception behaved on SIB1 after the SIB change notification on an OD-SIB1 NES cell. We understood that it is up to NW implementation whether NCD-SSB (CORESET0 is not indicated) or CD-SSB (CORESET0 is indicated) is transmitted along with (OD-)SIB1 following SIB change indication.

In the latter case, NW is supposed to turn NCD-SSB in OD-SIB1 NES cell into CD-SSB by changing the parameters in MIB/PBCH, and then, UE can receive SIB1 properly according to indicated CORESET0 in the legacy manner. Although this can be done by NW implementation, there seems to be some problems, e.g., any legacy UEs may camp on the cell (which is likely NES cell with OD-SIB1 operation) during that period.

In the former case, a UE is expected to receive (OD-)SIB1 based on the parameters stored as UL WUS configurations. If NW wants to update CORESET0 and perhaps other parameters in MIB/PBCH that define the SIB1 resources, UE has no way to identify the time/frequency resources of SIB1 properly. If we have to address this issue, since we have limited usable signalling before SIB1 transmission, such as paging and MIB, there seems only way some parameters for updated essential information are indicated in these signalling.

We agree that the aforementioned situation indeed will be problematic. However, we do not think it is necessary to enhance anything at this later stage. There are some reasons, such as, if NW really wants to update CORESET0, CD-SSB will work enough, updating these parameters is expected to be a very corner case in a real deployment, and UE is still supposed to able to find SIB1 blindly (without indication as CORESET0) by its implementation since there are limited number of patterns to allocate SSB and SIB1 PDCCH.

Proposal 2: (Issue 2 of RRC OD-SIB1)

- Upon SIB update notification, we understand that it is up to NW implementation whether NCD-SSB (CORESET0 is not indicated) or CD-SSB (CORESET0 is indicated) is transmitted along with (OD-)SIB1

- For updating CORESE0, the following way can be considered, and we can live with Opt.1.

  - Opt.1: No spec enhancement, i.e, CD-SSB should be transmitted

  - Opt.2: Enhancement on some signaling (SIB change indication or MIB/PBCH) to inform revised CORESET0.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SIB1 operation for idle/inactive mode UEs network energy saving.

Proposal 1: (Issue 1 of RRC OD-SIB1)

- No need for adding the following description in the field description.

  - “a UE supporting OD-SIB1 in RRC_CONNECTED applies “si-BroadcastStatus” in the stored SIB1.

Proposal 2: (Issue 2 of RRC OD-SIB1)

- Upon SIB update notification, we understand that it is up to NW implementation whether NCD-SSB (CORESET0 is not indicated) or CD-SSB (CORESET0 is indicated) is transmitted along with (OD-)SIB1

- For updating CORESE0, the following way can be considered, and we can live with Opt.1.

  - Opt.1: No spec enhancement, i.e, CD-SSB should be transmitted

  - Opt.2: Enhancement on some signaling (SIB change indication or MIB/PBCH) to inform revised CORESET0.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#128 meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes

- 3GPP, RAN2#129bis meeting, Chairman’s notes

- 3GPP, RAN2#130 meeting, Chairman’s notes

- Report of [POST130][107][NES] (Ericsson), Ericsson