3GPP TSG-RAN WG2 Meeting #131	R2-2505531

Bangalore, India, 25th – 29th Aug. 2025

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

- (RRC-3) LP-WUS and dual DRX group

Based on the previous discussion with [2], now RAN2 have the following working assumption on LP-WUS UE’s dual DRX support:

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

When LP-WUS is configured with secondary DRX, and the UE is monitoring LP-WUS outside Active Time with a new data burst, it is beneficial to wake up all cells within both DRX groups to achieve maximum throughput.

Therefore, we suggest RAN2 to confirm the working assumption as an agreement.

Proposal 1: (RRC-3, P1 from [213]38.321CR[3]) Propose RAN2 to confirm the below working assumption:

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

- 3 Conclusion

Proposal 1: (RRC-3, P1 from [213]38.321CR) Propose RAN2 to confirm the below working assumption:

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] R2-2504291 LP-WUS and secondary DRX, Ericsson, Huawei, HiSilicon, NEC, LGE, Apple, InterDigital, OPPO, CATT

[3] R2-25XXXXX_[Post130][213][LPWUS] 38.321 (Apple)_Comments and Open Issues_v15_QC2.docx