3GPP TSG-RAN WG2 Meeting #131                                    	R2-2505533

Bengaluru, India, 25th – 29th August 2025

Agenda item:	8.8.2

Source:	Qualcomm Incorporated

Title:	Msg4 PDSCH repetition capability indication

Document for:	Discussion and Decision

# Introduction

RAN2 received LS from RAN1 in [1] informing us of the following agreement.

Agreement

For the activation/deactivation of Msg4 PDSCH repetition:

- Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0:

  - Signaling uses re-interpretation of 1 MSB in MCS field in DCI.

  - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH.

    - Note: RAN1 considers there is no difference between capability and request

    - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs.

  - The aggregation factor is configured in SIB1, with possible value 2 or 4

    - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

Send LS to RAN2 informing about the above agreement, asking RAN2 to consider the agreement when designing the report in Msg3 PUSCH.

In this document, we further discuss the RAN2 aspects of early UE capability indication for Msg4 PDSCH repetitions.

# Discussion

Early UE capability indication for UL coverage enhancement

Currently a UE can indicate whether it supports or needs Msg4 ACK repetitions via LCID code points. There are six LCID code points used for this purpose as follows.

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

For downlink coverage enhancement purpose, RAN1 has made following agreement.

  - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH.

We understand that RAN1 has asked for a capability/request in Msg3 PUSCH for Msg4 PDSCH. This is specifically for downlink coverage. It is possible UE may need just downlink coverage enhancements, this can be due to downlink link budget. We think downlink and uplink coverage enhancements should be de-coupled and UE should be able to indicate following combinations in Msg3.

- LCID code point to indicate UE supports only Msg4 ACK repetition

- LCID code point to indicate UE supports only Msg4 PDSCH repetitions

- LCID code point to indicate UE supports both Msg4 ACK repetition plus Msg4 PDSCH repetitions.

There are plenty of LCID code points available and further use of such LCID code points is not expected given the start of 6G study. Therefore, we propose to use new 6 LCID code points to indicate UE supports only Msg4 PDSCH repetitions and additional 6 new LCID code points to indicate UE supports both Msg4 ACK repetition plus Msg4 PDSCH repetitions.

RAN1 has agreed UE capability/request indication in Msg3 for downlink coverage only not for uplink coverage. There are plenty of LCID code points available for early indication in Msg3.

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 | (216 + 328) | CCCH of size 48 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 9 | (216 + 323) | CCCH of size 64 bits for PDSCH repetition of Msg4, except for an (e)RedCap UE |
| 10 | (216 + 329) | CCCH of size 48 bits for PDSCH repetition of Msg4 of a RedCap UE |
| 11 | (216 + 330) | CCCH of size 64 bits for PDSCH repetition of Msg4 of a RedCap UE |
| 12 | (216 + 331) | CCCH of size 48 bits for PDSCH repetition of Msg4 of an eRedCap UE |
| 13 | (216 + 332) | CCCH of size 64 bits for PDSCH repetition of Msg4 of an eRedCap UE |
| 14 | (216 + 333) | CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 15 | (216 + 334) | CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 16 | (216 + 335) | CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 17 | (216 + 336) | CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 18 | (216 + 337) | CCCH of size 48 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 18 | (216 + 338) | CCCH of size 64 bits for PDSCH repetition of Msg4 and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 19 to 63 | (216 + 339) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

use new 6 LCID code points to indicate UE supports only Msg4 PDSCH repetitions.

use additional 6 new LCID code points to indicate UE supports both Msg4 ACK repetitions plus Msg4 PDSCH repetitions.

# Conclusion

Following proposals are made.

Observation 1.	RAN1 has agreed UE capability/request indication in Msg3 for downlink coverage only not for uplink coverage. There are plenty of LCID code points available for early indication in Msg3.

Proposal 1	use new 6 LCID code points to indicate UE supports only Msg4 PDSCH repetitions.

Proposal 2	use additional 6 new LCID code points to indicate UE supports both Msg4 ACK repetitions plus Msg4 PDSCH repetitions.

# References

[1] R2-2505024, “LS on Msg4 PDSCH repetition” RAN2#131, Source RAN1, To RAN2.