3GPP TSG-RAN WG2 #131                                          	    R2-2505545

Bengaluru, India, August 25-29, 2025

Agenda item:	   8.6.4

Source: 		Qualcomm Incorporated

Title: 	Discussion on conditional intra-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2#129b agreed the following regarding conditional LTM [1]:

Agreements on stage-3 details: - Support the co-existence of CLTM and (e)RedCap. - Support the co-existence of CLTM and CovEnh. - (RRC-9) Use the term “LTM” for parts which are common to LTM and CLTM. Use the term “CLTM” only for parts which apply only to CLTM (but not LTM). This will be used in all RAN2 specifications. Agreements on C-LTM: - (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed. - For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant. - Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. - During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. - In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS. - RAN2 will not do anything for the separate TCI mode optimization.

This paper discusses further aspects related to intra-CU conditional LTM.

# 2	CLTM candidate TA handling for LTM Cell Switch Command MAC CE (Open issue MAC-14)

In [POST130][111][MOB] email discussion for MAC, the following item is discussed for handling CLTM candidate TA value while TAT is running (i.e., valid candidate TA) when an (Enhanced) LTM Cell Switch Command MAC CE is received.

FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received.

According to [2], the main discussion point for MAC-14 is the expected UE behaviour when it receives the (Enhanced) LTM CSC MAC CE with TA value set to ‘FFF’ while the CLTM candidate TAT is running for the target candidate cell. While some companies proposed to let UE repurpose the CLTM candidate TA for LTM cell switch command, but this proposal has some concerning points as below:

Firstly, NW would lose the choice of being able to trigger RACH-based LTM at all if UE reuses the CLTM candidate TA. NW is expected to have the most up-to-date information for NW-triggered LTM decision, therefore when NW triggers the LTM cell switch to a candidate cell, the parameter values in the cell switch command should be followed by UE. Since CLTM candidate cell is a subset of LTM candidate cells and the same early TA acquisition procedure is used for both LTM and CLTM, NW should maintain the TA value of a candidate cell for LTM cell switch as well as CLTM switch and could specify it in LTM cell switch command MAC CE.

Observation 1a: The network is expected to have the most up-to-date information for TA information. Therefore, it should still be able to trigger RACH-based LTM for a target candidate cell even if CLTM TA timer is running at UE for that candidate cell.

Secondly, determining the time alignment timer for PTAG is different between LTM and CLTM. As shown in the snippet of Clause 5.2 in MAC running CR, for LTM cell switch case, the RRC-configured time alignment timer associated with the PTAG is (re)started when LTM Cell Switch Command MAC CE has a valid TA value (not set of ‘FFF’). However, for CLTM cell switch case, the remaining time of CLTM TA timer is converted to the time alignment timer associated with the PTAG, and then it is (re)started upon CLTM cell switch. As there are two different ways to derive the time alignment timer for PTAG between LTM and CLTM, it would be simpler to keep those two cases separate.

Observation 1b: Handling of the time alignment timer for PTAG is different between LTM and CLTM. Reusing CLTM TA value would unnecessarily complicate the specification and it would be better to separate LTM and CLTM cases completely.

Clause 5.2 snippet in MAC running CR: 1> when an (Enhanced) LTM Cell Switch Command MAC CE is received and the Timing Advance Command is not set as FFF: 2> apply the Timing Advance Command for the PTAG as specified in clause 6.1.3.75; 2> start or restart the timeAlignmentTimer associated with the PTAG as specified in clause 6.1.3.75. 1> when an (Enhanced) LTM Cell Switch Command MAC CE is received, and the Timing Advance Command is set as FFF, and the UE has successfully measured the Timing Advance as in clause 5.18.35: 2> apply the measured Timing Advance for the PTAG; 2> start or restart the timeAlignmentTimer associated with the PTAG. 1> when a conditional LTM cell switch procedure is triggered for a CLTM candidate cell or indicated by upper layer as specified in clause 5.y.3: 2> if the CLTM candidate cell is not configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell is running as specified in clause 5.2x: 3> apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 3> start or restart the timeAlignmentTimer associated with the PTAG with the length of the remaining time of the ltm-Candidate-TimeAlignmentTimer. 2> if the CLTM candidate cell is configured with two TAGs and the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the CLTM candidate cell for the TAG associated with the selected SSB or selected CSI-RS for CLTM is running as specified in clause 5.2x: 3> apply the stored TA value associated with the CLTM candidate cell for the PTAG as specified in clause 6.1.3.4x; 3> start or restart the timeAlignmentTimer associated with the PTAG with the length of the remaining time of the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2. 2> else if the UE has successfully measured the Timing Advance as in clause 5.18.35: 3> apply the measured Timing Advance for the PTAG; 3> start or restart the timeAlignmentTimer associated with the PTAG.

Proposal 1: RAN2 agree that UE follows the legacy behaviour for the TA value in LTM Cell Switch Command MAC CE, irrespective of CLTM TA value & TA timer status for the target candidate cell.

    - If LTM Cell Switch Command MAC CE has a TA value other than ‘FFF’, UE performs RACH-less LTM cell switch.

    - If LTM Cell Switch Command MAC CE has a TA value as ‘FFF’, UE performs RACH-based LTM cell switch unless UE successfully measured TA value.

# 3	RACH-less CLTM recovery

In RAN2#129, it was agreed to reuse Rel-18 LTM failure recovery mechanism, i.e., CBRA-based LTM switch procedure. While Rel-18 CBRA-based LTM fast recovery can be naturally supported for CLTM, the possibility of utilizing CG resources for RACH-less CLTM fast recovery should be considered.

Rel-18 LTM fast recovery had to be RACH-based LTM switch because only the target candidate cell in LTM CSC MAC CE activates the reception of CG resources and all the other candidate cells wouldn’t know about LTM switch of UE.

In Rel-19 CLTM, however, the serving cell would have to inform several candidate cells (which the UE may try CLTM switch) of activating CG resource receptions because it doesn’t know which candidate cell will be chosen by the UE. As agreed in RAN2#129, the UE would try CG-based RACH-less CLTM when at least one candidate beam fulfils the event condition and the UE has a valid TA (either through TA MAC CE or UE-measured TA). Therefore, it is possible that one or more candidate cells still monitor CG resources when UE tries CLTM fast recovery, in which case utilizing the CG resource could further reduce the recovery latency compared to RACH-based CLTM fast recovery.

Observation 2: Unlike Rel-18 LTM, there could be more than one candidate cell that is indicated by the serving cell to monitor CG resources for potential CLTM execution by the UE. Then it would be beneficial to utilize those CG resources for RACH-less CLTM fast recovery.

Proposal 2: RAN2 agree to support RACH-less CLTM recovery for Rel-19 intra-CU CLTM when the following conditions are met:

- The selected cell is one of CLTM candidate cell AND

- The selected CLTM candidate cell has configured CG resource AND

- UE has a valid TA for the selected CLTM candidate cell by either (i) TA from TA MAC CE for which TA timer is running or (ii) UE-measured TA.

# 4	Early CLTM target cell notification for RACH-less CLTM

When it comes to CG-based first UL transmission on target cell for RACH-less CLTM, due to the unknown CLTM cell switch timing to both serving cell and multiple candidate cells, CG time/frequency resources would have to be reserved earlier than actual CLTM timing and most of them would be left unused, i.e., those CG resources cannot be used for other PUSCH allocations due to the potential collision with CLTM UE’s first UL transmission. As more potential target candidate cells configure such CG resources, the overall resource inefficiency could become even more severe.

Therefore, it would be beneficial to introduce a new signaling procedure (“early target cell notification”) for UE to notify the potential target cell(s) and beam(s) for CLTM to the serving cell such that the candidate cells can start monitoring CG resources only for a certain time period after receiving the notification message from the serving cell. This procedure would allow other candidate cells (which were not selected) to reuse the configured CG resources for other PUSCH allocations without any collision issue.

Observation 3: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 3: For RACH-less CLTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target CLTM cell(s) and beam(s).

# 5	Early TA signalling and maintenance

## 5.1	Candidate cell TA group

For the successful RACH-less CLTM, it is necessary for UE to have a valid TA value for the target candidate cell. Through measurement reports, serving cell would be able to roughly know which candidate cells are potentially target candidate cells for CLTM. However, if the number of those candidate cells is not small enough, these multiple PDCCH ordered RACH procedures would become overhead such that DL and/or UL performance would degrade due to interruptions.

To alleviate the PDCCH ordered RACH overhead issue, the candidate cell TA group can be considered where UE can assume an acquired TA value from one candidate cell (either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells within the TA group. By adopting this TA group, serving cell can trigger only a few PDCCH ordered RACH procedures for all candidate cells within the TA group, hence the RACH overhead could be significantly reduced.

Observation 4: For successful RACH-less CLTM, the serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could increase overhead and interruptions in data transmission and reception at the UE.

Proposal 4: RAN2 to support the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

## 5.2	TA validation procedure

In RAN2#129, it was agreed the TA timer-based TA validation procedure, i.e., if CLTM TA timer starts/re-starts when UE receives TA value for the candidate cell via MAC CE, and as long as the TA timer associated with the candidate cell is running UE considers the TA for the candidate cell is valid.

Since the purpose of TA adjustment in uplink transmissions is to compensate for the additional propagation delay, the TA validation procedure should be able to track the propagation delay change, which can be also identified by RSRP change (reflecting a pathloss change).

The TA validation procedure at UE has been discussed in other topics before such as CG-SDT transmission or SRS transmission for positioning, and the existing validation procedure can be used as a baseline. According to Section 5.72.2 in TS 38.321 MAC specification, the following procedure is defined for the TA validation for CG-SDT.

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running

From above three conditions, the second (RSRP change threshold) and the third (TA timer) would be relevant for CLTM. The first condition (valid RSRP) is to cover UE in RRC_INACTIVE state, so it is not necessary for CLTM.

In addition to the TA timer that has been already agreed, checking the RSRP change (L1 or L3) can be used to track UE’s pathloss change which impacts the propagation latency.

Observation 5: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in CLTM execution.

Proposal 5: RAN2 agree to the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

# Conclusion

This contribution discussed aspects related to conditional intra-gNB LTM. The following observations and proposals have been made:

Observation 1a: The network is expected to have the most up-to-date information for TA information. Therefore, it should still be able to trigger RACH-based LTM for a target candidate cell even if CLTM TA timer is running at UE for that candidate cell.

Observation 1b: Handling of the time alignment timer for PTAG is different between LTM and CLTM. Reusing CLTM TA value would unnecessarily complicate the specification, and it would be better to separate LTM and CLTM cases completely.

Proposal 1: RAN2 agree that UE follows the legacy behaviour for the TA value in LTM Cell Switch Command MAC CE, irrespective of CLTM TA value & TA timer status for the target candidate cell.

    - If LTM Cell Switch Command MAC CE has a TA value other than ‘FFF’, UE performs RACH-less LTM cell switch.

    - If LTM Cell Switch Command MAC CE has a TA value as ‘FFF’, UE performs RACH-based LTM cell switch unless UE successfully measured TA value.

Observation 2: Unlike Rel-18 LTM, there could be more than one candidate cell that is indicated by the serving cell to monitor CG resources for potential CLTM execution by the UE. Then it would be beneficial to utilize those CG resources for RACH-less CLTM fast recovery.

Proposal 2: RAN2 agree to support RACH-less CLTM recovery for Rel-19 intra-CU CLTM when the following conditions are met:

- The selected cell is one of CLTM candidate cell AND

- The selected CLTM candidate cell has configured CG resource AND

- UE has a valid TA for the selected CLTM candidate cell by either (i) TA from TA MAC CE for which TA timer is running or (ii) UE-measured TA.

Observation 3: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 3: For RACH-less CLTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target CLTM cell(s) and beam(s).

Observation 4: For successful RACH-less CLTM, the serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could increase overhead and interruptions in data transmission and reception at the UE.

Proposal 4: RAN2 to support the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

Observation 5: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in CLTM execution.

Proposal 5: RAN2 agree to the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

# References

- Chairman Notes; TSG RAN WG2 Meeting #130

- R2-2504701, Discussion report on [AT130][101][MOB] MAC open issues