3GPP TSG RAN WG2#131	R2-2505553

Bengaluru, IN, August 25th – 29th, 2025

Agenda Item:	8.17

Source:	OPPO

Title:	Discussion on IoT NTN TDD mode

Document for:	Discussion, Agreement

Introduction

In RAN#105 meeting, a new WID on IoT NTN TDD mode was approved, which was further revised to [1] in RAN#106 meeting. This work item aims to enable Iridium’s TDD band to provide NB-IoT service leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures.

In the last RAN2 meetings, TDD support for NB IoT NTN was discussed, and the following agreements were reached.

Agreements in RAN2#129:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

- RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- RAN2 can continue the discussion also on RAR window

- Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

Agreements in RAN2#129bis:

1.	Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe.

2.	In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell.

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

4.	When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe

5.	When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

Agreements in RAN2#130:

1.	The SI-message transmission can be postponed to the next valid D frame within the SI-Window

2.	It is up to NW implementation to avoid SI-window overlap

3.	SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed)

4.	In IoT-NTN TDD mode, the RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted (i.e. no spec change)

5.	For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for FDD (i.e., 10.24s) is used for IoT NTN TDD.

6.	In IoT-NTN TDD mode the same formula as for RA-RNTI calculation for FDD is reused

7.	No extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD

8.	The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs.

9.	It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact)

10.	Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications:
IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (can revisit this based on the TP being prepared by RAN1)

11.	In Rel19, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

12. In IoT_NTN_Ph3-Core, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode. RAN2 understands that, as part of the IoT_NTN_TDD WI, we can discuss on a case by case basis whether minor specific enhancements – not affecting other WGs - can be supported to ensure that (some of) the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode

In this contribution, we further discuss left open issues on TDD support for NB IoT NTN and present our view.

Discussion

- In RAN1#120bis, RAN1 made the following agreements regarding NPRACH transmission in NTN TDD mode.

Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported

- Based on RAN1 agreements, in NTN TDD mode, the minimum NPRACH periodicity is 90ms, which is equal to the TDD frame periodicity, so in each periodicity of 90ms, there would be at most one NPRACH transmission in non-U NB-IoT subframes to be postponed to the next U NB-IoT subframes. Therefore, there is no collision for NPRACH transmission.

- There is no collision issue for NPRACH transmission.

Regarding ULSPS，RAN2 has agreed that when the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next U subframe, so there may be SPS collision between different UEs. As UE cannot be aware of the UL SPS collision, it should be up to network configuration to avoid this collision, e.g.UE can be configured with a (starting) subframe id within the 8 UL NB-IoT subframes for UL SPS transmission. Since there are also UL SPS occasions located within the U NB-IoT subframes, these UL SPS occasions without any postponement may collide with other postponed UL SPS occasions. To resolve the problem, it is needed that for a SPS occasion within the 8 UL NB-IoT subframes, its (starting) subframe id also follow the configured (starting) subframe id. In other words, the configured (starting) subframe id should be applied to all UL SPS transmissions, regardless of the UL SPS occasion overlapping with non-U NB-IoT subframes or not.

- There may be SPS collision between different UEs, and UEs cannot be aware of the UL SPS collision.

In NTN TDD mode, UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

- In TS36.321, a note on scheduling restriction is given as follow.

NOTE 4: For NB-IoT, for operation in FDD mode, and for operation in TDD mode with a single HARQ process when the associated HARQ RTT timer has been started, DL and UL transmissions will not be scheduled in parallel, i.e. if a DL transmission has been scheduled an UL transmission will not be scheduled until HARQ RTT Timer of the DL HARQ process has expired (and vice versa).

- According to the above note, for a NB-IoT UE configured with a single HARQ process, the UE is not required to monitor PDCCH since a DL or UL transmission has been scheduled. This has also been reflected in the start condition of onDurationTimer as highlighted below.

- if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

- In NTN TDD mode, for UL or DL transmission with repetition, if transmission duration is longer than 8ms, the transmission may be segmented. For UL segmented transmission, there will be DL NB-IoT subframes located between successive UL segmented transmissions. Following the principle in the above note, UE should not monitor PDCCH during these DL NB-IoT subframes, and this should be considered as an additional start condition restriction for onDurationTimer

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Conclusion

Based on the discussion we propose the following:

- In NTN TDD mode, UE is configured with a (starting) subframe id within the 8 UL NB-IoT subframes for all UL SPS transmissions.

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Reference

[1] RP-243293, Revised WID on introduction of IoT-NTN TDD mode