3GPP TSG-RAN WG2 Meeting #131	R2-2505565

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	Open issues on AIoT paging

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses several open issues as identified by the Rapporteur in [POST130][027][AIoT] MAC Running CR (Huawei) and concerns more specifically AIoT paging aspects.

# 2	Discussion

In the previous meeting, RAN2 made the following agreements:

Agreements on paging - For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. - For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device). - To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. - Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message. - Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

In the context of the inventory / command use case, we observe that a command may be delivered with non-negligible delay after a device is identified by the reader / network. There are multiple reasons for this. A command may be generated only based on the results of the inventory phase in which one or more devices are “discovered” by the network in a given area and a serviceable connection is established. The application layer may need time to process a multitude of devices are the same time, possibly in a rather remote server, subject to application-specific constraints (eg, access to a database).

All of this may cause a further delay before a command to a specific device is generated and delivered to the reader, which on its own may carry out internal prioritization.

Observation 1: A command may be delivered with a non-negligible delay from Msg 2.

Moreover, the same AIoT request may be sent to multiple readers at the same time, especially if the location of the targeted devices is not well known, devices are mobile, or a larger time interval passed after the last successful contact with a device. Also, multiple (sub)-commands may be delivered within the same AIoT request (transaction) from the network, for example as part of application-layer communications with the device based on command/response exchange.

Observation 2: There may be multiple (sub)-commands associated within the same transaction.

To ensure that a command is delivered (received) by the correct device in the correct context, a transaction ID should always be included in the associated MAC message.

Proposal 1: To avoid process and/or reader ambiguity, transaction ID is sent with every command message.

Given the possibility of non-negligible delays, the device should expect the command message only for a limited time to for example save its energy resources (less monitoring) and have the option to release data from its buffer (if permissible from application / implementation point of view).

Proposal 2: Device expects command for pre-defined duration after Msg 2. FFS details (finite time / counter, reception of specific message).

As the core network may issue an AIoT request to multiple gNBs (eg all gNBs withing the same RNA or predefined area), it seems desired from efficiency and management point of view to structure the content of a transaction ID. More specifically, the transaction ID may deliver key information about the CN process and reader that triggered it. This simplifies addressing and routing, especially in circumstances when multiple readers handle multiple separate or replicated requests in a given coverage area. Multi-reader operation was agreed to be conducted without any explicit support from the specification, that is, based on pure implementation basis.

Observation 3: Core network may issue multiple simultaneous requests (including replication of duplicates) to multiple readers.

Two parameters seem to be particularly relevant for characterizing the communication chain between the core network and an AIoT device: the correlation ID and the reader ID. Ideally, a transaction ID reflects uniquely both of these parameters which however could result in a longer format if full length IDs are use (eg based on simple concatenation). To reduce the signalling overhead, only portions of the correlation and reader IDs may be used but it is important to ensure at least local uniqueness (in a given coverage area or within a set of active processes), critical for said efficient addressing and routing.

Proposal 3: RAN2 assumes that the transaction ID consists of

- one portion that is uniquely derived from the CN correlation ID

- another portion that uniquely identifies the reader.

To make more specific proposal in this direction (eg the nature of said unique mapping), the transaction ID size however needs to be determined, preferably by SA2 and/or RAN3.

Proposal 4: Send an LS to SA2/RAN3 to inform them about RAN2 agreements on transaction ID content, and ask for the size of transaction ID.

# 3	Conclusion

This document has made the following observations:

Observation 1: A command may be delivered with a non-negligible delay from Msg 2.

Observation 2: There may be multiple (sub)-commands associated within the same transaction.

Observation 3: Core network may issue multiple simultaneous requests (including replication of duplicates) to multiple readers.

And proposed the following:

Proposal 1: To avoid process and/or reader ambiguity, transaction ID is sent with every command message.

Proposal 2: Device expects command for pre-defined duration after Msg 2. FFS details (finite time / counter, reception of specific message).

Proposal 3: RAN2 assumes that the transaction ID consists of

- one portion that is uniquely derived from the CN correlation ID

- another portion that uniquely identifies the reader.

Proposal 4: Send an LS to SA2/RAN3 to inform them about RAN2 agreements on transaction ID content, and ask for the size of transaction ID.