3GPP TSG-RAN WG2 Meeting #131	R2-2505584

Bangalore, India  Aug 25th – 29th , 2025

| Agenda Item: | 8.6.4 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Remaining issues on CLTM |
| Document for: | Discussion and Decision |

1.	Introduction

This paper discusses the remaining RRC and MAC open issues for Rel-19 conditional intra-CU LTM.

For RRC, we discuss two following open issues: 1) misalignment between the agreement and RRC running CR; 2) How to restart CLTM evaluation after reconfiguration with sync. For the first issue, we propose to revisit and modify the agreement to in line with the lateast RRC running CR (i.e., keep the current ASN.1 and text in RRC running CR). Regarding the second issue, the RRC Rapporteur suggested that it is not an issue and we agree with that. However, we think that similar issue could also happen in normal handover (i.e., reconfiguration with sync) so that it needs to be clarified to avoid repeating similar discussions. We propose that it is up to network whether UE performs CLTM evaluation after reconfiguration with sync procedure.

For MAC, we discuss the open issues summarized by the MAC Rapporteur as follows:

| MAC-28 | During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, if UE based TA is valid, whether/how to fallback. [Rapp]: New added based on Lenovo’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| --- | --- | --- |
| MAC-29 | When CLTM candidate configuration is reconfigured/released, whether/how to handle the stored TA value and the running TAT for the corresponding candidate cell. [Rapp]: New added based on Offinno’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| MAC-30 | If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer. [Rapp]: New added based on OPPO’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| MAC-31 | Upon the CLTM execution, whether activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated or not. [Rapp]: New added based on vivo’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |

2.	Discussion

2.1.	RRC open issues

RRC-1: Misalignment between the agreement and RRC running

Based on the latest RRC running CR [1], the execution condition for initial CLTM is included in LTM-Config, i.e., outside of CLTM configuration of each candidate cell, which does not seem to be in line with the agreement (i.e., yellow highlighted part).

To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

In our view, it is good to keep the current ASN.1 implementation (i.e., the execution condition for initial CLTM is included in LTM-Config). This is because it is natural to include initial LTM related configuration for the current serving cell into LTM-config (i.e., not into each candidate configuration).

However, leaving the agreement wording as is could cause confusion for other working groups (e.g., RAN3). Therefore, we need to clarify and modify the agreement e.g. as follows: The LTM configuration (i.e., LTM-Config) shall include the execution condition for initial CLTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

Proposal 1. Revisit the following agreement made in RAN2#128, and modify the wording to in line with the ASN.1 implementation in the current RRC running CR.

- Original agreement: The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- Modified agreement: The LTM configuration (i.e., LTM-Config) shall include the execution condition for initial CLTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

RRC-2: How to restart CLTM evaluation after reconfiguration with sync

In [Post130][110], the following open issue is raised:

| Ofinno | According to the current running CR, UE starts performing CLTM condition evaluation only in 2 cases: Start case 1) if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup Start case 2) On executing LTM to a candidate cell and ltm-ExecutionCondition is configured within the LTM-Candidate IE for the selected LTM candidate configuration There are 2 scenarios where the CLTM condition evaluation is stopped: Stop case 1) During CLTM execution. In this case, the UE restarts performing the CLTM evaluation after applying the candidate cell configuration, according to Start case 2 above. Stop case 2) On initiating MCG failure information procedure. In this case, the current running CR does not have a procedure for the UE to resume/ start performing the CLTM condition evaluation. We think the UE should resume CLTM condition evaluation after successful recovery from MCG failure RRC reconfiguration for MCG is received while T316 is running; Alternatively, the handling of CLTM condition evaluation during MCG failure information procedure can be simplified if the UE ensures that CLTM is only executed when timer T316 is not running. | If this issue is not address, UE will cannot perform CLTM condition evaluation/ CLTM execution until an network sends an RRC reconfiguration message including ltm-config comprising the field ltm-ServingCellExecutionCondition set to setup. |
| --- | --- | --- |

The issue described above is that CLTM evaluation is not restarted after MCG failure information procedure based recovery is successfully completed. To be specific, MCG failure information procedure based recovery is successfully completed when the RRCReconfiguration message including reconfigurationWithSync for the PCell has been received while T316 is running. The problem described above is fundamentally caused by the lack of procedure to restart CLTM evaluation after receiving/applying a new RRCReconfiguration including reconfigurationWithSync for PCell.

In other words, based on the current RRC, CLTM evaluation is not restarted after receiving/applying a new RRCReconfiguration including reconfigurationWithSync for PCell. This implies that CLTM evaluation is also not restarted after L3 handover completion. In order to resolve the issue, we think it is better to scope reconfiguration with sync procedure rather than MCG failure information procedure, i.e., T316 based approach only handles the limited case.

Basically, CLTM is specified for intra-CU scenario whereas reconfiguration with sync procedure (i.e., L3 handover) is specified for all mobility scenarios including both inter-CU and intra-CU scenarios. This means that the configured CLTM may become invalid after the reconfiguration with sync procedure for inter-CU mobility. From this, in order for UE to autonomously restart after reconfiguration with sync procedure, it is necessary to introduce a signaling indicating whether the L3 handover is inter-CU mobility or intra-CU mobility, which may require lengthy discussion. Since Rel-19 WI is in its final stages, a simple and straighforward approach is appropriate for now. That is, UE performs CLTM evaluation after reconfiguration with sync procedure based on the network configuration. For example, the RRCReconfiguration for reconfiguration with sync includes ltm-config comprising the field ltm-ServingCellExecutionCondition set to setup, which is up to network.

Proposal 2. It is up to network whether UE performs CLTM evaluation after reconfiguration with sync procedure.

2.2.	MAC open issues

In [2], several CLTM related open issues are raised. In this section, some open issues are addressed.

MAC-28: During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, if UE based TA is valid, whether/how to fallback.

| Lenovo | Last meeting, we made the following agreement based on the offline R2-2504921. They are captured in section 5.y.3 of the latest running CR. Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. As we commented in the last meeting, UE based TA is also supported besides PDCCH order based early TA. The above agreement is related to the case that only PDCCH order based TA is provided without UE based TA. Therefore, we need to further consider the case that both PDCCH order based TA and UE based TA are available upon CLTM execution. [Rapp]: Thank you. It is added as MAC-28. |
| --- | --- |

Before discussing this issue, we think it is necessary to clarify which TA is used for RACH-less CLTM if both PDCCH order based TA and UE based TA are available upon CLTM execution. In Rel-18 LTM, the UE applies the TA received from LTM CSC MAC CE even if UE based TA of the target cell has been obtained. This is because, according to the LTM principle, the network-indicated TA is given priority over the UE-based TA for a specific candidate cell. It makes sense that the principle also applies to this issue. Therefore, UE follows the PDCCH order based TA for the case that both PDCCH order based TA and UE based TA are available upon CLTM execution.

Proposal 3. [MAC-28] UE follows the PDCCH order based TA for the case that both PDCCH order based TA and UE based TA are available upon CLTM execution.

In our view, there may be a meaningful correlation between the PDCCH order based TA and the UE based TA because the both TAs are obtained/calculated based on the same serving cell and the same candidate cell. In other words, although the acquisition procedure for each TA is different, but the physical factors (e.g., source cell, target cell, UE mobility, etc.) considered for acquisition are the same. From this, it is reasonable to assume that UE based TA is invalid when the PDCCH order based TA has been invalid. If RACH-less CLTM is continued by using UE-based TA after TAT expiry (i.e., PDCCH order based TA is invalid), CLTM failure may occur due to T304 expiry, which may increase latency. Futhermore, use of UE based TA after TAT expiry for ongoing RACH-less CLTM may make specification more complicated. Therefore, we argue that during CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM even if UE based TA has been obtained.

Proposal 4. [MAC-28] During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM even if UE based TA has been obtained.

MAC-29: FFS how to handle the candidate TA if the corresponding candidate cell is released.

| Ofinno | For CLTM: According to the current running CR, a UE may continue the candidate TA timers after a candidate cell is removed/ released from the LTM configuration. We think the candidate TA timers need to be stopped when the corresponding candidate cell, for which the candidate TA timer is running, is released. [Rapp]: On the one hand, it is reasonable to remove the stored TA value and stop TAT timer for the corresponding candidate cell. On the other hand, there is no issue if the TAT timer keeps running. Anyway, an open issue in MAC-29 is added for discussion. |
| --- | --- |

In legacy, the correspoding TAT keeps running if SCG has been deactivated or the serving cell has been changed by handover because there is no critical issue incurred by the TAT handling. Further, since the network knows the TAT keeps running and how long the TAT will last, the network can handle the corresponding TA if a problem happens.For these reasons, it is reasonable to follow the legacy principle, i.e., the corresponding TAT keeps running if the candidate cell is released.

Proposal 5. [MAC-29] The corresponding TAT keeps running if the candidate cell is released.

MAC-30: FFS how UE obtains the MAC PDU from HARQ buffer to Msg3/A buffer if RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration

| OPPO | For CLTM： If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer? [Rapp]: Since Msg3/A buffer is empty during RACH-less CLTM, the RRCReconfigruaitonComplete message couldn’t be transimitted via Msg3/A buffer. We think RRCReconfigruaitonComplete could be retx after the competition of the RACH procedure for RRCReconfigruaitonComplete initial transmission. But let’s add one open issue in MAC-30 for discussion. |
| --- | --- |

First of all, when RACH-less CLTM falls back to RACH-based CLTM due to PTAG expiration, it is hard to handle the retransmission of the first transmission (i.e., RRCReconfigurationComplete) at MAC layer. This is because all the HARQ buffers are flushed upon TAT expiry. Basically, an RRC message transmitted via SRB1 is on RLC AM mode, i.e., RLC will retransmit the RRC message if the UE RLC does not receive the acknowledgement corresponding to the RRC message transmission. In our view, it is simple and straighforward to rely on RLC retransmission for this issue.

Proposal 6. [MAC-30] When RACH-less CLTM falls back to RACH-based CLTM due to PTAG expiration, the transmission of the RRCReconfigurationComplete message relys on RLC retransmission.

MAC-31: FFS whether to deactivate the activated candidate TCI state(s) other than the TCI state associated with the triggered beam upon CLTM execution

| vivo | the UE deactivates the activated Candidate TCI state upon receiving the LTM CSC MAC CE in Rel-18, it is not clear that whether the same behaviour should be applied in CLTM in R19, i.e. upon the CLTM execution, activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated. [Rapp]: OK. It seems same behaviour as Rel-18 could be considered for CLTM in Rel-19. MAC-31 is added. |
| --- | --- |

In Rel-18 LTM, the UE deactivates the activated Candidate TCI state upon receiving the LTM CSC MAC CE. In our view, the rationale of Rel-18 behaviour is that the target cell does not know which Candidate TCI state is activated. This implies that, in order to continue the activation/deactivation state of a candidate TCI state after LTM cell switch, the source cell should forward the corresponding information to the target cell before/during LTM cell switch. This necessitate an additional RAN3 signaling, which is not an appropriate issue to address now due to lack of WI time. Therefore, the UE deactivates the activated Candidate TCI state(s) other than the TCI state associated with the triggered beam upon CLTM execution.

Proposal 7. [MAC-31] The UE deactivates the activated Candidate TCI state(s) other than the TCI state associated with the triggered beam upon CLTM execution.

3.	Conclusion

In this paper, we have made the following observations and proposals:

RRC open issues

Proposal 1. Revisit the following agreement made in RAN2#128, and modify the wording to in line with the ASN.1 implementation in the current RRC running CR.

- Original agreement: The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- Modified agreement: The LTM configuration (i.e., LTM-Config) shall include the execution condition for initial CLTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

Proposal 2. It is up to network whether UE performs CLTM evaluation after reconfiguration with sync procedure.

MAC open issues

Proposal 3. [MAC-28] UE follows the PDCCH order based TA for the case that both PDCCH order based TA and UE based TA are available upon CLTM execution.

Proposal 4. [MAC-28] During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM even if UE based TA has been obtained.

Proposal 5. [MAC-29] The corresponding TAT keeps running if the candidate cell is released.

Proposal 6. [MAC-30] When RACH-less CLTM falls back to RACH-based CLTM due to PTAG expiration, the transmission of the RRCReconfigurationComplete message relys on RLC retransmission.

Proposal 7. [MAC-31] The UE deactivates the activated Candidate TCI state(s) other than the TCI state associated with the triggered beam upon CLTM execution.

4.	References

[1] [POST130][110][MOB] RRC CR (Ericsson) Running RRC CR for mobility enhancements phase 4

[2] [POST130][111][MOB] MAC running CR(Vivo)