3GPP TSG-RAN WG2 Meeting #131                                      R2-2505590

Bangalore, India, 25-29 Aug 2025

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID [1] of Rel-19 SBFD related to random access is given as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

This contribution provides our view on the remaining issues of SBFD RACH.

- Discussion

Open issue MAC-2-1, 2-2: CBRA and CFRA contain same RO type

[Open Issue MAC-2-1] Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA? [Open Issue MAC-2-2] If so, please clarify what the changes should be.

If gNB does not configure any SBFD RO for CBRA on a BWP, it means the gNB does not listen/monitor the SBFD RO in CBRA. It may happen when a cell has small amount of SBFD-aware UEs, or gNB only wants to use SBFD resource for additional UL transmission, or gNB would like to save its power to reduce keeping on monitoring additional ROs without knowing when the UE will access using the additional ROs. Since anyway the legacy RO for CBRA is always there, whether gNB wants to configure SBFD RO for CBRA is NW implementation.

However CFRA resource is sent to UE dedicatedly after gNB acquires UE’s capability and CLI measurement, if any. So gNB is quite aware of the UE’s interference and whether UE will access well in SBFD ROs. So gNB can set CFRA resource with SBFD RO to some dedicated UEs. So it is also gNB’s decision on whether to configure SBFD RO for CFRA.

Therefore, it is gNB’s decision on whether to configure SBFD RO in CFRA/CBRA resource. Configuring SBFD RO in CBRA and configuring SBFD RO in CFRA can be decoupled without mandatory configuration relationship.

Observation 1: It is feasible (depends on NW configuration) that CBRA only provides legacy RACH resource, but the CFRA in the same BWP can indicate to use SBFD RO.

RAN2#129-bis has agreed the following working assumption:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

RAN2#130 has agreed the following:

Agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA.

If CFRA indicates to use SBFD RO, UE will select SBFD RO in the first step, then UE selects set within the RACH partition configuration of the SBFD RO among the common RACH resources on the BWP, in the beginning of RA procedure for fallback use. However, the CBRA may not even configure any SBFD ROs, so the fallback set in this case should be selected based on the RACH partition configuration on the legacy RO.

Considering this, the example procedure of CFRA fallback to CBRA can be as follows:

Figure 1. Procedure of UE selecting SBFD RO in a CFRA case and then fallback

Proposal 1: Support the case that CFRA indicates SBFD RO, but the corresponding common RACH resource on the same BWP does not indicate any SBFD RACH resource.

Proposal 2: If a CFRA case indicates to use SBFD RO, when selecting set for the CFRA, UE should:

- If common RACH resource of the same BWP provides SBFD ROs, UE selects set within SBFD RACH resources;

- If common RACH resource of the same BWP does not provide SBFD ROs, UE selects set within the legacy RACH resources.

An example TP is given as below:

5.1.1b Selection of the set of Random Access resources for the Random Access procedure The MAC entity shall: … - else if contention-free Random Access Resources have been provided for this Random Access procedure by PDCCH order: 2> if the RO_TYPE is set to 2nd-RO for the current contention-free Random Access procedure, and the contention-based Random Access Resources does not contain the resource of the second PRACH occasions: 3> Select set(s) of Random Access resources for current contention-free Random Access procedure from the set(s) of Random Access resources associated with the first PRACH occasions; … - else if contention-free Random Access Resources have been provided for this Random Access procedure in the LTM Cell Switch Command MAC CE: 2> if the RO_TYPE is set to 2nd-RO for the current contention-free Random Access procedure, and the contention-based Random Access Resources does not contain the resource of the second PRACH occasions: 3> Select set(s) of Random Access resources for current contention-free Random Access procedure from the set(s) of Random Access resources associated with the first PRACH occasions; … - else if contention-free Random Access Resources have been provided for this Random Access procedure in rach-ConfigDedicated: 2> if the RO_TYPE is set to 2nd-RO for the current contention-free Random Access procedure, and the contention-based Random Access Resources does not contain the resource of the second PRACH occasions: 3> Select set(s) of Random Access resources for current contention-free Random Access procedure from the set(s) of Random Access resources associated with the first PRACH occasions; … - else if contention-free Random Access Resources have been provided for this Random Access procedure in the BeamFailureRecoveryConfig: 2> if the RO_TYPE is set to 2nd-RO for the current contention-free Random Access procedure, and the contention-based Random Access Resources does not contain the resource of the second PRACH occasions: 3> Select set(s) of Random Access resources for current contention-free Random Access procedure from the set(s) of Random Access resources associated with the first PRACH occasions; …

Open issue MAC-3-1, 3-2: SBFD configuration type in CFRA

[Open Issue MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale. [Open Issue MAC-3-2] If so, please provide your preferred solution with the rationale, including but not limited to: - Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE).

The following section provides the reason why the UE has to know SBFD RACH configuration type (option 1 or option 2) before performing RA.

RAN1 has achieved several agreements/conclusions to describe the SBFD RACH configuration option 1 and option 2, and what kind of additional RO that the RACH configuration option 1 and option 2 actually contains:

RAN1#116-bis Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used RAN1#117 Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs RAN1#117 Agreement A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE: - The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH. - UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols - There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH. - Other assumptions are not precluded. NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB. RAN1#118 Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs). RAN1#118 Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

Based on the above RAN1 agreements, we have the following observations:

- Each of the additional RO in SBFD RACH option 1 should have at least one symbol that is a legacy D symbol, and all the symbols of each of the additional RO should be SBFD symbol; (i.e., an additional RO in option 1 does not allow to be all in legacy F symbols, and one additional RO does not allow to extent from SBFD symbol to non-SBFD symbol)

- Each of the additional RO in SBFD RACH option 2 can be either legacy D symbol or legacy F symbol, and at least one symbol of each of the additional RO should be SBFD symbol. (i.e., an additional RO in option 2 can be all legacy D symbols, all legacy F symbols, or legacy D symbol + legacy F symbol, and one additional RO can extent from SBFD symbol to non-SBFD symbol)

Therefore, even if with the same RACH-ConfigGeneric and same TDD configuration and same SBFD time/frequency location configured, RRC SBFD RACH configuration option 1 and option 2 will derive different additional RO location for a SBFD-aware UE, see the following figure as example:

Figure 2. With same RACH-ConfigGeneric and same TDD and SBFD time/frequency location, option 1 and option 2 will derive different additional RO location

Observation 2: Even if with the same RACH-ConfigGeneric, same TDD T/F location and same SBFD T/F location, RRC SBFD RACH configuration option 1 and option 2 will derive different additional RO location for SBFD-aware UE.

RAN1 agrees that in SBFD RACH configuration option 1, the preambleReceivedTargetPower can be replaced by a separate parameter sbfd-RACH-SingleConfig-preambleReceivedTargetPower; in SBFD RACH configuration option 2, the preambleReceivedTargetPower should come from sbfd-RACH-DualConfig-r19. Currently the preambleReceivedTargetPower and the sbfd-RACH-SingleConfig-preambleReceivedTargetPower are both included in RACH-ConfigGeneric, and RACH-ConfigGeneric is included in the CFRA cases. If UE does not know which SBFD RACH configuration type the CFRA uses, the UE cannot determine whether preambleReceivedTargetPower or the sbfd-RACH-SingleConfig-preambleReceivedTargetPower in the RACH-ConfigGeneric should be used for determining RA transmitting power. Thus, it is essential to let UE know the RO type in CFRA case.

Figure 3. In CFRA resource, RACH-ConfigGeneric contains both preambleReceivedTargetPower and the sbfd-RACH-SingleConfig-preambleReceivedTargetPower

Observation 3: If UE does not know which SBFD RACH configuration type the CFRA uses, the UE cannot determine whether preambleReceivedTargetPower or the sbfd-RACH-SingleConfig-preambleReceivedTargetPower in the RACH-ConfigGeneric should be used for determining preamble transmitting power.

Based on above analysis, UE should know which of the SBFD RACH configuration type (i.e., either option 1 or option 2) is provided to the UE in the CFRA case, then UE can decide how to derive the location of additional RO.

Proposal 4: In CFRA, UE should know the SBFD RACH configuration type (option 1 or option 2) to derive the correct RO location, before performing RA on SBFD ROs.

However, currently option 1 and option 2 will not be reflected/provided in each CFRA case. To solve the issue, there can be several solutions:

- One method is to adopt the corresponding common SBFD RACH configuration type for CBRA (if corresponding common RACH resource also provides SBFD RACH configuration).

- Based on analysis in 2.1, CBRA may not configure any SBFD resources. In this case if CFRA indicates to use SBFD RO, there will be no reference on which SBFD RACH configuration type the UE should refer to. So under this scenario, the SBFD RACH configuration type should be explicitly provided for the specific CFRA case.

- Or, UE and gNB can always assume one RACH configuration type for each of the CFRA cases, no matter the SBFD RACH configuration type in common RACH parameters (CBRA) is provided or not.

Therefore, we propose the following:

Proposal 5: RAN2 to agree that:

- If the CBRA indicates SBFD RACH configuration type, the CFRA (using SBFD RO) in the same BWP should adopt the same SBFD RACH configuration type as the CBRA indicates;

- If the CBRA does not indicate SBFD RACH configuration type (i.e., CBRA only contains legacy RO), gNB should indicate which SBFD RACH configuration type the CFRA (using SBFD RO) should adopt.

Open issue MAC-4-1, 4-2: Initialization of RACH parameters

[Open Issue MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale. [Open Issue MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale.

Firstly the analysis is given on whether/how UE performs initialization under other fallback cases (Msg1 low repetition number  Msg1 high repetition number, 2-step RACH  4-step RACH).

The MAC procedure regarding low Msg1 repetition number to high repetition number is as follows:

5.1.4 Random Access Response reception … 2> if the Random Access procedure is not completed: 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources. 3> select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF; 3> if the criteria (as defined in clause 5.1.2) to select contention-free Random Access Resources is met during the backoff time: 4> perform the Random Access Resource selection procedure (see clause 5.1.2). 3> else if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured: 4> delay the subsequent Random Access transmission until the Random Access Procedure is triggered by a PDCCH order with the same ra-PreambleIndex, ra-ssb-OccasionMaskIndex, and UL/SUL indicator TS 38.212 [9]. 3> else: 4> perform the Random Access Resource selection procedure (see clause 5.1.2) after the backoff time.

For Msg 1 low repetition number fallback to Msg 1 high repetition number, they belong to two sets but the RACH-ConfigCommon of the two sets should be same, only FeatureCombinationPreambles-r17 are different. So after UE selects set, UE only needs to initialize the parameters contained in the newly selected FeatureCombinationPreambles-r17.

The reason why the Msg1 repetition number fallback has a separate MAC spec brunch to initialize several parameter is that, the fallback Msg1 repetition number belongs to same RACH-ConfigCommon but different FeatureCombinationPreambles-r17, initializing the parameters in FeatureCombinationPreambles-r17 dedicatedly is to prove this conclusion.

Observation 4: For Msg 1 low repetition number fallback to Msg 1 high repetition number, UE re-selects set with almost the same RRC parameters, then UE only needs to initialize some different RRC parameters in the newly selected FeatureCombinationPreambles-r17.

The MAC procedure regarding 2-step RA fallback to 4-step RA is as follows:

5.1.4a MSGB reception and contention resolution for 2-step RA type … 2> if the Random Access procedure is not completed: 3> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 4> set the RA_TYPE to 4-stepRA; 4> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a; 4> if the Msg3 buffer is empty: 5> obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer; 4> flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer; 4> discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any; 4> perform the Random Access Resource selection procedure as specified in clause 5.1.2.

Since UE firstly selects set, then determines RA type, so 2-step RACH resource and 4-step RACH resource belongs to one set. But they may belong to different RRC RACH configuration, i.e., 2-step RACH resource comes from RACH-ConfigCommonTwoStepRA-r16, but 4-step RACH resource comes from RACH-ConfigCommon. So when fallback, UE has to initialize variables according to clause 5.1.1a.

The reason why the 2-step RACH fallback to 4-step RACH has a separate brunch to initialize several parameter according to clause 5.1.1a is that, UE does not reselect set, UE directly falls back to initialization section (5.1.1a). If UE directly falls back to set reselection, the initialization will be performed naturally.

Observation 5: When 2-step RACH resource falls back to 4-step RACH resource, UE does not select set, UE directly performs initialization of variables according to clause 5.1.1a.

For the RO type fallback, different RO type results to different sets, and different RO type may have different RACH-ConfigCommon. So RO type fallback does require a parameter initialization. And, the whole RACH-configCommon should be initialized, such as RO time/frequency location, RACH partition preamble configuration, etc.

However, according to the program order described in clause 5.1.1, UE will:

- UE selects carrier and BWP;

- UE selects RO type;

- UE selects set of RACH resource (5.1.1b);

- UE initializes some RRC parameters of the selected set;

- UE selects RA type;

- UE initializes UE variables of the selected RA type (5.1.1a);

- UE selects the RACH resource such as RO, preamble (5.1.2);

- UE sends preamble on the selected RO type (5.1.3);

- UE fails X times and fallback to another RO type, UE re-selects set (5.1.4).

Therefore, when UE falls back to another type of RO, UE re-selects set, and UE will continuously follow the order of clause 5.1.1. That is to say, UE will initialize the parameters in RACH-ConfigCommon of the newly selected set without a separate MAC brunch for initialization. UE fallback from 9) to 3), then 4) and 6) will be performed naturally.

Proposal 6: UE does need to initialize the RACH-ConfigCommon after RO type switch, however the current MAC procedure already supports it. No need to add a separate MAC brunch to initialize dedicated RRC parameters after RO type switch.

CSI-RS used in SBFD RO in CFRA

RAN2 has already agreed that SBFD RO supports CSI-RS based CFRA:

Agreement For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion.

For the ra-OccasionList IE in RRC:

CFRA-CSIRS-Resource ::= SEQUENCE { csi-RS CSI-RS-Index, ra-OccasionList SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1), ra-PreambleIndex INTEGER (0..63), ... } CFRA-CSIRS-Resource field descriptions. csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.. ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.. ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS..

One RA-OccasionList provides a list of RO index per CSI-RS resource. Before Rel-19, the RO can be directly sequentially numbered since prach-ConfigurationIndex and msg1-FDM can only derive one set of ROs. However in Rel-19, legacy RO and SBFD RO can use same prach-ConfigurationIndex and msg1-FDM, which means same prach-ConfigurationIndex and msg1-FDM can derive two sets of ROs. So the RO should be sequentially numbered per RO type.

Suggest to adopt the following TP:

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered of the indicated RO type, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

Proposal 7: In CSI-RS based CFRA, the ROs of the ra-OccasionList should be sequentially numbered per RO type. Adopt the TP as baseline.

Fallback from 2-step legacy RO to 4-step SBFD RO

RAN2 has achieved the agreement:

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

The motivation of introducing legacy RO to fall back to SBFD RO is to increase the RA attempt for better successful rate. From this point of view, the legacy 4-step RO and legacy 2-step RO should both support to fall back to SBFD RO. Thus, the maximum failure number threshold should be configured not only in RACH-ConfigCommon, but also in RACH-ConfigCommonTwoStepRA.

Proposal 8: Support 2-step legacy RO to fall back to 4-step SBFD RO. The maximum failure number threshold should also be configured in RACH-ConfigCommonTwoStepRA.

Early identification of SBFD-aware UE

RAN2#127-bis meeting has agreed the following:

Agreement RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

That is to say Msg1 based early identification is not supported in legacy ROs. The question is whether to support Msg3 based early identification when legacy RO is selected.

Figure 4. Example of initial access timeline

Firstly, as the figure indicates, if early identification is achieved in Msg3, the benefit is only on Msg5 UL, NAS security UL and AS security UL (see green arrows), these UL transmission can be scheduled on UL subband. When gNB acquires UE capability, the support of SBFD feature will be directly known by gNB. However the NAS security and AS security set up procedure is only applicable in RRC_IDLE, i.e., even if UE resumes from RRC_INACTIVE, these two steps are not needed. So the scenario of scheduling UL transmission in UL subband becomes more limited.

Secondly, Msg3 based early identification will achieve less benefit than Msg1 based early identification. If companies thinks early identification is not an urgent feature to support for Msg1 based early identification, then Msg3 based early identification shouldn’t be supported at all due to the lower benefit.

Thirdly, if the NW indication and RSRP threshold are not configured, SBFD-aware UE is better to use SBFD RO by default. So early identification can also be achieved when SBFD-aware UE always selects SBFD RO in Msg1/A, the motivation of enhancing Msg3 is further weakened.

Based on the above, we have the following proposal:

Proposal 9: Do NOT support Msg3 based early identification for SBFD-aware UE.

SUL and SBFD RACH configuration

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP can be allocated on NUL carrier or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE performs random access in SBFD symbols, the ROs should occupy or overlap with both SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than NUL DL carrier to provide larger UL coverage, in order to occupy or overlap with resources in the UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 5. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Current stage-2 38.300 running CR does not clearly mention the restriction:

- X SBFD - X.1 General Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their non-overlapping respective sub-bands. From UE perspective, full duplex is not supported. The configurations of cell-specific SBFD time and frequency resources are provided through SIB1 or dedicated signalling. A UE can be semi-statically configured with SBFD sub-bands in downlink symbols and flexible symbols provided by tdd-UL-DL-ConfigurationCommon and this UE is referred to as an SBFD aware UE. The maximum number of UL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is one. The maximum number of DL sub-bands for SBFD operation in an SBFD symbol within a TDD carrier is two.

Therefore we suggest to add the restriction that SBFD operation can only be configured in NUL. Taking SBFD RACH configuration as example, the following example TP is provided:

#### – BWP-UplinkCommon The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell, excluding additionalRACH-perPCI-ToAddModList and additionalRACH-perPCI-ToReleaseList, are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling. BWP-UplinkCommon information element -- ASN1START -- TAG-BWP-UPLINKCOMMON-START BWP-UplinkCommon ::= SEQUENCE { genericParameters BWP, rach-ConfigCommon SetupRelease { RACH-ConfigCommon } OPTIONAL, -- Need M pusch-ConfigCommon SetupRelease { PUSCH-ConfigCommon } OPTIONAL, -- Need M pucch-ConfigCommon SetupRelease { PUCCH-ConfigCommon } OPTIONAL, -- Need M ..., [[ rach-ConfigCommonIAB-r16 SetupRelease { RACH-ConfigCommon } OPTIONAL, -- Need M useInterlacePUCCH-PUSCH-r16 ENUMERATED {enabled} OPTIONAL, -- Need R msgA-ConfigCommon-r16 SetupRelease { MsgA-ConfigCommon-r16 } OPTIONAL -- Cond SpCellOnly2 ]], [[ enableRA-PrioritizationForSlicing-r17 BOOLEAN OPTIONAL, -- Cond RA-PrioSliceAI additionalRACH-ConfigList-r17 SetupRelease { AdditionalRACH-ConfigList-r17 } OPTIONAL, -- Cond SpCellOnly2 rsrp-ThresholdMsg3-r17 RSRP-Range OPTIONAL, -- Need R numberOfMsg3-RepetitionsList-r17 SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17 OPTIONAL, -- Cond Msg3Rep mcs-Msg3-Repetitions-r17 SEQUENCE (SIZE (8)) OF INTEGER (0..31) OPTIONAL -- Cond Msg3Rep ]], [[ additionalRACH-perPCI-ToAddModList-r18 SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF RACH-ConfigTwoTA-r18 OPTIONAL, -- Cond 2TA-Only additionalRACH-perPCI-ToReleaseList-r18 SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17 OPTIONAL, -- Need N rsrp-ThresholdMsg1-RepetitionNum2-r18 RSRP-Range OPTIONAL, -- Need R rsrp-ThresholdMsg1-RepetitionNum4-r18 RSRP-Range OPTIONAL, -- Need R rsrp-ThresholdMsg1-RepetitionNum8-r18 RSRP-Range OPTIONAL, -- Need R preambleTransMax-Msg1-Repetition-r18 ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200} OPTIONAL -- Cond Msg1Rep1 ]], [[ preambleTransMaxRO-Type-r19 ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200} OPTIONAL, -- Need R sbfd-RO-Type-r19 ENUMERATED {sbfd, non-sbfd} OPTIONAL, -- Need S sbfd-RSRP-ThresholdRO-Type-r19 RSRP-Range OPTIONAL, -- Need S sbfd-RSRP-ThresholdRO-TypeUsage-r19 ENUMERATED {above,below} OPTIONAL, -- Need S sbfd-RSRP-ThresholdMsg1-RepetitionNum2-r19 RSRP-Range OPTIONAL, -- Need R sbfd-RSRP-ThresholdMsg1-RepetitionNum4-r19 RSRP-Range OPTIONAL, -- Need R sbfd-RSRP-ThresholdMsg1-RepetitionNum8-r19 RSRP-Range OPTIONAL, -- Need R sbfd-RACH-Config-r19 CHOICE { sbfd-RACH-SingleConfig-r19 NULL, sbfd-RACH-DualConfig-r19 SBFD-RACH-DualConfig-r19 } OPTIONAL -- Cond NULOnly ]] } Conditional Presence; Explanation. Msg1Rep1; This field is optionally present, Need R, if the set(s) of Random Access resources with MSG1 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise.. Msg3Rep; This field is optionally present, Need S, if the set(s) of Random Access resources with MSG3 repetition indication are configured in the BWP-UplinkCommon. It is absent otherwise.. RA-PrioSliceAI; The field is optionally present in SIB1, Need R, if both parameters ra-PrioritizationForAccessIdentity and the ra-PrioritizationForSlicing/ra-PrioritizationForSlicingTwoStep are present in SIB1. It is absent otherwise.. SpCellOnly2; The field is optionally present, Need M, in the BWP-UplinkCommon of an SpCell. It is absent otherwise.. 2TA-Only; The field is optionally present, Need N in the BWP-UplinkCommon if additionalPCI-ToAddModList is present in ServingCellConfig and if tag2 is present in ServingCellConfig. It is absent otherwise.. NoSingleConfig; This field is optionally present, Need R, if sbfd-RACH-Config in BWP-UplinkCommon is set to sbfd-RACH-DualConfig. It is absent otherwise.. NULOnly; This field is optionally present, Need R, if the UL BWP is included in NUL. It is absent otherwise..

Proposal 10: The SBFD operation and SBFD RACH configuration can only be configured in NUL, not on SUL. Add a condition tag in the corresponding RRC signaling. Take the TP as baseline.

- Conclusion

In this contribution, we propose the following observations and proposals:

Open issue MAC-2-1, 2-2: CBRA and CFRA contain same RO type

Observation 1: It is feasible (depends on NW configuration) that CBRA only provides legacy RACH resource, but the CFRA in the same BWP can indicate to use SBFD RO.

Proposal 1: Support the case that CFRA indicates SBFD RO, but the corresponding common RACH resource on the same BWP does not indicate any SBFD RACH resource.

Proposal 2: If a CFRA case indicates to use SBFD RO, when selecting set for the CFRA, UE should:

- If common RACH resource of the same BWP provides SBFD ROs, UE selects set within SBFD RACH resources;

- If common RACH resource of the same BWP does not provide SBFD ROs, UE selects set within the legacy RACH resources.

Open issue MAC-3-1, 3-2: SBFD configuration type in CFRA

Observation 2: Even if with the same RACH-ConfigGeneric, same TDD T/F location and same SBFD T/F location, RRC SBFD RACH configuration option 1 and option 2 will derive different additional RO location for SBFD-aware UE.

Observation 3: If UE does not know which SBFD RACH configuration type the CFRA uses, the UE cannot determine whether preambleReceivedTargetPower or the sbfd-RACH-SingleConfig-preambleReceivedTargetPower in the RACH-ConfigGeneric should be used for determining preamble transmitting power.

Proposal 4: In CFRA, UE should know the SBFD RACH configuration type (option 1 or option 2) to derive the correct RO location, before performing RA on SBFD ROs.

Proposal 5: RAN2 to agree that:

- If the CBRA indicates SBFD RACH configuration type, the CFRA (using SBFD RO) in the same BWP should adopt the same SBFD RACH configuration type as the CBRA indicates;

- If the CBRA does not indicate SBFD RACH configuration type (i.e., CBRA only contains legacy RO), gNB should indicate which SBFD RACH configuration type the CFRA (using SBFD RO) should adopt.

Open issue MAC-4-1, 4-2: Initialization of RACH parameters

Observation 4: For Msg 1 low repetition number fallback to Msg 1 high repetition number, UE re-selects set with almost the same RRC parameters, then UE only needs to initialize some different RRC parameters in the newly selected FeatureCombinationPreambles-r17.

Observation 5: When 2-step RACH resource falls back to 4-step RACH resource, UE does not select set, UE directly performs initialization of variables according to clause 5.1.1a.

Proposal 6: UE does need to initialize the RACH-ConfigCommon after RO type switch, however the current MAC procedure already supports it. No need to add a separate MAC brunch to initialize dedicated RRC parameters after RO type switch.

CSI-RS used in SBFD RO in CFRA

Proposal 7: In CSI-RS based CFRA, the ROs of the ra-OccasionList should be sequentially numbered per RO type. Adopt the TP as baseline.

Fallback from 2-step legacy RO to 4-step SBFD RO

Proposal 8: Support 2-step legacy RO to fall back to 4-step SBFD RO. The maximum failure number threshold should also be configured in RACH-ConfigCommonTwoStepRA.

Early identification of SBFD-aware UE

Proposal 9: Do NOT support Msg3 based early identification for SBFD-aware UE.

SUL and SBFD RACH configuration

Proposal 10: The SBFD operation and SBFD RACH configuration can only be configured in NUL, not on SUL. Add a condition tag in the corresponding RRC signaling. Take the TP as baseline.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06