3GPP TSG-RAN WG2 #131	R2-2505597

Bengaluru, India, 25 – 29 August 2025

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on LP-WUS in RRC_CONNECTED

Document for:	Discussion and decision

Agenda Item:	8.4.4

- Introduction

In RAN2#130 meeting [1], the following open issue was discussed with no conclusion:

| RRC-5 | whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2 Editor’s NOTE: There is no conclusion on whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

In this contribution, we will further discuss the above issue for LP-WUS in RRC_CONNECTED.

- Discussion

RAN2 agreed in RAN2#129bis meeting that UE could provide the preferred time offset via UAI signalling [2].

If configured, the UE can signal a preferred time offset via UAI signalling.

The remaining issue is whether it is allowed to report an empty UAI offset for LP-WUS monitoring for both option 1-1 and option 1-2. In our understanding, there is no motivation to restrict empty UAI on the offset for LP-WUS monitoring as the existing UAI report allows UE to report empty UAI, e.g., preferredDRX-InactivityTimer. Potential issue is ambiguity of UE behavior when it reports empty UAI offset. We assume that, in that case, UE has no preference for the time offset between LP-WUS and PDCCH monitoring, and gNB can configure an offset value based on UE capability report.

Proposal 1: It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2. For empty UAI on the time offset, UE has no preference on the time offset between LP-WUS and PDCCH monitoring, and gNB can configure an offset value based on UE capability report.

- Conclusion

In this contribution, we discuss the remaining issue for LP-WUS in RRC_CONNECTED. Based on the discussion, we have the following proposal.

Proposal 1: It is allowed to report an empty UAI on time offset for LP-WUS monitoring for both option 1-1 and option 1-2. For empty UAI on the time offset, UE has no preference on the time offset between LP-WUS and PDCCH monitoring, and gNB can configure an offset value based on UE capability report.

- References

[1]	R2-2504672, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)

[2]	R2-2502982, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others, RAN2 Vice Chairman (CATT)